3GPP TSG-SA5 Meeting #150 	S5-235666

Goteborg, Sweden, 21 - 25 August 2023

# Source:	Ericsson, Deutsche Telekom

# Title:	Discussion paper on communication services and service profile

# Document for:	Approval

Agenda Item:	6.5.2

# 1	Decision/action requested

The group is asked to endorse the detailed proposal in 4.

# 2	References

[1]	3GPP TS 28.530 Management and orchestration; Concepts, use cases and requirements

[2]	3GPP TS 28.531 	Management and orchestration; Provisioning

[3]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

# 3	Rationale

## 3.1	Introduction

A ServiceProfile captures external service requirements that a placed on a 3GPP system. Currently the ServiceProfile is dedicated to capture the requirements of the services that are allocated to a network slice. For that reason, the use of the ServiceProfile is limited to lifecycle management of NetworkSlices.

A ServiceProfile is also a generic service description of services that are instantiated in a 3GPP network. How such service is actually implemented is from the perspective of a service user not relevant as long the service request towards the 3GPP system is fulfilled and assured (monitored).

A ServiceProfile is one-to-one mapped to a SliceProfile, except some requirements a few which are not applicable for SliceProfile. A SliceProfile can capture the e2e requirements (mapped from ServiceProfile), the RAN, CN, and TN requirements.

The ServiceProfile and SliceProfile are defined in TS 28.541, see reference [3] as follows:

ServiceProfile, this data type represents the properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network. The network slice related requirements apply to a one-to-one relationship between a Network Slice Customer (NSC) and a Network Slice Provider (NSP). A network slice can be tailored based on the specific requirements adhered to an SLA agreed between NSC and NSP, see clause 2 of [50]. An NSP may add additional requirements not directly derived from SLA’s, associated to the NSP internal [business] goals. The GST defined by GSMA (see [50]) and the service performance requirements defined in 3GPP TS 22.261 [28] and TS 22.104 [51] are all considered as input for the network slice related requirements.

SliceProfile, this data type represents the properties of network slice subnet related requirement that should be supported by the NetworkSliceSubnet instance in a 5G network.

Table 3.1.1 Comparison ServiceProfile and SliceProfile

| ServiceProfile -Attribute name | SliceProfile -Attribute name |
| --- | --- |
| serviceProfileId | sliceProfileId |
| pLMNInfoList | pLMNInfoList |
| maxNumberofUEs | maxNumberofUEs |
| coverageArea | coverageAreaTAList |
| dLLatency | dLLatency |
| uLLatency | uLLatency |
| uEMobilityLevel | uEMobilityLevel |
| networkSliceSharingIndicator | resourceSharingLevel |
| sST |  |
| availability |  |
| delayTolerance | delayTolerance |
| dLDeterministicComm | dLDeterministicComm |
| uLDeterministicComm | uLDeterministicComm |
| dLThptPerSlice | dLThptPerSliceSubnet |
| dLThptPerUE | dLThptPerUE |
| uLThptPerSlice | uLThptPerSliceSubnet |
| uLThptPerUE | uLThptPerUE |
| dLMaxPktSize | dLMaxPktSize |
| uLMaxPktSize | uLMaxPktSize |
| maxNumberofPDUSessions | maxNumberOfPDUSessions |
| kPIMonitoring |  |
| userMgmtOpen |  |
| v2XCommModels |  |
| termDensity | termDensity |
| activityFactor | activityFactor |
| uESpeed | uESpeed |
| jitter |  |
| survivalTime | survivalTime |
| radioSpectrum | nROperatingBands |
| reliability | reliability |
| maxDLDataVolume |  |
| maxULDataVolume |  |
| nBIoT |  |
| synchronicity | synchronicity |
| positioning | positioning |
| sliceSimultaneousUse | sliceSimultaneousUse |
| energyEfficiency | energyEfficiency |
| nssaaSupport | nssaaSupport |
| n6Protection | n6Protection |

## 3.2	Analysis

Reflecting on the names of the attributes in the ServiceProfile shows that some of them are network slice specific, limiting the use to network slice only, these attributes are shown in Table 3.2.1.

Table 3.2.1 Attributes with network slice in name and proposed updates

| ServiceProfile attribute name | Observation |
| --- | --- |
| networkSliceSharingIndicator | Name and description limit use of this attribute to network slice. In a 5G scenario the sharing indicates if an S-NSSAI is dedicated or shared. A better phrase could be SharingIndicator. |
| sST | Name and description limit use of this attribute to network slice. In a 5G scenario the ServiceType indicates what sST is applicable. |
| dLThptPerSlice | There is no need to use “PerSlice” it should be obvious from the context. |
| uLThptPerSlice | There is no need to use “PerSlice” it should be obvious from the context. |
| sliceSimultaneousUse | Name and description limit use of this attribute to network slice. A better phrase could be simultaneousUse, just removing “slice”. |
| nssaaSupport | Name and description limit use of this attribute to network slice. A better phrase could be saaSupport, just removing “ns”, (network slice). |

## 3.4	Observation

A single instance of a ServiceProfile should describe the service requirements from a CSC to a 3GPP system. The CSP operating the 3GPP system decides how to fulfil the service requirements, the service requirements should therefore be network slice agnostic. Currently some of the ServiceProfile attributes use network slice in the name while the service requirements should be agnostic on how they will be fulfilled.

## 3.5	Conclusion

The group agrees that the terminology used for the ServiceProfile should be agnostic to the solution used to realize the service i.e., remove all references to network slice. The proposal is to make the serviceProfile datatype more generic without impact on the behaviour of a network slice producer.

# 4	Detailed proposal

The group is asked to endorse the conclusions in section 3.5 and the implementation of the conclusion in TS 28.541 CR number..xxx.