3GPP TSG-SA5 Meeting #150 	S5-235784

Goteborg, Sweden, 21 - 25 August 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduce ECUR message flow

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce ECUR message flow

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to introduce ECUR message flow.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

##### 5.X.2.2.w	Network slice-specific authentication and authorization – NSSAAF - ECUR

The following figure 5.X.2.2.w-1 describes a Network slice-specific authentication and authorization charging in ECUR scenario for NSSAAF:

Figure 5.X.2.2.w-1: Network slice-specific authentication and authorization – NSSAAF - ECUR

Steps 1 to 4 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure, AMF sends the EAP Identity Response to the NSSAAF.

4ch-a: NSSAAF sends Charging Data Request [Initial] to CHF with GPSI and S-NSSAI.

4ch-b: Account, Rating, control  by the CHF be granted authorization for NSSAA.

4ch-c: The CHF opens a CDR.

4ch-d: CHF provides response to NSSAAF.

Steps 5 to 17 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

17ch-a: NSSAAF procedure with AAA-S is completed:  NSSAAF sends Charging Data Request [Terminationt] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

17ch-b: Account, Rating, control by the CHF for NSSAA.

17ch-c: The CHF closes the CDR.

17ch-d: CHF provides response to NSSAAF.

Step 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

End of change