3GPP TSG-SA5 Meeting #150	S5-235877

Göteborg, Sweden, 21-25 August 2023

# Source:	Huawei

# Title:	Clarify solution #1.2 on the reporting per RG

# Document for:	Approval

Agenda Item:	7.5.1

1	Decision/action requested

Clarify solution description.

2	References

[1] 		TS 28.826: "Study on Nchf charging services phase 2 improvements and optimizations".

3	Rationale

According to the current charging mechanism TS 32.255, the charging reporting can be per rating group or per combination of rating group/service id. The charging reporting from SMF is based on the triggers setting. The proposed solution provides the clarification for the reporting mechanism.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.826 [1].

First change

5.1.5.2	Solution #1.2: Keep rating group information together

A possible solution for key issue #1e covering requirements REQ-CH_INFO-01, charging request optimization.

According to the current charging mechanism specified in 3GPP TS 32.255 [x] and 3GPP TS 32.290 [x], the trigger per RG does not distinguish with quota management (online) or without quota management (offline). If the rating group have service with quota management (online), then the charging data request  will report all used units for that rating group, including the quota management (online) and without quota management (offline) triggered in the rating group.  This is to ensure that the CHF won’t interpret any reporting of used units for the rating group as a release of all granted service units for the rating group.

End of change