# 3GPP TSG SA WG5 Meeting #150                 	           S5-235883

# Göteborg, Sweden, 21-25 August 2023

# Source:	Huawei, Ericsson

# Title:	Update the solution evaluation and conclusion in clauses 5.1, 5.2, 5.3, 5.4, and 5.5

# Document for:	Approval

Agenda Item:	7.5.1

1	Decision/action requested

Update the solution evaluation and conclusion.

2	References

[1] 		TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations".

3	Rationale

The solution evaluation and conclusion in clauses 5.x are discussed, including clauses 5.1, 5.2, 5.3, 5.,4, and 5.5.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.826 [1].

First change

5.1.6	Evaluation

Solution #1.1 solve key issues #1a, #1b, and #1c.

-	Solution #1.1: introduces a new trigger, that will hold the charging data request until there is traffic.

Solution #1.8, #1.9, #1.18 all solves key issue #1d.

- Solution #1.8: requires CTF to control the size of charging information per ChargingDataRequest based on the maxChargingData specified in the ChargingDataResponse. It is an optional attribute in ChargingDataResponse, up to CHF.

- Solution #1.9: requires CTF to keep monitoring the size of charging information based on the maxChargingData specified in the Trigger.

- Solution #1.18: introduce a new attribute in PDUContainerInformation or QFIContainerInformation that can uniquely identify itself. It limits the amount of data sent by allowing the CTF to reference between containers so that information doesn’t have to be repeated.

Solution #1.2 solve key issue #1e.

-	Solution #1.2: requires all used units reported for a rating group, both online and offline, to be delivered in the same request. It is already supported in the current specification. No further enhancement is needed.

Solutions #1.15 and #1.x solve key issues #1f and #1g.

-	Solution #1.15: introduces new triggers and information elements for the SMF, that will trigger and report based on payload counts.

-	Solution #1.x: introduces new triggers and information elements for the NEF, that will trigger and report based on number of API invocations or notifications.

Solutions #1.11, #1.12, #1.13, #1.16 and #1.17 all solve key issue #1h and #1i.

-	Solution #1.11: All the information in the Open API can be stored. All levels of the CDR structure should be changed. CHF should deal with the undefined CDR structure parameters.

-	Solution #1.12: All the information in the Open API can be stored. Enhance the sequence number for multiple attributes. CHF should deal with the undefined CDR structure parameter.

-	Solution #1.13: All the information in the Open API can be stored. The main level of the CDR structure should be changed. CHF should deal with the undefined CDR structure parameter.

-	Solution #1.16: CHF can generate the CDRs based on the supported feature negotiation. Unsupported information in the Open API will not be stored in the CDRs.

-	Solution #1.17: Depending on the operator’s selection.

-	Solution #1.x: Reuse the existing reserved range to store undefined value.

Solutions #1.3 and #1.14 solve key issue #1j.

-	Solution #1.3: requires that the NF (CTF) provides and ensure the level of accuracy required by the CHF.

-	Solution #1.14: requires the CHF determine if the accuracy meets current requirements based on the reported accuracy level and Location change received time.

Editor’s note: The meaning of accuracy should be clarified.

Solution #1.10 solve key issue #1k.

-	Solution #1.10: requires that the UPF and SMF enhancement to support the bitrate aggregation and reporting.

-	Solution #1.x: reuses the existing mechanism Quota-Consumption-Time to support tiered charging.

Solutions #1.4, #1.5, #1.6, and #1.7 all solve key issue #1l.

-	Solution #1.4: NF (CTF) stops service delivery if no response is received before all the quotas (between the threshold and the granted quota) have been used and discards used units in the case of termination is received.

-	Solution #1.5: NF (CTF) terminates the service if no response is received before all the quotas (between the threshold and the granted quota) have been used and reports the used units in a termination request. This means that it will in some cases terminate the service even though there are funds on the account.

-	Solution #1.6: NF (CTF) stops or continues service delivery if no response is received all the quotas (between the threshold and the granted quota) have been used depending on CHF indication and reports used units in the next request. This requires new functionality in both CHF and NF (CTF).

-	Solution #1.7: NF (CTF) stops service delivery and waits for response when all the quotas (between the threshold and the granted quota) have been used and reports used units in the next request. This is possible even if it is not specified.

5.1.7	Conclusions

For key issue #1d, solution #1.8 is recommended into normative work, controlling the charging information size by specifying the data limit in ChargingDataResponse.

For key issue #1h and #1i, solution #1.x and #1.17 is recommended into normative work, using the reserved range to store undefined value.

For key issue #1j, solution #1.3, which requires CTF to ensure the accuracy of location, is recommended into normative work.

For key issue #1k, solution #1.x is recommended into normative work. tiered charging could be supported by adopting the Quota-Consumption-Time mechanism.

For key issue #1l, the solution #1.7 is selected for the normative work.

Editor’s note: Conclusions are FFS.

Next change

5.2.6	Evaluation

Solutions #2.1, and #2.2 both solve key issues #2a and #2b.

-	Solution #2.1: reuses the current triggers. It has already defined usage for operation management events, the usage of operation management triggers are related to management events and have no more information about why it was triggered therefore it may cause confusion as to why the trigger was considered met.

-	Solution #2.2: requires introduction of a new trigger. Separate the trigger from any other types of triggers. It is not necessary to define the extension of private triggers in the standard. No more information about why it was triggered therefore it may cause confusion as to why the trigger was considered met.

Solution #2.3 solves key issue #2b and #2c.

-	Solution #2.3: requires introduction of a new trigger profile, that can also solve issues for event based trigger handling.. This requires both CTF and CHF maintain a unique and persistent triggerProfileId as the reference key.

5.2.7	Conclusions

Editor’s note: Conclusions are FFS.

Next change

5.3.6	Evaluation

Solution #3.1, #3.2, #3.3, #3.4, and #3.5 all solve key issues #3a, #3b, and #3c.

-	Solution #3.1: the new service id list holds the used service ids which can provide the service level information (e.g. video, email, file transfer, VoIP) which will make it easier for the CHF to allocate quota for the rating group. It may cause confusion as to if this is the service id for the requested units or the reported used units.

-	Solution #3.2: the new service id list holds the used service ids which can provide the service level information (e.g. video, email, file transfer, VoIP) and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

- Solution #3.3: the new QoS information which can provide the service quality and service experience requirement information (e.g. the authorized bandwidth, latency) holds the currently applied QoS for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for different service quality requirement of the same rating group.

PCC rules include the static and dynamic PCC rules. For dynamic PCC rules, service Ids and QBCs are dynamically determined.

- Solution #3.4: the new additional information holds both the QoS information and service ids the currently applied for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

Editor’s note: Further evaluation is FFS.

5.3.7	Conclusions

Editor’s note: Conclusions are FFS.

Next change

5.4.6	Evaluation

Solutions #4.1, #4.2 and #4.3 solve key issue #4a.

Solution #4.1:

-	requires an extension to current service operation with a specific service operation (i.e. Nchf_ConvergedCharging_Cancel) and uses the current Event type and chargingDataRef.

-	The allocation of ChargingDataRef should be unique to void duplication, otherwise, it may cause an error.

-	The ChargingDataRef can not be recycled until a cancel is no longer allowed for a = charging session or event.

Solution #4.2:

-	reuses the current event with specific information to determine refund, making the refund implicitly defined.

-	using a Nchf_ConvergedCharging Release service operation for events to support carry the previous refundInformation in the case of a cancel, in essence making the event into a sessio.

-	The negotiation is completed based on the charging data response, if the refundinformation is included in the Charging Data Response, the CHF support the event cancelling.

Solution #4.3:

-	requires an extension to current service operation with a specific service operation (i.e. Nchf_ConvergedCharging_Cancel)

-	Requires anextentsion to current event type with a specific event type (i.e. Cancel)

-	Uses the current chargingDataRef in a new attribute, charging id or both, the use of both makes it easier to find the Charging Data Request that is to be cancelled.

-	requires feature negotiation for Cancelling the event, the chargingDataRef may not be stored after a period of time and may be directly recycled because that CHF are not aware of the event may be cancelled.

5.4.7	Conclusions

For key issue #4a, solution #4.3 is recommended into normative work.

Next change

5.5.6	Evaluation

Solution #5.1 and #5.5 all solve key issue #5d.

Solution #5.1:

-	Keep alignment with the Figure 5.3.2.3.2: SCUR - Session based charging with Decentralized and Centralized Unit Determination, Centralized Rating, immediate start of service delivery (non-blocking mode) specified in TS 32.290 [12], before sent the charging data request, the service delivery starts.

- 	A new indication is added in the charging data request.

- 	CHF may instruct to stop the service delivery based on the non-blocking mode awareness, not only for the disable/enable the non-blocking mode.

Solution #5.5:

-	The start of the service delivery does not need to wait the Charging Data Response from CHF. Keep alignment with the common service flow about non-blocking specified in the TS 32.290 [12]. Needs to be combined with solution #5.1 to allow the indication to be sent in both the blocking and non-blocking case. A new indication is added in the charging data request.

-	CHF may instruct to stop the service delivery based on the non-blocking mode awareness, not only for the disable/enable the non-blocking mode.

Solution #5.2, #5.3, #5.4 and #5.6 all solve key issue #5a, #5b and #c.

Solution #5.2:

-	Allows the CHF to directly control the non-blocking mode, by introducing an attribute. This means that both PCF and CHF will control the non-blocking mode.

-	The change of non-blocking mode can be per service data flow or per RG.

-	The Validity Period of non-blocking mode change from CHF is per PDU session. When the new PDU session is established, the non-blokcing mode will reuse the PCC rule from PCF which is not able to know the change from the CHF.

Solution #5.3:

-	Allows the CHF to indirectly control the non-blocking mode, by using the current policy counters. This means that only PCF will control the non-blocking mode, with input from CHF.

-	The change of non-blocking mode can be per service data flow or per RG.

-	The PCF can be able to know the change of non-blocking mode from the CHF dynamically.

Solution #5.4:

-	The granularity of the control of non-blocking mode is per UE, not for special service data flow or per RG.

-	The new type of Nchf_ SpendlingLimitControl service for spending limit should be supported.

-	the controlling on the Non-blocking mode need to add the new value of currentStatus to present that the non-blocking mode is the changing value (disable or enable) which is not be specified in the standard.

Solution #5.6:

-	reuses the current mechanism and by that the start of the service delivery does not need to wait the Charging Data Response from CHF. Keep alignment with the common service flow about non-blocking specified in the TS 32.290 [12] without any impact on the charging data request.

-	the new policy counter for spending limit is connected to one or more Rating Groups in the CHF which cannot be supported in the existing Nchf_SpendingLimitControl Service and cannot support the control of non-blocking mode per service data flow or per RG.

-	the new type of Nchf_ SpendlingLimitControl service for spending limit should be supported.

-	the controlling on the Non-blocking mode need to add the new value of currentStatus to present that the non-blocking mode is the changing (disable or enable) which is not be specified in the standard.

-	cannot be aware of the non-blocking mode information from PCF, CHF provides the notification about the policy counter status for the PCF, but how the PCF synchronize the policy counter status of non-blocking mode if changed by the policy control is unclear.

5.5.7	Conclusions

Editor’s note: Further conclusions is FFS.

End of change