3GPP TSG-SA5 Meeting #150 	S5-235909

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0458 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance measurements for the UE assistance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no performance measure defined in TS 28.552 quantifying the utilization of this feature in both network and UE side. In this CR, it is proposed to define the number of RRCReconfiguration for configuring UE assistance information for overheating as well as number of received UE assistance information for overheating. This performance measure is used to derive UAI enablement and utilization for overheating per connection KPIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing the number of RRCReconfiguration for configuring UE assistance information for overheating as well as number of received UE assistance information for overheating. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, average number of UAI enablement per connection for overheating and average number of UAI utilization per connection for overheating (TS 28.554 (CR0136) cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.y (new), 5.1.1.z (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

###### 5.1.1.y	Number of RRCReconfiguration for configuring UE assistance information for overheating

a)	This measurement provides the number of RRCReconfiguration carrying the UAI configuration with overheatingAssistanceConfig for an RRC connection established within the existing NRCellCU.

b)	CC

c)	On transmission of RRCReconfiguration message to the UE carrying otherConfig (see TS 38.331 Clause 5.3.5.9), if the message contains setup of the overheatingAssistanceConfig, the counter will be incremented.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.OtherConfig.overheatingAssistance

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI enablement.

# >>>>NEXT CHANGE<<<<

###### 5.1.1.z	Number of received UE assistance information for overheating

a)	This measurement provides the number of OverheatingAssistance messages received by the network for an RRC connection established within the existing NRCellCU (see TS 38.331 Clause 5.7.4.1). Note that for each RRC connection the network may receive multiple UAI messages for each feature. The measurement is split into subcounters for each UAI OverheatingAssistance request.

b)	CC

c)	Upon reception of UEAssistanceInformation message by the network, if the message contains an update to a OverheatingAssistance feature, the corresponding subcounter will be incremented. The existing OverheatingAssistance features are as follows: reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedBW-FR2-2-DL,reducedBW-FR2-2-UL, reducedMIMO-LayersFR1-DL,reducedMIMO-LayersFR1-UL, reducedMIMO-LayersFR2-DL,reducedMIMO-LayersFR2-UL, reducedMIMO-LayersFR2-2-DL,reducedMIMO-LayersFR2-2-UL

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.Overheating.feature, where feature denotes the corresponding overheating assistance feature.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i) One usage of this performance measurement is to characterize the UAI utilization.

# >>>>NEXT CHANGE<<<<

A.X Use case for UE assistance information related measurements

The number of RRCReconfiguration carrying the UE assistance information configuration for each feature is used to derive the percentage of the RRC connections which are enabled to potentially use the related UAI feature. Furthermore, the number of UEAssistanceInformation messages received by the network for each particular UAI feature can be utilized to quantify what percentage of the UAI-enabled RRC connection are utilizing the corresponding UAI feature. Furthermore, the network can incorporate this information to only enable the UE assistance information features for the devices which are supporting these features.

# >>>>END OF CHANGES<<<<