3GPP TSG-SA5 Meeting #150 	S5-235958

Goteborg, Sweden, 21 - 25 August 2023										Revision of SP-22XXXX

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.201 | CR | 0010 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Charging Data message content for NSPA Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the consistent of the mandatory attributes specified in the common message contents, the Invocation Sequence Number should be present and Multiple Unit Usage should be the Oc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction the Invocation Sequence Number and Multiple Unit Usage in NSPA charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NSPA charging information reporting is not aligned with the other specifications, hence, the NSPA charging cannot be supported if not corrected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for performance and analytics based charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Subscriber Identifier | - | This field is not applicable. |
| Tenant Identifier | OM | This field if present is the identifier of subscriber of network slice. |
| NF Consumer Identification | M | Described in TS 32.290 [57]. |
| NF Functionality | M | Described in TS 32.290 [57]. |
| NF Name | OC | Described in TS 32.290 [57]. |
| NF Address | OC | Described in TS 32.290 [57]. |
| NF PLMN ID | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | M | Described in TS 32.290 [57]. |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57]. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Notify URI | - | This field is not applicable. |
| Service Specification Information | OC | Described in TS 32.290 [57]. |
| Triggers | OC | Described in TS 32.290 [57]. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57], with the exception that quota management is not applicable. |
| Rating Group | M | Described in TS 32.290 [57]. |
| Requested Unit | - | This field is not applicable. |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| Service Identifier | - | This field is not applicable. |
| Quota management Indicator | - | This field is not applicable. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the network slice performance and analytics specific triggers described in clause 5.2.1. |
| Trigger Timestamp | OC | Described in TS 32.290 [57] |
| Time | - | This field is not applicable. |
| Total Volume | - | This field is not applicable. |
| Uplink Volume | - | This field is not applicable. |
| Downlink Volume | - | This field is not applicable. |
| Service Specific Unit | - | This field is not applicable. |
| Event Time Stamps | - | This field is not applicable. |
| Local Sequence Number | OM | Described in TS 32.290 [57] |
| NSPA Container Information | OC | This field holds the network slice performance and analytics container specific information described in clause 6.2.1.3. |
| NSPA Charging Information | OC | This field holds the network slice information, which is reported to the CHF described in clause 6.2.1.2. |

Next change

6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by CEF for network slice performance and analytics converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3-1 defines the basic structure of the supported fields in the Charging Data Request message for CEF converged charging.

Table 6.2.3-1: Supported fields in Charging Data Request message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | - |
| Subscriber Identifier |  | - |
| Tenant Identifier |  | E |
| NF Consumer Identification |  | E |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | E |
| Retransmission Indicator |  | - |
| One-time Event |  | E |
| One-time Event Type |  | E |
| Notify URI |  | - |
| Triggers |  | E |
| Multiple Unit Usage |  | E |
| Rating Group |  | E |
| Used Unit Container |  | E |
| Triggers |  | E |
| NSPA Container Information |  | E |
| NSPA Charging Information |  |  |
| Single NSSAI |  | E |

Table 6.2.3-2 defines the basic structure of the supported fields in the Charging Data Response message for CEF converged charging.

Table 6.2.3-2: Supported fields in Charging Data Response message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Sequence Number |  | - |
| Session Failover |  | E |
| Triggers |  | - |
| Multiple Unit information |  | - |

End of change