3GPP TSG-SA5 Meeting #151 	S5-236707

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0460 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Correction of invocation timestamp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The record opening time should be the time when the CHF opens the record and not the invocation was done. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of invocation time to the CDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The time of the invocation will not be included in the CDR, which may cause issues for the billing systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2, and 6.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.298 CR 0960 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.3.2	PDU session charging CHF CDR data

If enabled, CHF CDRs for PDU session charging shall be produced for each PDU session. In roaming Home routed scenario, the PDU session charging CHF CDR shall cover both Flow based Charging and Qos flow Based Charging (QBC) from H-SMF.

The fields of PDU session charging CHF CDR are specified in table 6.1.3.2.1.

Table 6.1.3.2.1: PDU session charging CHF record data

| Field |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Record Type |  | M |  | CHF record. |  |
| Recording Network Function ID |  | OM |  | This field holds the name of the recording entity, i.e. the CHF id. |  |
| Subscriber Identifier |  | OM |  | This field holds the Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. The detail of SUPI is specified in clause 5.9.2 of TS 23.501 [200] |  |
| NF Consumer Information |  | M |  | This field holds the information of the SMF that used the charging service. |  |
| NF Functionality |  | M |  | This field contains the function of the node (i.e. SMF) |  |
| NF Name |  | OC |  | This field holds the name of the SMF used. |  |
| NF Address |  | OC |  | This fields holds the IP Address of the SMF used. |  |
| NF PLMN ID |  | OC |  | This field holds the PLMN identifier (MCC MNC) of the SMF. |  |
| Invocation Timestamp |  | OM |  | This field holds the timestamp of the charging service invocation, described in TS 32.290 [57]. |  |
| List of Multiple Unit Usage |  | OM |  | This field holds a list of changes in charging conditions for all service data flows within this PDU session.This list is categorized per rating group or per combination of rating group and service id or per combination of rating group, sponsor identity and application service provider identity. In addition, usage is differentiated between with and without quota management. Each change is time stamped. Charging conditions are used to categorize traffic volumes, elapsed time and number of events, such as per tariff period. |  |
| Rating Group |  | OM |  | This filed holds the rating group. |  |
| Used Unit Container |  | OC |  | This field holds the used units and information connected to the reported units. |  |
| Service Identifier |  | OC |  | This field holds the Service Identifier. |  |
| Quota management Indicator |  | OM |  | This field holds an indicator on whether the used units are with or without quota management. |  |
| Triggers |  | OC |  | This field holds the reason for closing the used unit container. |  |
| Trigger Timestamp |  | OC |  | This field holds the timestamp of the trigger. |  |
| Time |  | OC |  | This field holds the amount of used time. |  |
| Total Volume |  | OC |  | This field holds the amount of used volume in both uplink and downlink directions. |  |
| Uplink Volume |  | OC |  | This field holds the amount of used volume in uplink direction. |  |
| Downlink Volume |  | OC |  | This field holds the amount of used volume in downlink direction. |  |
| Service Specific Unit |  | OC |  | This field holds the amount of used service specific units. |  |
| Event Time Stamps |  | OC |  | This field holds the timestamps of the event reported in the Service Specific Units, if the reported units are event based. |  |
| Rating Indicator |  | OC |  | This field indicates if the units have been rated or not. |  |
| Local Sequence Number |  | M |  | This field holds the container sequence number. |  |
| PDU Container Information |  | OC |  | This field holds the 5G data connectivity specific information defined in clause 6.2.1.3. |  |
| UPF ID |  | OC |  | This field holds the UPF identifier used to identify the UPF when reporting the usage for the UPF. |  |
| Multi-homed PDU address |  | Oc |  | This field holds the Multi-homed IPv6 prefix used by UPF, identified by the UPF ID. It may only be used for reporting used units. |  |
| Record Opening Time |  | M |  | Time stamp when the PDU session is activated in the SMF or record opening time on subsequent partial records. |  |
| Duration |  | M |  | This field holds the duration of this record. |  |
| Record Sequence Number |  | C |  | Partial record sequence number, only present in case of partial records. |  |
| Cause for Record Closing |  | M |  | The reason for the release of the record. |  |
| Diagnostics |  | OM |  | This field holds a more detailed reason for the release of the PDU session, when a single cause is applicable. |  |
| Local Record Sequence Number |  | OM |  | Consecutive record number created by the CDF. The number is allocated sequentially including all CDR types. |  |
| Record Extensions |  | OC |  | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |  |
| PDU Session Charging Information |  | OM |  | This field holds the 5G data connectivity specific information defined in clause 6.2.1.2. |  |
| Roaming QBC information |  | OC |  | This field holds the roaming QBC specific information defined in clause 6.2.1.4, when applicable. |  |

Second change

#### 6.1.3.3	Roaming QBC CHF CDR data

If enabled, CHF CDRs for Roaming QBC shall be produced in VPLMN for each PDU session established for an in-bound roamer. The fields of Roaming QBC CHF CDR are specified in table 6.1.3.3.1.

Table 6.1.3.3.1: Roaming QBC CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | M | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party, if available. |
| NF Information | OC | This field holds the information of the V-SMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node. |
| NF Name | OC | This field holds the name of the V-SMF used. |
| NF Address | OC | This fields holds the IP Address of the V-SMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the V-SMF. |
| Invocation Timestamp | OM | This field holds the timestamp of the charging service invocation, described in TS 32.290 [57]. |
| Record Opening Time | M | Time stamp when the PDU session is activated in the SMF or record opening time on subsequent partial records. |
| Duration | M | This field holds the duration of this record. |
| Record Sequence Number | C | Partial record sequence number, only present in case of partial records. |
| Cause for Record Closing | M | The reason for the release of the record. |
| Diagnostics | OM | This field holds a more detailed reason for the release of the PDU session, when a single cause is applicable. |
| Local Record Sequence Number | OM | Consecutive record number created by the CHF. The number is allocated sequentially including all CDR types. |
| Record Extensions | OC | A set of network operator/manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| PDU Session Charging Information | OM | This field holds the 5G data connectivity specific information defined in clause 6.2.1.2. |
| Roaming QBC Information | OM | This field holds the 5G data connectivity Roaming QBC specific information defined in clause 6.2.1.4 |

End of changes