3GPP TSG-SA5 Meeting #151 	S5-237014

Xiamen, China, 9 - 13 October 2023

# Source:	Ericsson

# Title:	Evaluation and clarification charging information optimization solutions

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

The bitrate handling needs further clarification.

# 4	Detailed proposal

First change

#### 5.1.5.21	Solution #1.x: Bitrate charging using volume and time measurement

A possible solution for key issue #1k and requirement REQ-CH_INFO-06, Bitrate Charging.

The SMF(CTF) reports the actual duration with the traffic and the volume of used usage during the actual duration for data transmission.

Figure 5.1.5.21-1: Message flow for bitrate charging supporting from CHF

1) 	Charging data request [Initial, Quota Requested]: The SMF sends the request to CHF for the charging session establishment for the PDU session establishment or start of the service date flow.

2) 	Open CDR:  CHF generates the CDR based on the charging data request.

3)	Bitrate Charging Determine: CHF decides to start the bitrate charging.

4)	Charging Data Response [Initial, Quota Granted]: The CHF responds and if the service is under quota management it includes a quota granted for the service start.

5)	Service delivery start: there is a connection setup for service delivery start and the uplink or downlink data traffics is transferred.

6)	Trigger event: the SMF generates charging information related to the service delivered, due to that an immediate trigger for usage reporting,

7)	Charging data request [Update, Unit used]: the SMF sends the request to the CHF reporting the used units, including both the time and the volume of the traffic.

8)	Calculate bitrate: The CHF calculates the mean bitrate, between the initial and update, by taking the volume in bytes times 8 (to get bits) and dividing it with the time in seconds.

9)	Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

10)	Charging Data Response [Update]: The CHF responds the Charging Data request.

11)	Service delivery is continuing, and the charging session is continuing between the SMF and the CHF.

End of changes