3GPP TSG-SA5 Meeting #152  	S5-237497

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0228 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Correct the description of Procedure of Network Slice Instance allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incorrect indexed steps and incomplete descriptions in the network slice allocation procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the flow diagram, step 4.b-2 should be step 3b-2. - Add the description for the missing step 5.b-2. - Add descriptions on attributes configured to improve the readbility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification leads to confusion and incorrect implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.3, C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1a-1)	Network Slice Management Service Provider (NSMS_P) receives an AllocateNsi request (see AllocateNsi operation defined in clause 6.5.1) from Network Slice Management Service Consumer (NSMS_C) with network slice related requirements (the network slice related requirements are defined as the attributes in the ServiceProfile see clause 6.3.3 in TS 28.541 [6]).

1a-2)	The NSMS_P creates a NetworkSliceController MOI and configures attribute serviceProfileId to control and monitor network slice allocation process. NSMS_P starts to perform the network slice allocation process.

1a-3)	The NSMS_P sends AllocateNsi response to the NSMS_C. The serviceProfileId is included in the AllocateNsi response.

1a-4) NSMS_P sends notification (see notifyMOICreation defined in TS 28.532 [8]) to NSMS_C to notify the DN of the created NetworkSliceController instance.

1b-1)	NSMS_P receives a createMOI request (see createMOI operation defined in TS 28.532 [8]) from NSMS_C with network slice controller (see NetworkSliceController IOC and its attributes defined in TS 28.541[6]) including the network slice related requirements in attribute inputServiceProfile.

1b-2)	The NSMS_P creates an MOI of NetworkSliceController.

1b-3)	The NSMS_P sends the response to NSMS_C for the createMOI request.

NOTE: Network slice allocation may be initiated using AllocateNsi operation as described in steps 1a or createMOI operation as described in steps 1b.

2)	Based on the network slice related requirements and the knowledge of the capabilities of existing deployed network slices, the NSMS_P may compare/match the provided requirements against all the candidate NetworkSlice MOIs, and then decides whether to use an existing NetworkSlice MOI or create a new NetworkSlice MOI. If the network slice related requirements allow the requested NetworkSlice MOI to be shared and if an existing suitable NetworkSlice MOI can be reused, the NSMS_P may decide to use the existing NetworkSlice MOI.

3a-1)	If using an existing NetworkSlice MOI, the NSMS_ P identifies the NetworkSlice MOI to be re-used.

3a-2)	The NSMS_P configures the attribute networkSliceControllerRef of the NetworkSlice MOI with the DN of NetworkSliceController MOI.

3a-3)	NSMS_P configures the NetworkSliceController MOI attribute networkSliceRef with the DN of NetworkSlice MOI (identified in step 3a-1).

3a-4) If the existing NetworkSlice MOI needs to be modified to satisfy the network slice related requirements, the NSMS_P invokes the procedure of network slice instance modification as described in clause 7.6.

3b-1) If creating a new NetworkSlice MOI, the NSMS_P derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_P derives the network slice subnet related requirements, NSMS_P may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

3b-2)	The NSMS_P invokes the procedure of network slice subnet instance allocation as described in clause 7.3. Before NSMS_P invokes the procedure, NSMS_P may invoke corresponding procedure of feasibility check and reservation of network slice subnet as described in clause 7.14.

3b-3)	The NSMS_P creates the NetworkSlice MOI and configures the attribute networkSliceSubnetRef with the DN of NetworkSliceSubnet MOI and the attribute networkSliceControllerRef with the DN of NetworkSliceController MOI. Other configuration information may be configured for the created NetworkSlice MOI.

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

3a-5)	NSMS_P configures the NetworkSliceController MOI attribute networkSliceRef with the DN of NetworkSlice MOI (created in step 3b-3).

4)	NSMS_P configures NetworkSliceController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor to indicate completion of the procedure.

The NSMS_C may check the status and completion of the network slice allocation procedure any time by monitoring the values of NetworkSliceController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor by querying the values or by subscribing to notifications.

The following step 5 describes the procedures for NSMS_C to monitor network slice allocation progress and result. These steps can happen anytime after the NetworkSliceController MOI is created and its DN is notified to the NSMS_C (step 1a-4 or step 1b-3).

5a)	The NSMS_P sends notification (see notifyMOIAttributeValueChanges defined in TS 28.532 [8]) to NSMS_C to notify the progress and result for network slice allocation process (see attributes in NetworkSliceController IOC).

5b-1)	The NSMS_C sends query request to NSMS_P to query the attribute values of NetworkSliceController MOI (see getMOIAttributes operation defined in TS 28.532 [8]) to obtain the progress and result (including DN of the NetworkSlice MOI) for network slice allocation process.

5b-2)	The NSMS_P sends the response to NSMS_C with the NetworkSliceController information

End of First change

Start of Second change

7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new network slice subnet instance or using an existing network slice subnet instance to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1a-1)	Network Slice Subnet Management Service Provider (NSSMS_P) receives an AllocateNssi request (see AllocateNssi operation defined in clause 6.5.2) from Network Slice Subnet Management Service Consumer NSSMS_C with network slice subnet related requirements (network slice subnet related requirements defined in SliceProfile see clause 6.3.4 in TS 28.541 [6]).

1a-2)	The NSSMS_P creates an MOI of NetworkSliceSubnetController and configures attribute sliceProfileId to control and monitor network slice subnetallocation process. NSSMS_P starts to perform the network slice subnet allocation process.

1a-3) The NSSMS_P sends AllocateNssi response to the NSSMS_C. The sliceProfileId is included in the AllocateNssi response.

1a-4) NSSMS_P sends notification (see notifyMOICreation defined in TS 28.532 [8]) to NSSMS_C to notify the DN of the created NetworkSliceSubnetController instance.

1b-1)	NSSMS_P receives a createMOI request (see createMOI operation defined in TS 28.532 [8]) from NSSMS_C with network slice subnet controller (see NetworkSliceSubnetController IOC and its attributes defined in TS 28.541[6])) including the network slice subnet related requirements in attribute inputSliceProfile.

1b-2)	The NSSMS_P creates an MOI of NetworkSliceSubnetController.

1b-3)	The NSSMS_P sends the response to NSSMS_C for the createMOI request.

NOTE: Network slice subnet allocation may be initiated using AllocateNssi operation as described in steps 1a or createMOI operation as described in steps 1b.

2) NSSMS_P may invoke corresponding procedure of feasibility check and reservation of network slice subnet as described in clause 7.14. If the network slice subnet related requirements can be satisfied, the following step 3) to step 5) are needed, else go to step 6).

3)	Based on the network slice subnet related requirements and the existing NetworkSliceSubnet MOI capabilities, NSSMS_P decides whether to use an existing NetworkSliceSubnet MOI or create a new NetworkSliceSubnet MOI. If the network slice subnet related requirements allow the requested NetworkSliceSubnet MOI to be shared and if an existing suitable NetworkSliceSubnet MOI can be reused, the NSSMS_P decides to use the existing NetworkSliceSubnet MOI.

4a-1) If using an existing NetworkSliceSubnet MOI, the NSSMS_ P identifies the NetworkSliceSubnet MOI to be reused.

4a-2)	The NSSMS_P configures NetworkSliceSubnet MOI (attribute networkSliceSubnetControllerRef) with the DN of NetworkSliceSubnetController MOI.

4a-3)	NSSMS_P configures the NetworkSliceSubnetController MOI (attribute networkSliceSubnetRef) with DN of NetworkSliceSubnet MOI (identified in step 4a-1).

4a-4)	If the existing NetworkSliceSubnet MOI needs to be modified to satisfy the network slice subnet related requirements, the NSSMS_P invokes the procedure of network slice subnet instance modification as described in clause 7.7.

4b-1)	If creating a new NetworkSliceSubnet MOI, the NSSMS_P creates an MOI of NetworkSliceSubnet. The NSSMS_P configures NetworkSliceSubnet MOI (attribute networkSliceSubnetControllerRef) with DN of NetworkSliceSubnetController MOI.

4b-2)	NSSMS_P configures the NetworkSliceSubnetController MOI (attribute networkSliceSubnetRef) with DN of NetworkSliceSubnet MOI (identified in step 4b-1).

4b-3) NSSMS_P derives the corresponding network slice subnet constituent (i.e., NF, constituent NetworkSliceSubnet) related requirements and transport network related requirements (e.g., 3GPP endpoint information, latency requirements, bandwidth requirements and isolation requirements) from the received network slice subnet related requirements. Part of these requirements may be referenced by attribute epTransportRef as defined in clause 6.3.2.2 in TS 28.541[6]. Before NSSMS_P derives the constituent network slice subnet related requirements, NSSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

4b-4)	If the NetworkSliceSubnet MOI contains virtualisation part (i.e. VNF or VL), NSSMS_P derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. NSSMS_P determines new VNF instance(s) that need to be deployed and the existing VNF instance(s) that need to be reused according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. NSSMS_P chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. NSSMS_P invokes the NS instantiation procedures to create a NS instance.

Note:	NS instantiation procedure is described in TS 28.526 [7].

4b-5)	NSSMS_P configures NetworkSliceSubnet MOI attribute nsInfo with NS instance information.

4b-6) For each required constituent network slice subnet (NetworkSliceSubnet MOIs), the following step 4b-6a) and 4b-6b) are needed:

4b-6a)	NSSMS_P invokes procedure of network slice subnet instance allocation.

4b-6b)	NSSMS_P configures NetworkSliceSubnet MOI attribute networkSliceSubnetRef with DN of NetworkSliceSubnet MOI.

4b-7)	For each required constituent NF instance (ManagedFunction MOI), the following step 4b-7a) and 4b-7b) are needed:

4b-7a)	NSSMS_P invokes procedure of NF instance creation described in clause 7.10 or procedure of NF instance modification described in clause 7.11.

4b-7b)	NSSMS_P configures NetworkSliceSubnet MOI attribute managedFunctionRef with DN of ManagedFunction MOI.

4b-8)	For each required transport network related requirements, NSSMS_P invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

5)	NSSMS_P configures NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor to indicate completion of the procedure.

The NSSMS_C may check the status and completion of the network slice subnet allocation procedure any time by monitoring the values of NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor by querying the values or by subscribing to notifications.

Following step 6) describes the procedures for NSSMS_C to monitor network slice subnet allocation progress and result. These steps can happen anytime after the NetworkSliceSubnetController MOI is created and its DN is notified to the NSSMS_C (step 1a-4 or step 1b-3).

6a) The NSSMS_P sends notification (see notifyMOIAttributeValueChanges defined in TS 28.532 [8]) to NSSMS_C to notify the progress and result for network slice subnet allocation process (see attributes in NetworkSliceSubnetController IOC).

6b-1) The NSSMS_C sends query request to NSSMS_P to query the attribute value of NetworkSliceSubnetController MOI (see getMOIAttributes operation defined in TS 28.532 [8]) to obtain the progress and result (including DN of theNetworkSliceSubnet MOI) for network slice subnet allocation process.

6b-2)	The NSMS_P sends the response to NSMS_C with the NetworkSliceSubnetController information

End of Second change

Start of Third change

C.2	Procedure of network slice instance allocation

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

actor "NSMS_C" as NSMS_Consumer

participant "\tNSMS_P\t\t" as NSMS_Provider

alt NSMS_C sends AllocateNsi request

NSMS_Consumer -> NSMS_Provider: 1a-1. AllocateNsi(ServiceProfile) Request

NSMS_Provider -> NSMS_Provider: 1a-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1a-3. AllocateNsi(ServiceProfileId) Response

NSMS_Provider --> NSMS_Consumer: 1a-4. notifyMOICreation(\n\tobjectClass=NetworkSliceController, \n\tobjectInstance=DN of NetworkSliceController MOI,...)

else NSMS_C sends createMOI request

NSMS_Consumer -> NSMS_Provider: 1b-1. createMOI(\n\t managedObjectClass=NetworkSliceController, \n\t attributeListIn=inputServiceProfile,...) Request

NSMS_Provider -> NSMS_Provider: 1b-2. Creates NetworkSliceController MOI

NSMS_Provider --> NSMS_Consumer: 1b-3. createMOI() Response

end alt

NSMS_Provider -> NSMS_Provider: 2. Decides to create a new NetworkSlice MOI\

\n\t or use an existing NetworkSlice MOI.

alt Using an existing NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3a-1. Identifies the NetworkSlice MOI to re-use

NSMS_Provider -> NSMS_Provider: 3a-2. Configures NetworkSlice MOI with \n DN of NetworkSliceController MOI

NSMS_Provider -> NSMS_Provider: 3a-3. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

opt If the existing NetworkSlice MOI needs to be modified

ref over NSMS_Provider

3a-4. Procedure of Network Slice Instance Modification

end ref

end opt

else Creating a new NetworkSlice MOI

NSMS_Provider -> NSMS_Provider: 3b-1. Derives the network slice subnet related requirements

ref over NSMS_Provider

3b-2. Procedure of Network Slice Subnet Instance Allocation

end ref

|||

NSMS_Provider -> NSMS_Provider: 3b-3. Creates NetworkSlice MOI and \n configures NetworkSlice MOI with \nDN of NetworkSliceController MOI and \nDN of NetworkSliceSubnet MOI

NSMS_Provider -> NSMS_Provider: 3b-4. Configures NetworkSliceController MOI with \n DN of NetworkSlice MOI

|||

end alt

NSMS_Provider -> NSMS_Provider: 4. Updates attributes of NetworkSliceController MOI\nindicating completion of the procedure

group Monitoring network slice allocation progress and result. \nThis can be happened anytime after step NetworkSliceController is created (i.e., step 1a-2 or step 1b-2)

alt obtain the progress and result by subscribe-notification method

NSMS_Provider -> NSMS_Consumer: 5a. Notify network slice allocation progress and result \n (DN of NetworkSliceController MOI and its attributes)

else obtain the progress and result by query method

NSMS_Consumer -> NSMS_Provider: 5b-1. Request to query the \n NetworkSliceController information (DN of NetworkSliceController)

NSMS_Provider -> NSMS_Consumer: 5b-2. Response with the NetworkSliceController \n information (DN of NetworkSliceController and values of its attribute)

end alt

@enduml

End of Third change