# 3GPP TSG-SA5 Meeting #152	S5-237520

# 13 - 17 November 2023, Chicago, IL, USA

# Source:	Nokia, Nokia Shanghai Bell

# Title:		Rel-18 Input to DraftCR 28.105 ML entity versions and artefacts

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-235988 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

An ML Entity could be identified in three main scenarios with possibly three different names:

- The vendor use-case-related name: name identifying the object provided by the vendor/developer using vendor-specific naming convention that could incorporate the Use case for which the ML entity was created (e.g., VendorX _ DLB_Urban). The vendor name can only be changed by the Model producer/vendor.

- The ML entity behavioral version: The naming that identifies different versions of the same ML entity. ML entity version changes whenever the model architecture, Hyperparameters or internal Logic/input/output mappings are changed, (e.g., VendorX_DLB_Urban_Feb2023). Newer versions are typically indicative of improved performance similar to software versions. New versions are created by the Model producer/vendor, so vendor name can only be changed by the Model producer/vendor.

- A ML entity artifact version name: can identify different trained versions of the same ML entity version, i.e., the ML entity artifact version is a by-product of the training process. Training using a different data set or using different training validation criteria (e.g., number of Epochs, acceptable accuracy, etc.) will result in a different artifact version. In other words, different ML entity artifact versions could produce different outputs when presented with the same input data.

Note that each of these may also have an instance which is identified using the corresponding DN.

TS105 currently describes one identifier and 1 version but it is not clear which of the three candidate characteristics are referred to by these two attributes.

| mLEntityId | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. |
| --- | --- |
| mLEntityVersion | It indicates the version number of the ML entity. allowedValues: N/A. |

Accordingly, changes are required.

- In the context of SA5, both the vendor use-case-related name and the ML entity behavioral version name are assigned before the ML entity is introduced into the network /management system. In that respect, for SA5 related descriptions, they can be captured by a single name: ML entity identifier. However, the description of ML entity identifier should be extended to clarify this.

- The current description of the ML entity version does not distinguish between the ML entity behavioral version and the artifact version. The description of the ML entity version should be extended to clarify this difference.

This pCR adds the description of configurable features and requirements for AI/ML-based Distributed Energy Saving Functions.

# 4	Detailed proposal

Start of First change

# 4A	AI/ML management functionality and service framework

## 4A.0	AI/ML operational workflow

### 4A.0.1 AI/ML operational workflow Processes

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN, and management system), the generic AI/ML operational workflow in the lifecycle of an ML entity, is depicted in Figure 5.0-1.

Figure 5.0-1: AI/ML operational workflow

The workflow involves 4 main phases; training, emulation, deployment, and inference phase. The main tasks for each phase are briefly described below:

Training phase:

-	ML model training: training, including initial training and re-training, of an ML model or a group of ML models. It also includes validation of the ML entity to evaluate the performance when the ML entity performs on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML model associated with that entity needs to be re-trained. The ML model training is the initial phase of the workflow.

-	ML testing: testing of the validated ML entity to evaluate the performance of the trained ML model when it performs on testing data. If the testing result meets the expectation, the ML entity may proceed to the next phase, otherwise the ML model associated with that entity may need to be re-trained.

Emulation phase:

-	ML emulation: running an ML entity for inference in an emulation environment. The purpose is to evaluate the inference performance of the ML entity in the emulation environment prior to applying it to the target network or system.

NOTE: 	The emulation phase is considered optional and can be skipped in the AI/ML operational workflow.

Deployment phase:

-	ML entity loading: the process (a.k.a. a sequence of atomic actions) of making a trained ML entity available for use at the target AI/ML inference function.

The deployment phase may not be needed in some cases, for example when the training function and inference function are co-located.

Editor’s note:

1.	The relationship between SA5 loading processe and RAN, SA1 (model transfer) and SA2 (NWDAF) processes may be investigated later.

2.	The term “loading” may be revisited.

Inference phase:

-	AI/ML inference: performing inference using the ML entity by the AI/ML inference function.

### 4A.0.2	AI/ML Entity identifiers

An ML Entity introduced into the network or managed system indludes 2 base characteristics:

- The vendor use-case with a corresponding name that identifies the object developed by the vendor/developer named using vendor-specific naming convention that could incorporate the use case for which the ML entity was created (e.g., VendorX _DLB_Urban).

- The ML entity behavioral variation with corresponding naming that identifies different versions of the same base ML entity. ML entity version changes whenever the model architecture, Hyperparameters or internal Logic/input/output mappings are changed, (e.g., VendorX_DLB_Urban_Feb2023 or VendorX_DLB_Urban_DNNFeb2023). Newer versions are typically indicative of improved performance similar to software versions.

For ML management purposes, both the vendor use-case-related name and the ML entity behavioral version name are assigned before the ML entity is introduced into the network /management system. The ML entity identifier indicates the name assigned to an ML entity by the ML entity vendor when ML entity is introduced for the first time into the network or the Management system. The name includes both the base name and any behavior variation, e.g., VendorX_DLB_Urban_DNNFeb2023.

When an ML entity is trained a new artifact is created which may have different performance from another artefact of the same ML entity. The different training may differ in the used data sets or training validation criteria (e.g., number of Epochs, acceptable accuracy, etc.) and as such will result in artifacts with different performance. The ML entity version identifies the different artefacts created from the different training processes, e.g., "CityA_202307" which may indicate training done in July 2012 using data on CityA.

Next change

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntityId | It identifies the ML entity and the ML entity identifier indicates the name assigned to an ML entity by the ML entity vendor when ML entity is introduced for the first time into the network or the Management system The string for the mLEntityId includes both the base name (using e.g., a vendor-specific naming convention that incorporates the use case for which the ML entity was created) and any behavior variation. An example mLEntityId may be VendorX_DLB_Urban_DNNFeb2023 The combination of mLEntityId and mLEntityVersion is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion | It indicates the version number of an artifact of the ML entity created from the training process, which may have different performance from another artefact of the same ML entity.An example name may for example be "CityA_202307" which may indicate training done in July 2012 using data on CityA. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTraining | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING ” + DN of the MLEntity being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML entity for inference. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTrainRef | It identifies the DN of the MLEntity requested to be trained. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEnityGeneratedRef | It identifies the DN of the MLEntity generated by the ML training. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityRepositoryRef | It idenfifies the DN of the MLEntityRepository. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML entity when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityIdList | It identifies a list of ML entities. allowedValues: N/A. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, TESTING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTestRef | It identifies the DN of the MLEntity requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTesting | It indicates the performance score of the ML entity when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| supportedPerformanceIndicators | It identifies a specific PerformanceIndicator of an ML entity. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML training for the ML entity Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML testing for the ML entity. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| mLUpdateId | It identifies the function responsible for MLUpdate allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestID | It identifies a request for MLUpdate allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateJobID | It identifies an update process instantiated by the function responsible for MLUpdate It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateJobRef | It is the DN of the mLUpdateJob MOI that represents the process of updating an ML entity. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availMLCapabilityReportRef | It is the DN of the availMLCapabilityReport MOI that represents the reports of the available capabilities. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the mLUpdateReport MOI that represents the reports of the ML update outcomes. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availMLCapabilityReportID | It indicates the identifier of an report providing information on (a newly) available ML capabilities that can be applied for the update. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| availableMLCapability | It indicates an available version of ML capabilities that can be used for update Note: The nature of is MLCapability FFS, to be agreed in another pCR | type: MLCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| achievedPerformanceGain | It indicates the performance gain that is achieved by updating the AI/ML entity or network function. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | Type: TimeWindow multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityRef | It indicates the list of references to MLEntity instances that can be updated. | Type: DN multiplicity: 1 .. * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| memberMLEntityRefList | It identifies the list of member ML entities within a level of an ML entity coordination group. allowedValues: DN list | Type: DN (see TS 32.156 [13]) multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTrainRef | It identifies the DN of the MlEntityCoordinationGroup requested to be trained. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEnityCoordinationGroupGeneratedRef | It identifies the DN of the MlEntityCoordinationGroup generated by the ML training. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTestRef | It identifies the DN of the MlEntityCoordinationGroup requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLEntity. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceTrainedMLEntityRef | It identifies the DN of the source trained MLEntity whose copy has been loaded from the ML entity repository to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingRequest.requestStatus | It describes the status of a particular ML entity loading request. allowedValues: NOT_STARTED, LOADING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLEntityLoadingRequest.cancelRequest | It indicates whether the MnS consumer cancels the ML entity loading request. Setting this attribute to "TRUE" cancels the ML entity training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " LOADING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLEntityToLoadRef | It identifies the DN of a trained MLEntity requested to be loaded to the target inference function(s). | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| policyForLoading | It provides the policy for controlling ML entity loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML entity to be loaded, and the second threshold is related to the existing ML entity being used for inference. | Type: AiMlManagementPolicy multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdList | It provides the list of threshold. | Type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingProcess.cancelProcess | It indicates whether the MnS consumer cancels the ML entity loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.suspendProcess | It indicates whether the MnS consumer suspends the ML entity loading process. Setting this attribute to "TRUE" suspends the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.resumeProcess | It indicates whether the MnS consumer resumes the ML entity loading process. Setting this attribute to "TRUE" resumes the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "SUSPENDED". Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingRequestRef | It identifies the DN of the associated MLEntityLoadingRequest. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingPolicyRef | It identifies the DN of the associated MLEntityLoadingPolicyRef. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| performanceMetricName | It identifies the name of the performance metric or measurement. AllowedValues: the name of the performance metrics specified for the ML training phase and AI/ML inference phase. | Type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdDirection | It indicates the direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated threshold is triggered only when the subject performance metric value is equal to or greater than the threshold value. The threshold is not triggered, when the performance metric value is smaller than the threshold value. Vice versa, When the threshold direction is configured to "DOWN", the associated threshold is triggered only when the subject performance metric value is equal or smaller than the threshold value. The threshold is not triggered, when the performance metric value s greater than the threshold value. allowedValues: - UP - DOWN | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdValue | It specifies the value against which the monitored performance metric is compared. allowedValues: float or integer | Type: Union multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| LoadedMLEntityRef | It identifies the DN of the MLEntity that has been loaded to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| activationStatus | It describes the activation statuss. allowedValues: ACTIVATED, DEACTIVATED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML testing, the data set is the testing data set. |  |  |

### 7.5.2	Constraints

None.

End of Changes