3GPP TSG-SA5 Meeting #152 	S5-237535

Chicago, US, 13 - 17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0068 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel 18 CR TS 28.105 Resolve issues related to the usage of confidenceIndication attribute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Resolve issues related to the usage of confidenceIndication attribute. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified the description of confidenceIndication attribute. Added “requiredModelConfidence” attribute to MlTrainingRequest IoC. Updated the “confidenceIndication” attribute in MlTrainingReport IoC to “modelConfidenceIndication”. Corresponding stage 3 solution set definitions updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on the usage of confidenceIndication, decisionConfidenceScore and ML model convergence. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3a.1.2.2, 7.3a.1.2.3, 7.5.1, B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/901 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 7.3a.1.2.2	MLTrainingRequest

###### 7.3a.1.2.2.1	Definition

The IOC MLTrainingRequest represents the ML model training request that is created by the ML training MnS consumer.

The MLTrainingRequest MOI is contained under one MLTrainingFunction MOI. Each MLTrainingRequest is associated to at least one MLEntity.

The MLTrainingRequest may have a source to identify where it is coming from, and which may be used to prioritize the training resources for different sources. The sources may be for example the network functions, operator roles, or other functional differentiations.

Each MLTrainingRequest may indicate the expectedRunTimeContext that describes the specific conditions for which the MLEntity should be trained.

In case the request is accepted, the ML training MnS producer decides when to start the ML training. Once the MnS producer decides to start the training based on the request, the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training;

-	prepares and selects the required training data, with consideration of the consumer’s request provided candidate training data if any. The ML training MnS producer may examine the consumer's provided candidate training data and select none, some or all of them for training. In addition, the ML training MnS producer may select some other training data that are available in order to meet the consumer’s requirements for the MLentity training;

-	trains the MLEntity using the selected and prepared training data.

The MLTrainingRequest may have a requestStatus field to represent the status of the specific MLTrainingRequest:

-	The attribute values are "NOT_STARTED", "TRAINING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to "TRAINING_IN_PROGRESS", the ML training MnS producer instantiates one or more MLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MLT MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED".

###### 7.3a.1.2.2.2	Attributes

Table 7.3a.1.2.2.1-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceType | CM | T | F | F | T |
| candidateTrainingDataSource | O | T | T | F | T |
| trainingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | F | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| requiredModelConfidence | O | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLEntityToTrainRef | CM | T | F | F | T |

###### 7.3a.1.2.2.3	Attribute constraints

Table 7.3a.1.2.2.3-1

| Name | Definition |
| --- | --- |
| inferenceType Support Qualifier | Condition: MLTrainingRequest MOI represents the request for initial ML training. |
| mLEntityToTrainRef Support Qualifier | Condition: MLTrainingRequest MOI represents the request for ML re-training. |

###### 7.3a.1.2.2.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

Next Change

##### 7.3a.1.2.3	MLTrainingReport

###### 7.3a.1.2.3.1	Definition

The IOC MLTrainingReport represents the ML model training report that is provided by the training MnS producer.

The MLTrainingReport MOI is contained under one MLTrainingFunction MOI.

###### 7.3.1.2.3.2	Attributes

Table 7.3a.1.2.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntityId | M | T | F | F | T |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| modelConfidenceIndication | O | T | F | F | T |
| modelPerformanceTraining | M | T | F | F | T |
| areNewTrainingDataUsed | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingProcessRef | M | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |
| mLEnityGeneratedRef | M | T | F | F | T |

###### 7.3a.1.2.3.3	Attribute constraints

Table 7.3a.1.2.3.3-1

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was requested by the MnS consumer (via MLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was not initial training (i.e. the model has been trained before). |

###### 7.3a.1.2.3.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

Next Change

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntityId | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see 3GPP TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| requiredModelConfidence | It indicates the average confidence (in unit of percentage) vaue that the ML model should meet on training or testing data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelConfidenceIndication | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model, determined by the difference between actual labels and recommended labels by the ML model. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestStatus | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED, REQUIRED_MODEL_CONFIDENCE_NOT_ACHIEVABLE. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the ML training process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion | It indicates the version number of the ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelperformanceTraining | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - COLLECTING_DATA - PREPARING_TRAINING_DATA - TRAINING The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", “model confidence”, etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelRequest | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendRequest | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| cancelProcess | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendProcess | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML entity for inference. | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTrainRef | It identifies the DN of the MLEntity requested to be trained. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEnityGeneratedRef | It identifies the DN of the MLEntity generated by the ML training. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityRepositoryRef | It idenfifies the DN of the MLEntityRepository. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. |  |  |

Next Change

Annex B (normative):
OpenAPI definition of the AI/ML NRM

# B.1	General

This annex contains the OpenAPI definition of the AI/ML NRM in YAML format.

The information models of the AI/ML NRM are defined in clause 7.

Mapping rules to produce the OpenAPI definition based on the information model are defined in 3GPP TS 32.160 [14].

# B.2	Solution Set (SS) definitions

## B.2.1	OpenAPI document "TS28105_AiMlNrm.yaml"

openapi: 3.0.1

info:

title: AI/ML NRM

version: 18.1.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MLContext:

type: object

properties:

inferenceEntityRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

dataProviderRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

RequestStatus:

type: string

enum:

- NOT_STARTED

- TRAINING_IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

PerformanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

type: number

format: float

decisionConfidenceScore:

type: number

format: float

TrainingProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in “genericNrm.yaml” with specialisations for usage in the "MLTrainingProcess".

type: object

properties:

mLTrainingProcessId:

type: string

status:

type: string

enum:

- RUNNING

- CANCELLING

- CANCELLED

- SUSPENDED

- FINSHED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- COLLECTING_DATA

- PREPARING_TRAINING_DATA

- TRAINING

resultStateInfo:

type: string

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLEntityRepository:

$ref: '#/components/schemas/MLEntityRepository-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

MLTrainingFunction:

$ref: '#/components/schemas/MLTrainingFunction-Multiple'

MLEntityRepository:

$ref: '#/components/schemas/MLEntityRepository-Multiple'

MLTrainingFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

mLEntityRepositoryRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MLTrainingRequest:

$ref: '#/components/schemas/MLTrainingRequest-Multiple'

MLTrainingProcess:

$ref: '#/components/schemas/MLTrainingProcess-Multiple'

MLTrainingReport:

$ref: '#/components/schemas/MLTrainingReport-Multiple'

MLTrainingRequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLEntityId:

type: string

inferenceType:

type: string

candidateTrainingDataSource:

type: array

items:

type: string

trainingDataQualityScore:

type: number

format: float

trainingRequestSource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: '#/components/schemas/MLContext'

performanceRequirements:

$ref: '#/components/schemas/PerformanceRequirements'

requiredModelConfidence:

type: integer

cancelRequest:

type: boolean

suspendRequest:

type: boolean

mLEntityToTrainRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingProcess-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLTrainingProcessId:

type: string

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/TrainingProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLTrainingReport-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mLEntityId:

type: string

areConsumerTrainingDataUsed:

type: boolean

usedConsumerTrainingData:

type: array

items:

type: string

modelConfidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

areNewTrainingDataUsed:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingProcessRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

lastTrainingRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

mLEnityGeneratedRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MLEntity-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

mLEntityId:

type: string

inferenceType:

type: string

mLEntityVersion:

type: string

expectedRunTimeContext:

$ref: '#/components/schemas/MLContext'

trainingContext:

$ref: '#/components/schemas/MLContext'

runTimeContext:

$ref: '#/components/schemas/MLContext'

MLEntityRepository-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

mLRepositoryId:

type: string

MLEntity:

$ref: '#/components/schemas/MLEntity-Multiple'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

MLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingFunction-Single'

MLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingRequest-Single'

MLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingProcess-Single'

MLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MLTrainingReport-Single'

MLEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/MLEntity-Single'

MLEntityRepository-Multiple:

type: array

items:

$ref: '#/components/schemas/MLEntityRepository-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/MLTrainingFunction-Single'

- $ref: '#/components/schemas/MLTrainingRequest-Single'

- $ref: '#/components/schemas/MLTrainingProcess-Single'

- $ref: '#/components/schemas/MLTrainingReport-Single'

- $ref: '#/components/schemas/MLEntity-Single'

- $ref: '#/components/schemas/MLEntityRepository-Single'

End of Changes