3GPP TSG-SA5 Meeting #152 	S5-237565

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0035 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.254 SUPI and APF ID Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide the Application ID on the Charging Data Request Message used for Converged Charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the Charging Data messages with Application Identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subscriber Identifier IE continues to be used for identification of the application |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.255 CR 0469 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

##### 6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for NEF converged charging.

Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element | Category |  | Description |
| --- | --- | --- | --- |
| Session Identifier | OC |  | Described in TS 32.290 [57] |
| Subscriber Identifier | OM |  | Described in TS 32.290 [57] |
| Application Identifier | OC |  | Described in TS 32.290 [57], and holds the identifier of the AF |
| NF Consumer Identification | M |  | Described in TS 32.290 [57] |
| Charging Identifier | OM |  | Described in TS 32.290 [57] |
| Invocation Timestamp | M |  | Described in TS 32.290 [57] |
| Invocation Sequence Number | M |  | Described in TS 32.290 [57] |
| Retransmission Indicator |  | - | This field is not applicable. |
| One-time Event | OC |  | Described in TS 32.290 [57]. |
| One-time Event Type |  | OC | Described in TS 32.290 [57]. |
| Notify URI | - |  | This field is only applicable for the notification of abort charging.. |
| Supported Features | OC |  | Described in TS 32.290 [57] |
| Triggers | OC |  | This field is described in TS 32.290 [57] and holds the NEF specific triggers described in clause 5.4.1.2 |
| Multiple Unit Usage | OC |  | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M |  | Described in TS 32.290 [57] |
| Requested Unit | OC |  | Described in TS 32.290 [57] |
| Used Unit Container | OC |  | Described in TS 32.290 [57] |
| NEF API Charging Information | OM |  | This field holds the NEF API specific information described in clause 6.3.1.4 |

Second change

### B.2.4.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by CEF for 5G VN group management converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table B.2.4.3-1 defines the basic structure of the supported fields in the Charging Data Request message for CEF converged charging.

Table B.2.4.3-1: Supported fields in Charging Data Request message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Subscriber Identifier |  | - |
| AF Identification |  | E |
| NF Consumer Identification |  | E |
| Charging Identifier |  | - |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | E |
| One-time Event |  | E |
| One-time Event Type |  | E |
| Retransmission Indicator |  | E |
| Notify URI |  | - |
| Supported Features |  | E |
| Service Specification Information |  | E |
| Triggers |  | E |
| Multiple Unit Usage |  | - |
| 5G VNGM Charging Information |  | E |

Table B.2.4.3-2 defines the basic structure of the supported fields in the Charging Data Response message for CEF converged charging.

Table B.2.4.3-2: Supported fields in Charging Data Response message

| Information Element | Analytics and Performance | CEF |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Sequence Number |  | E |
| Session Failover |  | - |
| Supported Features |  | E |
| Triggers |  | E |
| Multiple Unit information |  | - |

End of Changes