3GPP TSG-SA5 Meeting #152 	S5-237575

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-18 pCR TR 28.826 Update of evaluation and conclusions clause 5.3

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Updating the evaluation and conclusions.

# 4	Detailed proposal

First change

### 5.3.6	Evaluation

Solution #3.1, #3.2, #3.3, #3.4, and #3.5 all solve key issues #3a, #3b, and #3c.

-	Solution #3.1: the new service id list, in MultipleUnitUsage type, holds the used service ids which can provide the service level information (e.g. video, email, file transfer, VoIP) which will make it easier for the CHF to allocate quota for the rating group, and preserves backward compatibility. It may cause confusion as to if this is the service id for the requested units or the reported used units.

-	Solution #3.2: the new service id list holds the used service ids which can provide the service level information (e.g. video, email, file transfer, VoIP) and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group. The used service id can be connected to the QoS and is currently in the used unit container type.

- Solution #3.3: the new QoS information which can provide the service quality and service experience requirement information (e.g. the authorized bandwidth, latency) holds the currently applied QoS for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for different service quality requirement of the same rating group.

PCC rules include the static and dynamic PCC rules. For dynamic PCC rules, service Ids and QBCs are dynamically determined.

- Solution #3.4: the new additional information holds both the QoS information and service ids the currently applied for the rating group and is part of the requested units which will make it easier for the CHF to allocate quota for the rating group.

### 5.3.7	Conclusions

For key issues #3.a, #3b and #3c a new attribute holding the possible service ids for a rating group can be added using solution #3.1 and #3.2, because it’s used whenever there is a quota management request and/or usage reporting. Further extensions can be done if the service id solution is deemed insufficient.

The advantage of using solution #3.1 it’s about preserving backward compatibility, though, it’s necessary to detail in which cases the service id is used.

End of changes