3GPP TSG-SA5 Meeting #152	S5-237588

Chicago, US, 11 - 17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 ML inference NRM clarifications and corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Basic text for the use case of AI/ML inference function configuration management has been discussed and agreed in the draft CR in SA5#151. The text require some further clarifications and correction to alighn with terms used by RAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added some clarifications and corrections to align with the terms used in RAN specifications.. Also used the same name of the functions as defined in RAN specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing text can lead to confusion and speculation, specialy by RAN WGs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 and subclauses. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-237244 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

This input to draft CR aims to add further necessary clarifications and corrections to existing text for the use case of AI/ML inference function configuration management which has been discussed and agreed in the draft CR in SA5#151.

# 4	Detailed proposal

1st modified section

### 7.3a.4	Information model definitions for ML inference phase

#### 7.3a.4.1	Class diagram

##### 7.3a.4.1.1	Relationships

Figure 7.3a.4.1.1-1: NRM fragment for ML Inference Control

Figure 7.3a.4.1.1-2: NRM fragment for intelligence functions

##### 7.3a.4.1.2	Inheritance

Figure 7.3a.4.1.2-1: ML Inference Inheritance Relations

Figure 7.3a.4.1.2-2: Inheritance Hierarchy for intelligence functions

Editor note: the relation between AiMlDNESFunction, AiMlDMOFunction, AiMlDLBFunction and DESManagementFunction[x], DMROFunction[x], DLBOFunction[x] are FFS.

Next modified section

### 6.5.4	AI/ML inference function configuration management

#### 6.5.4.1	Description

TBD

#### 6.5.4.2	Use cases

##### 6.5.4.2.1	Managing NG-RAN AI/ML-based distributed Network Energy Saving

An NG-RAN AI/ML-based distributed Network Energy Saving function may use one or more ML entities to derive energy saving recommendations.

This NG-RAN AI/ML-based distributed Network Energy Saving function needs to be managed.

##### 6.5.4.2.2	Managing NG-RAN AI/ML-based distributed Mobility Optimization

An AI/ML-based distributed Mobility Optimization function may use one or more ML entities to derive handover recommendations.

This NG-RAN AI/ML-based distributed Mobility Optimisation function needs to be managed.

##### 6.5.4.2.3	Managing NG-RAN AI/ML-based distributed Load Balancing

An NG-RAN AI/ML-based distributed Load Balancing function may use one or more ML entities to derive load balancing recommendations.

This NG-RAN AI/ML-based distributed Load Balancing function needs to be managed.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF-01 | The MnS producer of NG-RAN AI/ML-based distributed Network Energy Saving should enable an authorized MnS consumer to request to manage the Network Energy Saving inference function | Managing AI/ML-based for NG-RAN distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- AI/ML_INF-02 | The MnS producer of NG-RAN AI/ML-based distributed Mobility Optimization should enable an authorized MnS consumer to request to manage the Mobility Optimization inference function | Managing AI/ML-based for NG-RAN distributed Mobility Optimization (clause 6.5.4.2.2) |
| REQ- AI/ML_INF-03 | The MnS producer of NG-RAN AI/ML-based distributed Load Balancing should enable an authorized MnS consumer to request to manage the Load Balancing inference function | Managing AI/ML-based for NG-RAN distributed Load Balancing (clause 6.5.4.2.3) |

Next modified section

##### 7.3a.4.2.5	AiMlDNESFunction

###### 7.3a.4.2.5.1	Definition

This IOC represents the inference function supporting the NG-RAN AI/ML-based distributed Network Energy Saving (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.4.2.5.3	Attribute constraints

None.

###### 7.3a.4.2.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.6	AiMlDMOFunction

###### 7.3a.4.2.6.1	Definition

This IOC represents the inference function supporting the NG-RAN AI/ML-based distributed Mobility Optimization (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

###### 7.3a.4.2.6.3	Attribute constraints

None.

###### 7.3a.4.2.6.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3a.4.2.7	AiMlDLBFunction

###### 7.3a.4.2.7.1	Definition

This IOC represents the inference function supporting the NG-RAN AI/ML-based distributed Load Balancing (see 3GPP TS 38.300 [16]).

###### 7.3a.4.2.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |

Next modified section

# A.13	PlantUML code for Figure 7.3a.4.1.2-2: Inheritance Hierarchy for intelligence functions

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class AiMlInferenceFunction << InformationObjectClass >>

class AiMlDMOFunction <<InformationObjectClass>>

class AiMlDLBFunction <<InformationObjectClass>>

class AiMlDNESFunction <<InformationObjectClass>>

class MDAFunction <<InformationObjectClass>>

ManagedFunction <|-- AiMlInferenceFunction

AiMlInferenceFunction <|-- AiMlDMOFunction

AiMlInferenceFunction <|-- AiMlDLBFunction

AiMlInferenceFunction <|-- AiMlDNESFunction

AiMlInferenceFunction <|-- MDAFunction

@enduml

End of modifications