3GPP TSG-SA5 Meeting #152 	S5-237600

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | DraftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add solution for MDA assisted control plane congestion analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of Control plane congestion analysis is approved and added in Draft CR eMDAS_ph2 (See S5-235762). This document adds the MDA solution for this use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds solution for MDA assisted control plane congestion analysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDA solution for Control plane congestion analysis is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.4.x.1	5GC Control plane congestion analysis

8.4.x.1.1	MDA type

The MDA type for 5GC control plane congestion analysis is: ResourceAnalytics.5GCControlPlaneCongestionAnalysis.

8.4.x.1.2	Enabling data

The enabling data for ResourceAnalytics.5GCControlPlaneCongestionAnalysis MDA type are provided in table 8.4.x.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2-1: Enabling data for 5GC control plane congestion analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Registration procedure related measurements for AMF. | Number of registration requests (clause 5.2.2 of TS 28.552 [4]) Mean time of Registration procedure (clause 5.2.2.9 of TS 28.552 [4]) |
|  | Service Request procedure related measurements for AMF. | Number of service requests (clause 5.2.3.3 and clause 5.2.3.4 of TS 28.552 [4]) |
|  | Number of PDU sessions measurements for SMF | Number of PDU sessions (clause 5.3.1 of TS 28.552 [4]) |
|  | QoS flows measurements for SMF | QoS flows monitoring (clause 5.3.2 of TS 28.552 [4]) |
|  | VR (including Virtual CPU, Virtual Memory, and Virtual Disk) usage of NF | VR usage of NF (clause 5.7.1 of TS 28.552 [4]) |
| Alarm notifications | Alarm information, e.g. the alarm notification of network functions. | Alarm information and notifications as per TS 28.532 [11] |
| Configuration data | MOIs of 5GC NFs. | 5GC NRM as defined in TS 28.541 [15] |

8.4.x.1.3	Analytics output

The specific information elements of the analytics output for control plane congestion analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.3-1.

Table 8.4.3.1.3-1: Analytics output for 5GC control plane congestion analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| AffectedObject | Indication of 5GC NFs where congestion issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| CPCongestionIssueID | This filed holds the ID of this control plane congestion analysis which is reported. When reports, this identifier can be used to provide the information to management system to maintain. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may be (but not limited to): - scale out a list of NFs; | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

End of Change