3GPP TSG-SA5 Meeting #152 	S5-237615

Chicago, US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0010 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Correction on procedure figure for NPN provisioning by a network slice of a PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are typos in the procedure figure 6.3.1-1 which is for NPN provisioning by a network slice of a PLMN. It is proposed to fix this from Rapporteur point of view. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixing the typos in the procedure figure 6.3.1-1 for NPN provisioning by a network slice of a PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Typos exist in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, Annex B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.3.1	Solution for NPN provisioning by a network slice of a PLMN

A mobile network operator (playing the role of NPN-SP) decides to provision a PNI-NPN for private use by an enterprise (playing the role of NPN-SC) in the form of a network slice of a PLMN. NPN-SP and NPN-OP are assumed to be same in this case for simplicity in understanding.

Figure 6.3.1-1: Procedure for NPN provisioning by a network slice of a PLMN

The main aspects of NPN provisioning by a network slice of a PLMN illustrated in Figure 6.3.1-1 include:

1)	The NPN-SC provides the NPN related SLA requirements to NPN-SP. These requirements specify NPN related SLS according to different vertical industry requirements (e.g. coverage requirement within a specific geographic area, downlink/uplink throughput requirements, latency requirement, etc.) together with other business related information (e.g. NPN lifetime, NPN slice charging / accounting, etc.). The work flow between NPN-SP and NPN-SC is out of scope of the present document.

2)	The NPN-SP maps these SLS requirements into ServiceProfile attributes (see TS 28.541 [7]).

3)	The NPN-SP sends ServiceProfile in "AllocateNSI" request to NSMS_P.

4)	Then the NSMS_P follows the NSI allocation procedure as described in clause 7.2 in TS 28.531 (this implicitly follows sub-steps like deriving slice profile requirements for subnets from service profile, checking possibility of reusing existing or creating new slice, allocation of NSSI etc. as per procedure defined in TS 28.531 [8]).

-	The NG-RAN domain NSSMS_P determines to utilize the existing NG-RAN NE(s) or new NG-RAN NEs that are deployed in the PLMN network or deployed locally at the enterprise's premise or in the factory.

Based on the access policy from operator, from which the NSSMS_P can derive rules like days/time slots/occasions etc. for which an NPN UE can access a CAG cell, the NSSMF assigns the CAG ID identifying the CAG cells which enables the control of UE's access to related PNI-NPN. The NRCellDU should be configured with the CAG ID to support access control for PNI-NPN UEs. The details of NRCellDU see TS 28.541 [7].

-	The 5GC domain NSSMS_P determines to utilize new or existing 5GC NF(s) of the 5GC part that are deployed in the PLMN network.

-	If any, the TN domain related requirements are provided to the management system of TN domain.

5)	The NSMS_P sends NSI allocation result in AllocateNsi response to the NPN-SP including the relevant network slice instance information.

Next Change

## B.3	Procedure for NPN provisioning by a network slice of a PLMN

The following PlantUML source code is used to describe the procedure for NPN provisioning by a network slice of a PLMN, as depicted by Figure 6.3.1-1:

@startuml

"NPN-SC" -> "NPN-SP": 1. SLA requirements\n(coverage, DL/UL throughout, latency, NPN lifetime)

"NPN-SP" -> "NPN-SP": 2. Map SLS requirements \n into ServiceProfile

"NPN-SP" -> "NSMS_P":3. AllocateNsi request

note over NSMS_P, NSSMS_P: 4. NSI Allocation \n (Decide to create a new NSI \n or use an existing NSI)

"NSMS_P" -> "NPN-SP":5. AllocateNsi response

skinparam sequenceMessageAlign center

@enduml

End of changes