3GPP TSG-SA5 Meeting #152 	S5-237618

Chicago, US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0498 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add PLMN granularity for peak PRB used for data traffic measurements for NG-RAN MOCN network sharing scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS_ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This submission implements the solution for “PLMN granularity requirement of performance measurements for MOCN” as recommended in TR 28.835 (Issue #8). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN granularity for peak PRB used for data traffic measurements for NG-RAN MOCN network sharing scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.9, 5.1.1.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.2.9	Peak DL PRB used for data traffic

a)	This measurement provides the maximum number of PRBs used in downlink for data traffic. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI and subcounters per supported PLMN ID.

b)	SI.

c)	Each measurement is obtained by sampling at a pre-defined interval, the PRBs used for DL data traffic transmission per S-NSSAI and per PLMN ID during a time period T, and selecting the sample with the maximum value from the samples collected in a given period.

d)	Each measurement is a single integer value. If the optional measurements are perfomed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMNs.

e)	RRU.MaxPrbUsedDl, or optionally RRU.MaxPrbUsedDl.QoS, where the QoS identifies the target quality of service class and RRU.MaxPrbUsedDl.SNSSAI, where SNSSAI identifies the S-NSSAI, and RRU.MaxPrbUsedDl.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the DL PRB load of the radio physical layer per S-NSSAI to support RRM resources optimization (see TS 28.313 [30]).

5.1.1.2.10	Peak UL PRB used for data traffic

a)	This measurement provides the number of PRBs used in uplink for data traffic. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI and subcounters per supported PLMN ID.

b)	SI

c)	Each measurement is obtained by sampling at a pre-defined interval, the PRBs used for UL data traffic transmission per S-NSSAI and per PLMN ID during a time period T, and selecting the sample with the maximum value from the samples collected in a given period.

d)	Each measurement (number of PRBs) is a single integer value. If the optional measurements are perfomed, the number of measurements is equal to the number of QoS levels and the number of supported S-NSSAIs and the number of supported PLMNs.

.

e)	RRU.MaxPrbUsedUl, or optionally RRU.MaxPrbUsedUl.QoS, where the QoS identifies the target quality of service class and RRU.MaxPrbUsedUl.SNSSAI, where SNSSAI identifies the S-NSSAI, and RRU.MaxPrbUsedUl.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the UL PRB load of the radio physical layer per S-NSSAI to support RRM resources optimization (see TS 28.313 [30]).

End of Change