3GPP TSG-SA5 Meeting #152	S5-237641

# Chicago, US, 13-17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0159 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.423 Add the missing Trace record content for CHF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current trace specifications completely missed to include the entities CHF and NWDAF. These entities have been added in 23.501 as part of release 15 and not considered for tracing in SA5 specifications. Consequently, the operators cannot trace any of the interfaces of these 3GPP NFs currently. TS 32.421 lists explicitly all the 3GPP NFs and their corresponding interfaces that shall be traced. CHF and NWDAF has not been listed there and hence there is no requirement to trace their interfaces. FASMO check : CR author’s view is that these changes are essential from Release 15 to Release 18 as there is no means in any of these release to trace the interfaces of PCF and NWDAF. The risk of this is that the audience interested in tracing any of these NFs may create a local solution different from SA5 solution and this will undermine the trace architecture of SA5. There is an LS exchange between SA5 Charging and GSMA where GSMA is proposing few options to implement tracing of CHF and basically reinventing the wheel which has been already solved in SA5 using Trace and MDT. Hence it is essential to have these changes since the release when these NFs are introduced so that other SDOs like GSMA can refer to SA5 solution and not reinvent the wheel again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Trace record content has been added for CHF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No specification for trace record content for CHF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.a (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.421 CR 0121 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 4.a	CHF Trace Record Content

The following table shows the trace record content for CHF.

The trace record is the same for management based activation and for signalling based activation.

CHF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

Table 4.a.1 : CHF Trace Record Content

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| N28 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | PCF ID of the connected PCF CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from N28 messages between the traced CHF and the PCF. |
|  | Encoded* | X | X | M | Raw Messages: N28 messages between the traced CHF and the PCF. The encoded content of the message is provided. |
| N40 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | SMF ID of the connected SMF CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from N40 messages between the traced CHF and the SMF. |
|  | Encoded* | X | X | M | Raw Messages: N40 messages between the traced CHF and the SMF. The encoded content of the message is provided. |
| N41 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | AMF ID of the connected AMF in HPLMN CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from N41 messages between the traced CHF and the AMF in HPLMN. |
|  | Encoded* | X | X | M | Raw Messages: N41 messages between the traced CHF and the AMF in HPLMN. The encoded content of the message is provided. |
| N42 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | AMF ID of the connected AMF in VPLMN CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from N42 messages between the traced CHF and the AMF in VPLMN. |
|  | Encoded* | X | X | M | Raw Messages: N42 messages between the traced CHF and the AMF in VPLMN. The encoded content of the message is provided. |

Encoded* - the messages are left encoded in the format it was received.

End of Changes