3GPP TSG-SA5 Meeting #152 	S5-237676

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0025 | rev | - | Current version: | V18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.130 Add requirements for NG-RAN MOCN network sharing scenario with multiple Cell Identity broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS_ph2 |  |  |  |  |  | Date: |  |  | 2023-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 32.130 describes “The 3GPP management system of the MOP shall have the capability to configure the individual F1 interface for each POP.” and “The 3GPP management system of the MOP shall have the capability to configure the individual NR cell relation individually for each POP.” for management support for NG-RAN MOCN network sharing scenario with same Cell Identity broadcast. Meanwhile, the requirements need to be added in NG-RAN MOCN network sharing scenario with multiple Cell Identity broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirements for NG-RAN MOCN network sharing scenario with multiple Cell Identity broadcast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.4	Requirements for management support for NG-RAN MOCN network sharing scenario with same Cell Identity broadcast

Req-MOCN_SameCellId_Cfg-CON-1 The 3GPP management system of the MOP shall have the capability to configure NgC and NgU individually for each POP.

Req- MOCN_SameCellId_Cfg-CON-2 The 3GPP management system of the MOP shall have the capability to configure PLMNId individually for each POP.

Req- MOCN_SameCellId_Per-CON-3 The 3GPP management system of the MOP shall have the capability to collect and report some measurements (e.g. active UEs measurements, packet delay measurements) in PLMN granularity for each POP.

Req-MOCN-SameCellId-Cfg-CON-4 The 3GPP management system of the MOP shall have the capability to support operator-specific configurations.

Req-MOCN-SameCellId-Cfg-CON-5 The 3GPP management system of the MOP shall have the capability to support operator-specific performance measurement jobs.

2nd Change

5.1.5	Requirements for management support for NG-RAN MOCN network sharing scenario with multiple Cell Identity broadcast

Req-MOCN-MultiCellId-Cfg-CON-1 The 3GPP management system of the MOP shall have the capability to configure NgC and NgU individually for each POP.

Req-MOCN-MultiCellId-Cfg-CON-2 The 3GPP management system of the MOP shall have the capability to configure PLMN-IdentityInfo (including PLMNId, NR Cell Identity, TAC) individually for each POP.

Req-MOCN-MultiCellId-Perf-CON-3 The 3GPP management system shall have the capability to collect and report some measurements (e.g. active UEs measurements, packet delay measurements) in PLMN granularity for each POP.

Req-MOCN-MultiCellId-Cfg-CON-4 The 3GPP management system of the MOP shall have the capability to configure the common F1 interface for all POPs.

Req-MOCN-MultiCellId-Cfg-CON-5 The 3GPP management system of the MOP shall have the capability to configure the individual F1 interface for each POP.

Req-MOCN-MultiCellId-Cfg-CON-6 The 3GPP management system of the MOP shall have the capability to configure the individual NR cell relation individually for each POP.

Req-MOCN-MultiCellId-Cfg-CON-7 The 3GPP management system of the MOP shall have the capability to support operator-specific configurations.

Req-MOCN-MultiCellId-Cfg-CON-8 The 3GPP management system of the MOP shall have the capability to support operator-specific performance measurement jobs.

End of Changes