3GPP TSG-SA5 Meeting #152 	S5-237686

Chicago, US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0501 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add new measurement for average value of Timing Advance per SS-RSRP and AOA ranges |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.552, there are no measurements which consider SS-RSRP and TA at the same time. Operators can only monitor Timing Advance or RSRP. Timing Advance per SS-RSRP and AOA ranges should be monitored, as it reflects the UE distribution in NR cell with different coverage qualities. The operators can use this information to analyze the cell coverage and the capacity more accurately, thus it is helpful for network planning and network optimization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of “Average value of Timing Advance per SS-RSRP and AOA ranges” measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators can only monitor Timing Advance or RSRP, but cannot monitor Timing Advance covering quality information. It is not enough for accurate network planning and optimization |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.33, A.64 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.33.x Average Value of Timing Advance per SS-RSRP and AOA ranges

a) This measurement provides the average value of timing advance of the UEs which are in the given range of SS-RSRP and AOA for a cell.

b) SI

c) This measurement is obtained by computing the average value of timing advance of the UEs which are in the given range of SS-RSRP and AOA in the statistical period. The average value is obtained by this formula:

Where

denotes the average value of SS-RSRPs received by gNB from the -th UE in the cell within the -th sampling period in the statistical period, while  is in the -th RSRP range andis in the -th AOA range.

denotes the average value of TAs transmitted by the gNB to -th UE in the cell within the-th sampling period in the statistical period, while  is in the -th RSRP range andis in the -th AOA range.

denotes the average value of AOAs of the -th UE in the cell within the -th sampling period in the statistical period, while  is in the -th RSRP range andis in the -th AOA range.

,  and  are all statistical results of the -th UE within the-th sampling period.

The measurement  denotes the average value of timing advance when relative SS-RSRPs are in the -th RSRP range and relative AOAs  are in the -th AOA range.

d)	Each measurement is a real value.

e) L1M.TAAvg.SS-RSRPBin.AoABin,

Where SS-RSRPBin identifies the the range of reported SS-RSRP value, and AoABin identifies the range of reported AOA value.

NOTE: Number of bins and the ranges for SS-RSRP and AOA is left to implementation.

f) NRCellDU

g) Valid for packet switched traffic

h) 5GS

j) One usage of this measurement is to analyse the cell coverage and the capacity, thus help the network planning and network optimization.

2nd Change

A.64	Monitoring of RF performance

Monitoring of the quality of RF signal in the cell is useful for the purpose of network planning and network optimization.

In case the L1-RSRP report function is enabled, measurements of RSRP per beam reported by UEs is a useful metric reflecting RF signal strength. In 5G NR, gNB cells transmit many narrow beams targeting UEs in the cell that result in better link budget and lower interference. However, some areas between beams of neighbouring NR cells, or between the NR cell and the neighbor E-UTRA cell may experience poor coverage or coverage holes. Therefore, it is necessary to optimize the beam coverage by coordinating the beam management function across multiple neighboring NR cells.

The Absolute Timing Advance reflects the distance of the UE from the cell antenna. The distribution of Absolute Timing Advance reflects UE distribution in the NR cell, thus it is helpful for analyzing the coverage and the capacity.

The Average Value of Timing Advance per SS-RSRP and AOA ranges reflects the UE distribution in NR cell with different coverage qualities. It is helpful for analyzing the cell coverage and the capacity more clearly. Thus, it can help the network planning and optimization.

End of Change