3GPP TSG-SA5 Meeting #152	S5-237750

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	pCR 28.865 Add conclusion of issue#3 service assurance for PLC control

# Document for:	Approval

Agenda Item:	6.9.2.4

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	S5-234729: "draft TR 28.865 Study on deterministic communication service assurance"; v0.5.0

3	Rationale

This tdoc addresses the conclusion of issue#3 service assurance for PLC control and provide proposals for the normative work.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

[……]

5.3	Issue #3: Service assurance for PLC control

5.3.1	Description

In TS 22.104, some UCs are provided, e.g. periodic communication for video-operated remote control, real-time streaming data transmission (video data) from a mobile robot to the guidance control system.

For the use case of video operated remote control services, in which video backhaul and remote control are performed together. Remote control services are mainly characterized by small packet control URLLC services. Typical scenarios include remote control of gantry cranes or bridge cranes in smart ports and remote surgery in smart medical care, wireless external I/O of robots, and differential protection in the smart grid.The downlink control signaling has high requirements on delay and reliability. This type of service features small data packets, low bandwidth requirements, and high latency and reliability requirements. This documents provides the characteristics and requirements for its service assurance.

PLC control communication services have two characteristics: periodic small-packet interaction (such as data packets for control operations) and burst small-packet transmission (such as device alarms). The stability of the interaction delay is an important factor that affects the service quality. The stable receiving of messages is better than the fast receiving of messages but with great jitter, and is less likely to trigger alarms and service interruptions.

The three-layer model of service experience, service quality and network performance for PLC control is depicted in figure 4.

Figure 4： Service requirement modelling of PLC control service

Service experience layer: establishes service experience related indicators for PLC control based on subjective and objective evaluation. Analyzes experience-layer KQIs.

Service quality layer: studies the relation between KQIs and QoS indicators. It describes the impacts of indicators such as delay jitter and burst packet loss ratio etc in different scenarios.

Network capability layer: studies the relation between QoS and air interface bandwidth/delay, radio network performance measurements, and core network bandwidth/delay requirements, core network performance measurements and capacity etc.

In summary, some additional attributes may need to be included in the serviceProfile in the following Table A:

Table A: Additional requirements in the serviceProfile

| Attribute Name | Description |
| --- | --- |
| interactiveDelay | For some interactive services, e.g. VR, gaming etc, there are stringent two-way end-to-end latency (UL and DL). See descriptions in TS 22.261 [2]. |
| transInterval | Time difference between two consecutive transfers of application data from an application via the service interface to 3GPP system. See TS 22.104 [3]. |

The above information is used as the assurance objective of the PLC control service. The deterministic communication service assurance should support network preparation based on the assurance objective. The DCSA MnS producer collects related data, analyzes service experience and network performance, optimizes and verifies deterministic communication service experience, and achieving the SLA assurance objective through the above steps. The DCSA MnS producer may need to collaborate with other related management services, such as performance management, fault management, eMDAS, eCOSLA, and provisioning.

DCSA-REQ-PLC 1: 3GPP management system should provide service assurance capabilities for PLC control services.

DCSA-REQ-PLC 2: 3GPP management system should provide capabilities to enable consumers to monitor the service requirements fulfilment status and the corresponding analysis information for PLC control services.

5.3.2	Potential solutions

5.3.2.1	Potential solution #1: PLC control

5.3.2.1.1	Introduction

In the deterministic communication service requirement analysis phase, network requirements (data rate, delay, and reliability) are analyzed. Network deployment requirements of different areas need to be considered for network preparation. Deterministic communication services have high SLS requirements. For example, the planning of PLC control services focus on E2E latency, interactive latency, PLC control period, burst packet loss ratio etc. It demands on high delay stability and periodic deterministic of small data packets. The network requirements need to be derived based on the application characteristics of specific services. For example, network preparation information, such as network capacity, coverage, reliability, and data rate estimation, needs to be output based on the three-layer service requirement model of the PLC control service.

5.3.2.1.2	Description

1. Data collection:

Collect service experience data and network performance data of PLC control services based on the three-layer service requirement modeling. For example, the following data may collected: latency and throughput related performance data defined in TS 28.552 and TS 28.554. Abnormal information such as large round-trip delay, large interactive delay, large jitter, low data rate distribution, high packet loss rate, and service quality deterioration etc.

2. Analytics and demarcation

Service and network analysis of deterministic communication services includes monitoring, poor-QoE identification, demarcation, and root cause analysis. Identify abnormal issues, determine the impact scope, and restore services.

Monitoring and the poor-QoE identification requires two steps:

- Builds the three-layer service requirement model based on service characteristics, including poor experience information.

- Collects experience, service quality, and network-related indicators and abnormal incidents to identify poor-QoE users and services.

Fault demarcation is used to generate service interruption/ poor experience incident when a service anomaly occurs. The E2E network KPI, alarm information, terminal exceptions are correlated and analzed. Demarcation results for video monitoring services are provided accordingly.

The service characteristics and requirements regarding service experience, service quality and network performance quality for PLC control service are described in clause 5.Z.1. The monitoring of the service requirements fulfilment status and the corresponding MDA analysis information for PLC control service should be provided from management perspective. The abnormal issues should be identified and means should be taken to solve the issues. To support the assurance of PLC control services, some abnormal issues and some new MDA analysis outputs are as in the following table:

Table z: MDA analytics output of abnormal issues which may violate the service requirements

| Attribute Name | Data collection points |
| --- | --- |
| Communication service interruption | The reception failure of a data packet upon the deadline of the survival time. It is used to identify latency issues that may exceed survival time. |
| Excessive RTT | The times that the RTT is greater than a threshold exceed a predefined value within a detection period. It is used to identify end to end latency issues. |
| High packet loss rate | The packet loss rate is greater than a threshold within a detection period. It is used to identify potential packet level reliability issues. |

Some performance measurements are currently available in TS 28.552 [7], e.g. packet drop rate, average packet delay, IP latency measurements, UE throughput etc, which may be used as the enabling data for the above analysis.

3. Optimization and verification

Optimization:

For PLC control services, delay-related optimization are mainly considered. To optimize delay-related problems, it is needed to analyze the average delay, delay jitter, and delay reliability. It can analyzed the delay-related factors of the air interface, such as the uplink and downlink slot assignment proportion, air interface resource capacity, and scheduling policy.

Verification:

According to the service assurance objective of deterministic communication, that is, the quality of experience indicator of each service meets the deterministic requirement after optimization, it is further verified that the network capability meets the deterministic requirement. Select indicators that reflect PLC control service experience, such as PLC control period, survival time, interactive delay, interruption etc, or indicators that reflect network performance, such as packet loss rate, rate, and delay.

5.3.3	Conclusion - Impact on normative work

In TS 23.501 (System architecture for the 5G System), the QoS model of GFBR were defined. The PLC control service is a kind of URLLC service with GFBR kind of QoS control. The management of general aspects for the URLLC service with GFBR type QoS control should be considered for the normative work:

- New requirements for the deterministic aspects for the URLLC service with GFBR kind of QoS control: periodic of small packets (transfer interval), survival time, interactive delay, burst packet loss ratio etc, e.g. the impacts on serviceProfile, sliceProfile, and the corresponding performance measurements reporting.

- Impacts on MDA capability and the analytics output information for URLLC service with GFBR kind of QoS control: transfer interval, survival time, interactive delay, burst packet loss ratio etc, and abnormal issues which may impact the service performance.

End of change