3GPP TSG-SA5 Meeting #152 	S5-237795

Chicago, US, 13-17 November 2023

# Source:	Ericsson

# Title:	Addition of solution 4 Use NF type and HTTP redirect

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.840.

# 2	References

[1]		3GPP TR 28.840: "Study on CHF segmentation"

# 3	Rationale

New solution for key issue #4.1 using HTTP redirect and NF type.

# 4	Detailed proposal

First change

#### 5.4.4.1	Solution #4.1: Use NF type and CHF redirect

A possible solution for key issue #4.1, discovery by charging domains.

Reuse the NF type (TS 29.510 [2] clause 6.1.6.3.3) to cover the requirements on domain based discovery and charging. In most cases one NF type is connected to a domain the service specific charging information is also connected to the NF type rather than the domain. This means that in most cases the domain and required support of service specific charging information can be inferred by using the NF type.

If there is a need to have an NF type belonging to more than one domain, it could be solved by using a CHF redirect (HTTP status code 308). The first CHF would check if it can complete the request and if cannot complete the request it can select a new CHF based on internal configuration and NRF discovery. The first CHF would then redirect the request to the new CHF, and any further requests could be going to the new CHF.

End of changes