# 3GPP TSG SA WG5 Meeting #152                 	           S5-237865

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Add the TSN charging CHF record data

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the TSN charging CHF record data in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Describe the fields of TSN charging CHF CDR

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.1.3	CDR description on the Btsn interface

6.1.3.1	General

This clause describes the CDR content and format generated for time sensitive networking charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of 3GPP TS 32.240 [2]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in 3GPP TS 32.298 [7].

6.1.3.2	Time sensitive networking charging CHF CDR data

If enabled, CHF CDRs for time sensitive networking charging shall be produced for each AF.

The fields of TSN charging CHF CDR are specified in table 6.1.3.2-1.

Table 6.1.3.2-1: Time sensitive networking charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in 3GPP TS 32.298 [7]. |
| Recording Network Function ID | OM | Described in 3GPP TS 32.298 [7]. |
| Tenant Identifier | OC | This field holds the AF Identifier. |
| NF Consumer Information | M | This field holds the information of the TSN AF or TSCTSF. that used the charging service. |
| NF Functionality | M | Described in 3GPP TS 32.298 [7]. |
| NF Name | OC | Described in 3GPP TS 32.298 [7]. |
| NF Address | OC | Described in 3GPP TS 32.298 [7]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.298 [7]. |
| Record Opening Time | M | Described in 3GPP TS 32.298 [7]. |
| Duration | M | Described in 3GPP TS 32.298 [7]. |
| Record Sequence Number | C | Described in 3GPP TS 32.298 [7]. |
| Cause for Record Closing | M | Described in 3GPP TS 32.298 [7]. |
| Local Record Sequence Number | OM | Described in 3GPP TS 32.298 [7]. |
| Record Extensions | OC | Described in 3GPP TS 32.298 [7]. |
| TSN Charging Information | OM | This field holds the time sensitive networking charging information defined in clause 6.2.1.2. |

End of change