# 3GPP TSG SA WG5 Meeting #152                 	           S5-237867

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Define the information for 5GS bridge configuration and management

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the information for 5GS bridge configuration and management in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

The structure for 5GS Bridge Information and QoS and TSC Assistance Information are specified, applicable for both 5GS bridge configuration and management charging by TSN AF, and TSC and TS service enabler charging by TSCTSF.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.2.1.3	Definition of 5GS Bridge Information

Specific charging information for TSN charging is provided within the TSN Charging Information.

The detailed structure of the TSN Charging Information can be found in table 6.2.1.3-1.

Table 6.2.1.3-1: Structure of 5GS Bridge Information

| Information Element | Category | Description |
| --- | --- | --- |
| Bridge ID | M | This parameter holds the identifier of a TSN 5GS bridge. |
| NW-TT port numbers | OM | This parameter holds the port number of NW-TT. |
| DS-TT port number | OM | This parameter holds the port number of DS-TT. |

6.2.1.4	Definition of QoS and TSC Assistance Information

Common charging information used for 5GS bridge configuration and management charging, and QoS and TSC assistance service charging are provided within the QoS and TSC Assistance Container Information.

The detailed structure of the QoS and TSC Assistance Container Information can be found in table 6.2.1.3-1.

Table 6.2.1.3-1: Structure of QoS and TSC Assistance Information

| Information Element | Category | Description |
| --- | --- | --- |
| TSN QoS Information | OC | This parameter holds the information for TSN QoS, defined in clause 5.27 and clause 5.28 TS 23.501 [11].. |
| Priority | OM | This parameter indicates the priorities per port. |
| Bridge Delay | OM | This parameter indicates 5GS independentDelayMin and independentDelayMax values for each port pair and for each traffic class using: UE-DS-TT Residence Time, and Per traffic class minimum and maximum delays between the UE and the UPF/NW-TT. |
| Maximum TSC Burst Size | OM | This parameter indicates largest amount of data within a time period that is equal to the value of 5G-AN PDB of the 5QI. |
| Maximum Flow Bitrate | OM | This parameter indicates largest amount of bitrate of the TSC flow calculated by the TSN AF and used to set GBR. In this case, MBR is set equal to GBR. |
| TSC Assistance Container | OC | This parameter holds the information that describes TSC traffic characteristics for use in the 5G System, defined in clause 5.27 TS 23.501 [11]. |
| Flow Direction | OM | This parameter indicates the direction of the TSC flow, i.e. uplink or downlink. |
| Periodicity | OM | This parameter indicates the time period between start of two data bursts. |

End of change