3GPP TSG SA WG5 Meeting #152	S5-237874

Chicago, US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0488 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Add CHF to CHF interaction principle for LBO roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The architecture that support interactions between CHFs has been introduced in the 5G data connectivity charging. Hence, it is necessary to define the principles for the interaction between CHFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify how V-CHF select H-CHF in LBO roaming scenario when there are direct interaction between two CHFs. Specify the basic principles for interaction between two CHFs in LBO roaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CHF to CHF interaction principle is undefined for local breakout. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.9.2, 5.1.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The Figure 4.2.x refers to the new figure introduced in CR 0467 (SA5#151). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.9.2	CHF selection

In home routed scenario, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

-	The V-SMF for CHF selection in VPLMN, with the following differences:

-	CHF address(es) selection mechanisms based on PCF and UDM are not applicable.

-	When charging characteristics is used it will be based on local configuration .

-	When NRF is used, the V-CHF can be selected based on UE identified as in-bound roamer and the PLMN Id of the H-PLMN.

-	The H-SMF for CHF selection in HPLMN can be done as for non-roaming case.

In roaming home routed, PDU session upon V-SMF change:

-	Intra-PLMN V-SMF change: CHF address supplied by the old V-SMF shall be used.

-	Inter-PLMN V-SMF change: CHF selection mechanism as per V-SMF CHF selection in VPLMN at PDU session establishment.

In local breakout scenario with architecture in Figure 4.2.6, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

-	The V-SMF for CHF selection in VPLMN, is the same as in the home routed scenario.

-	The V-SMF for CHF selection in HPLMN, with the following differences.

-	CHF address(es) selection mechanisms based on PCF, UDM, and local configuration are not applicable.

-	 NRF based discovery, the H-CHF can be selected based on the H-PLMN of the UE.

In local breakout scenario with architecture in Figure 4.2.x, the V-CHF may select H-CHF based on the following and with this priority order (highest to lowest):

-	NRF based discovery.

-	V-CHF locally provisioned charging characteristics.

5.1.9.x		Interactions between CHFs in LBO roaming

The interaction between two CHFs in LBO roaming, e.g. V-CHF and H-CHF, H-CHF and A-CHF, is via the Nchf_ConvergedCharging service API.

The charging session between V-CHF and H-CHF is per PDU session, with one to one mapping with the charging session between V-SMF and V-CHF.

V-CHF generate the charging information to H-CHF based on the charging information received from V-SMF.

The capability for charging session between V-CHF and H-CHF and charging session between V-SMF and V-CHF will keep aligned. V-CHF send capability information to V-SMF based on the capability information from H-CHF, including:

-	Unit Count Inactivity Timer: using the smaller value in the two charging sessions.

-	Trigger Setting (enable/disable, trigger category): enable or immediate setting as the priority in the two charging sessions.

-	Trigger thresholds: using the smaller value in the two charging sessions.

-	Supported feature: using common supported features in the two charging sessions.

End of changes