3GPP TSG-SA5 Meeting #152	S5-237876

# Chicago,US, 13-17 November 2023													Revision of S5-237xxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0202 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.290 Quota mangement for CHF to CHF communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on conclusion of the study on 5G roaming charging architecture for wholesale and retail scenarios, based on solution #2.20, allowing the V-CHF to request more quota from the H-CHF than was requested from the NF, should be taken into normative work. The handling of the quota, that is in excess of what was requested by the NF, in the visited network CCS is considered outside the scope. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description about quota management for CHF to CHF communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LBO charging with the CHF to CHF communication is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.4.x(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.3.1	Basic principles

When offline charging and online charging are applicable to a service delivery, the charging information of both offline charging (without quota management) and online charging (with quota management) can be provided in a single command. The triggering for reporting the charging information can be any triggers of the offline charging or online charging (deferred or immediate triggers).

The invocation of the Charging Data Request for start of service, in case there is no valid quota for the rating group, can be done in either blocking mode or non-blocking mode:

-	blocking mode: the service delivery shall not start before its authorization from CHF;

-	non-blocking mode: the service delivery may start before its authorization from CHF.

For invoking the ConvergedCharging service with quota management, the ConvergedCharging service will operate in decentralized unit determination with the provided amounts of the Quota Requested information element otherwise if no amount is included in the Quota Requested information element, the ConvergedCharging service will operate in centralized unit determination and rating.

The quota management for converged charging is to furnish charging information to the CHF in order to perform credit control before the network resource usage is permitted, is specifed in the clause 5.1 TS 32.240 [1].

Next change

5.4.X	Quota management for multiple CHFs

For the communication between consumer CHF and producer CHF, converged charging with the quota management and without quota management are supported.

Upon receiving Charging Data Request [Initial/Update] from the NF Consumer (CTF) with quota management, consumer CHF may:

-	send the Charging Data Request [Initial/Update] to producer CHF with the amount of quota requested from NF Consumer (CTF), or

-	send the Charging Data Request [Initial/Update] to producer CHF with the amount of quota larger than the amout from NF Consumer (CTF), or

-	allocate the quota based on previous granted quota from the producer CHF.

When receiving Charging Data Response [Initial/Update] with granted quota from producer CHF, consumer CHF may send:

-	the quota to NF Consumer (CTF) as the amount granted by producer CHF, or;

-	the quota to NF Consumer (CTF) less than the amount granted by producer CHF.

End of change