3GPP TSG-SA5 Meeting #152	S5-237888

# Chicago, US, 13th – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0294 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 Align N38 in SMF with TS23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in TS29.502, N38 is the reference point between I-SMFs or V-SMFs. In TS23.501, Clause 5.25 covers scenarios with Support of Tracing in I-SMF/V-SMF where N38 is used. However, the N38 for SMF is missing in TS28.623. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding N38 for SMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned specifications for trace with TS23.501. Trace in SMF cannot be invoked at mobility of a PDU Session between I-SMFs or V-SMFs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.4.3, D.2.10. |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TS32.421 CR0108/0109/0110 TS32.422 CR0415/0416/0417 TS32.433 CR0153/0154/0155 TS28.623 CR0292/0293/0294 Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/908 at commit d36ecbe09753a38ba06d3a34056c3134a1b831ed Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/895 at commit fe719c0dd3a15e794eaa14f40f112c53235c3f29 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-237887 of S5-237560 |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

## C.4.3	OpenAPI document "TS28623_GenericNrm.yaml"

<CODE BEGINS>

openapi: 3.0.1

info:

title: Generic NRM

version: 18.4.0

description: >-

OAS 3.0.1 definition of the Generic NRM

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

RegistrationState:

type: string

enum:

- REGISTERED

- DEREGISTERED

VnfParameter:

type: object

properties:

vnfInstanceId:

type: string

vnfdId:

type: string

flavourId:

type: string

autoScalable:

type: boolean

PeeParameter:

type: object

properties:

siteIdentification:

type: string

siteDescription:

type: string

siteLatitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Latitude'

siteLongitude:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Longitude'

siteAltitude:

type: number

format: float

equipmentType:

type: string

environmentType:

type: string

powerInterface:

type: string

ThresholdInfo:

type: object

properties:

thresholdDirection:

type: string

enum:

- UP

- DOWN

- UP_AND_DOWN

thresholdValue:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

hysteresis:

oneOf:

- type: integer

minimum: 0

- type: number

format: float

minimum: 0

Operation:

type: object

properties:

name:

type: string

allowedNFTypes:

$ref: '#/components/schemas/NFType'

operationSemantics:

$ref: '#/components/schemas/OperationSemantics'

NFType:

type: string

description: ' NF name defined in TS 23.501 or TS 29.510'

enum:

- NRF

- UDM

- AMF

- SMF

- AUSF

- NEF

- PCF

- SMSF

- NSSF

- UDR

- LMF

- GMLC

- 5G_EIR

- SEPP

- UPF

- N3IWF

- AF

- UDSF

- DN

- BSF

- CHF

- NWDAF

- PCSCF

- CBCF

- HSS

- UCMF

- SOR_AF

- SPAF

- MME

- SCSAS

- SCEF

- SCP

- NSSAAF

- ICSCF

- SCSCF

- DRA

- IMS_AS

- AANF

- 5G_DDNMF

- NSACF

- MFAF

- EASDF

- DCCF

- MB_SMF

- TSCTSF

- ADRF

- GBA_BSF

- CEF

- MB_UPF

- NSWOF

- PKMF

- MNPF

- SMS_GMSC

- SMS_IWMSC

- MBSF

- MBSTF

- PANF

- TNGF

- W_AGF

- TWIF

- TSN_AF

OperationSemantics:

type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

SAP:

type: object

properties:

host:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/HostAddr'

port:

type: integer

NFServiceType:

type: string

enum:

- Namf_Communication

- Namf_EventExposure

- Namf_MT

- Namf_Location

- Nsmf_PDUSession

- Nsmf_EventExposure

- Others

TransportProtocol:

anyOf:

- type: string

enum:

- TCP

- type: string

SupportedPerfMetricGroup:

type: object

properties:

performanceMetrics:

type: array

items:

type: string

granularityPeriods:

type: array

items:

type: integer

minimum: 1

reportingMethods:

type: array

items:

type: string

enum:

- FILE_BASED_LOC_SET_BY_PRODUCER

- FILE_BASED_LOC_SET_BY_CONSUMER

- STREAM_BASED

reportingPeriods:

type: array

items:

type: integer

minimum: 1

ReportingCtrl:

oneOf:

- type: object

properties:

fileReportingPeriod:

type: integer

- type: object

properties:

fileReportingPeriod:

type: integer

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

- type: object

properties:

streamTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

Scope:

type: object

properties:

scopeType:

type: string

enum:

- BASE_ONLY

- BASE_ALL

- BASE_NTH_LEVEL

- BASE_SUBTREE

scopeLevel:

type: integer

ProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type without specialisations.

type: object

properties:

jobId:

type: string

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- PARTIALLY_FAILED

- CANCELLING

- CANCELLED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

resultStateInfo:

type: string

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

timer:

type: integer

FileDownloadJobProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type with specialisations

for usage in the "FileDownloadJob".

type: object

properties:

jobId:

type: string

status:

type: string

enum:

- NOT_STARTED

- RUNNING

- FINSHED

- FAILED

- CANCELLING

- CANCELLED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

resultStateInfo:

oneOf:

- type: string

enum:

- NULL

- UNKNOWN

- NO_STORAGE

- LOW_MEMROY

- NO_CONNECTION_TO_REMOTE_SERVER

- FILE_NOT_AVAILABLE

- DNS_CANNOT_BE_RESOLVED

- TIMER_EXPIRED

- OTHER

- type: string

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

timer:

type: integer

AreaScope:

oneOf:

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

- type: array

items:

$ref: '#/components/schemas/NrCellId'

- type: array

items:

$ref: '#/components/schemas/Tac'

- type: array

items:

$ref: '#/components/schemas/Tai'

Tai:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

tac:

$ref: '#/components/schemas/Tac'

AreaConfig:

type: object

properties:

freqInfo:

$ref: '#/components/schemas/FreqInfo'

pciList:

type: array

items:

type: integer

FreqInfo:

description: specifies the carrier frequency and bands used in a cell.

type: object

properties:

arfcn:

type: integer

freqBands:

type: array

items:

type: integer

MbsfnArea:

type: object

properties:

mbsfnAreaId:

type: integer

minimum: 1

earfcn:

type: integer

minimum: 1

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

IpAddr:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv4Addr'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Ipv6Addr'

ManagementData:

oneOf:

- type: array

items:

type: string

enum:

- COVERAGE

- CAPACITY

- ENERGY_EFFICIENCY

- MOBILITY

- ACCESSIBILITY

- type: array

items:

type: string

NodeFilter:

type: object

properties:

areaOfInterest:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AreaOfInterest'

networkDomain:

type: string

enum:

- CN

- RAN

cpUpType:

type: string

enum:

- CP

- UP

sst:

type: integer

SchedulingTime:

oneOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfWeek:

type: array

items:

type: string

enum:

- MONDAY

- TUESDAY

- WEDNESDAY

- THURSDAY

- FRIDAY

- SATURDAY

- SUNDAY

- type: object

properties:

timeIntervals:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeInterval'

daysOfMonth:

type: array

items:

type: integer

minimum: 0

maximum: 31

#-------- Definition of types used in Trace control NRM fragment------------------

jobType-Type:

type: string

description: Specifies whether the TraceJob represents only MDT, Logged MBSFN MDT, Trace or a combined Trace and MDT job. Applicable for Trace, MDT, RCEF and RLF reporting. See 3GPP TS 32.422 clause 5.9a for additional details.

enum:

- IMMEDIATE_MDT_ONLY

- LOGGED_MDT_ONLY

- TRACE_ONLY

- IMMEDIATE_MDT AND TRACE

- RLF_REPORT_ONLY

- RCEF_REPORT_ONLY

- LOGGED_MBSFN_MDT

listOfInterfaces-Type:

description: The interfaces to be recorded in the Network Element. See 3GPP TS 32.422 clause 5.5 for additional details.

type: object

properties:

MSCServerInterfaces:

type: array

items:

type: string

enum:

- A

- Iu-CS

- Mc

- MAP-G

- MAP-B

- MAP-E

- MAP-F

- MAP-D

- MAP-C

- CAP

MGWInterfaces:

type: array

items:

type: string

enum:

- Mc

- Nb-UP

- Iu-UP

RNCInterfaces:

type: array

items:

type: string

enum:

- Iu-CS

- Iu-PS

- Iur

- Iub

- Uu

SGSNInterfaces:

type: array

items:

type: string

enum:

- Gb

- Iu-PS

- Gn

- MAP-Gr

- MAP-Gd

- MAP-Gf

- Ge

- Gs

- S6d

- S4

- S3

- S13

GGSNInterfaces:

type: array

items:

type: string

enum:

- Gn

- Gi

- Gmb

S-CSCFInterfaces:

type: array

items:

type: string

enum:

- Mw

- Mg

- Mr

- Mi

P-CSCFInterfaces:

type: array

items:

type: string

enum:

- Gm

- Mw

I-CSCFInterfaces:

type: array

items:

type: string

enum:

- Cx

- Dx

- Mg

- Mw

MRFCInterfaces:

type: array

items:

type: string

enum:

- Mp

- Mr

MGCFInterfaces:

type: array

items:

type: string

enum:

- Mg

- Mj

- Mn

IBCFInterfaces:

type: array

items:

type: string

enum:

- Ix

- Mx

E-CSCFInterfaces:

type: array

items:

type: string

enum:

- Mw

- Ml

- Mm

- Mi/Mg

BGCFInterfaces:

type: array

items:

type: string

enum:

- Mi

- Mj

- Mk

ASInterfaces:

type: array

items:

type: string

enum:

- Dh

- Sh

- ISC

- Ut

HSSInterfaces:

type: array

items:

type: string

enum:

- MAP-C

- MAP-D

- Gc

- Gr

- Cx

- S6d

- S6a

- Sh

- N70

- N71

- NU1

EIRInterfaces:

type: array

items:

type: string

enum:

- MAP-F

- S13

- MAP-Gf

BM-SCInterfaces:

type: array

items:

type: string

enum:

- Gmb

MMEInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- S3

- S6a

- S10

- S11

- S13

SGWInterfaces:

type: array

items:

type: string

enum:

- S4

- S5

- S8

- S11

- Gxc

PDN_GWInterfaces:

type: array

items:

type: string

enum:

- S2a

- S2b

- S2c

- S5

- S6b

- Gx

- S8

- SGi

eNBInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- X2

en-gNBInterfaces:

type: array

items:

type: string

enum:

- S1-MME

- X2

- Uu

- F1-C

- E1

AMFInterfaces:

type: array

items:

type: string

enum:

- N1

- N2

- N8

- N11

- N12

- N14

- N15

- N20

- N22

- N26

AUSFInterfaces:

type: array

items:

type: string

enum:

- N12

- N13

NEFInterfaces:

type: array

items:

type: string

enum:

- N29

- N30

- N33

NRFInterfaces:

type: array

items:

type: string

enum:

- N27

NSSFInterfaces:

type: array

items:

type: string

enum:

- N22

- N31

PCFInterfaces:

type: array

items:

type: string

enum:

- N5

- N7

- N15

SMFInterfaces:

type: array

items:

type: string

enum:

- N4

- N7

- N10

- N11

- S5-C

- N38

SMSFInterfaces:

type: array

items:

type: string

enum:

- N20

- N21

UDMInterfaces:

type: array

items:

type: string

enum:

- N8

- N10

- N13

- N21

- NU1

UPFInterfaces:

type: array

items:

type: string

enum:

- N4

ng-eNBInterfaces:

type: array

items:

type: string

enum:

- NG-C

- Xn-C

- Uu

gNB-CU-CPInterfaces:

type: array

items:

type: string

enum:

- NG-C

- Xn-C

- Uu

- F1-C

- E1

- X2-C

gNB-CU-UPInterfaces:

type: array

items:

type: string

enum:

- E1

gNB-DUInterfaces:

type: array

items:

type: string

enum:

- F1-C

listOfNeTypes-Type:

description: The Network Element types where Trace Session activation is needed. See 3GPP TS 32.422 clause 5.4 for additional details.

type: array

items:

type: string

enum:

- MSC_SERVER

- SGSN

- MGW

- GGSN

- RNC

- BM_SC

- MME

- SGW

- PGW

- ENB

- EN_GNB

- GNB_CU_CP

- GNB_CU_UP

- GNB_DU

- AMF

- PCF

- SMF

- UPF

- AUSF

- SMSF

- HSS

- UDM

plmnTarget-Type:

type: object

description: The PLMN for which sessions shall be selected in the Trace Session in case of management based activation when several PLMNs are supported in the RAN (this means that shared cells and not shared cells are allowed for the specified PLMN. Note that the PLMN Target might differ from the PLMN specified in the Trace Reference, as that specifies the PLMN that is containing the management system requesting the Trace Session from the NE. See 3GPP TS 32.422 clause 5.9b for additional details.

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

required:

- mcc

- mnc

traceDepth-Type:

description: Specifies how detailed information should be recorded in the Network Element. The Trace Depth is a paremeter for Trace Session level, i.e., the Trace Depth is the same for all of the NEs to be traced in the same Trace Session. See 3GPP TS 32.422 clause 5.3 for additional details.

type: string

enum:

- MINIMUM

- MEDIUM

- MAXIMUM

- VENDORMINIMUM

- VENDORMEDIUM

- VENDORMAXIMUM

traceReference-Type:

type: object

description: The Trace Reference parameter shall be globally unique, therefore the Trace Reference shall compose as follows - MCC+MNC+Trace ID, where the MCC and MNC are coming with the Trace activation request from the management system to identify one PLMN containing the management system, and Trace ID is a 3 byte Octet String. See 3GPP TS 32.422 clause 5.6 for additional details.

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

traceId:

type: string

required:

- mcc

- mnc

- traceId

traceReportingFormat-Type:

type: string

description: Specifies whether file-based or streaming reporting shall be used for this Trace Session. See 3GPP TS 32.422 clause 5.11 for additional details.

enum:

- FILE-BASED

- STREAMING

traceTarget-Type:

type: object

description: Trace target conveying both the type and value of the target ID. For additional details see 3GPP TS 32.422

properties:

TargetIdType:

type: string

enum:

- IMSI

- IMEI

- IMEISV

- PUBLIC_ID

- UTRAN_CELL

- E-UTRAN_CELL

- NG-RAN_CELL

- eNB

- RNC

- gNB

- SUPI

TargetIdValue:

type: string

required:

- TargetIdType

- TargetIdValue

triggeringEvents-Type:

type: object

description: Specifies when to start a Trace Recording Session and which message shall be recorded first, when to stop a Trace Recording Session and which message shall be recorded last respectively. See 3GPP TS 32.422 clause 5.1 for additional detials.

properties:

MSC_SERVER:

type: array

items:

type: string

enum:

- MO_MT_CALLS

- MO_MT_SMS

- LU_IMSIattach_IMSIdetach

- HANDOVER

- SS

SGSN:

type: array

items:

type: string

enum:

- PDPcontext

- MO_MT_SMS

- RAU_GPRSattach_GPRSdetach

- MBMScontext

MGW:

type: array

items:

type: string

enum:

- CONTEXT

GGSN:

type: array

items:

type: string

enum:

- PDPcontext

- MBMScontext

IMS:

type: array

items:

type: string

enum:

- SIPsession_StandaloneTransaction

BM_SC:

type: array

items:

type: string

enum:

- MBMSactivation

MME:

type: array

items:

type: string

enum:

- UEinitiatedPDNconnectivityRequest

- ServiceRequest

- InitialAttach_TAU_Detach

- UEinitiatedPDNdisconnection

- BearerActivationModificationDeletion

- Handover

SGW:

type: array

items:

type: string

enum:

- PDNconnectionCreation

- PDNconnectionTermination

- BearerActivationModificationDeletion

PGW:

type: array

items:

type: string

enum:

- PDNconnectionCreation

- PDNconnectionTermination

- BearerActivationModificationDeletion

AMF:

type: array

items:

type: string

enum:

- Registration

- ServiceRequest

- Handover

- UEderegistration

- NetworkDeregistration

- UEMobilityFromEPC

- UEMobilityToEPC

SMF:

type: array

items:

type: string

enum:

- PDUsessionEstablishment

- PDUsessionModification

- PDUsessionRelease

- PDUsessionUPactivationDeactivation

- MobilityBtw3gppAndN3gppTo5GC

- MobilityFromEpc

PCF:

type: array

items:

type: string

enum:

- AMpolicy

- SMpolicy

- Authorization

- BDTpolicy

UPF:

type: array

items:

type: string

enum:

- N4Session

AUSF:

type: array

items:

type: string

enum:

- UEauthentication

NEF:

type: array

items:

type: string

enum:

- EventExposure

- PFDmanagement

- ParameterProvision

- Trigger

NRF:

type: array

items:

type: string

enum:

- NFmanagement

- NFdiscovery

NSSF:

type: array

items:

type: string

enum:

- NSSelection

- NSSAI

SMSF:

type: array

items:

type: string

enum:

- SMservice

UDM:

type: array

items:

type: string

enum:

- UEcontext

- SubscriberData

- UEauthentication

- EventExposure

anonymizationOfMdtData-Type:

description: Specifies level of MDT anonymization. For additional details see 3GPP TS 32.422 clause 5.10.12.

type: string

enum:

- NO_IDENTITY

- TAC_OF_IMEI

beamLevelMeasurement-Type:

description: Determines whether beam level measurements shall be included in case of immediate MDT M1 measurement in NR. For additional details see 3GPP TS 32.422 clause 5.10.40.

type: boolean

collectionPeriodRrmLte-Type:

description: See details in 3GPP TS 32.422 clause 5.10.20.

type: string

enum:

- 100ms

- 1000ms

- 1024ms

- 1280ms

- 2048ms

- 2560ms

- 5120ms

- 10000ms

- 10240ms

- 60000ms

collectionPeriodM6Lte-Type:

description: See details in 3GPP TS 32.422 clause 5.10.32.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

collectionPeriodM7Lte-Type:

description: See details in 3GPP TS 32.422 clause 5.10.33.

type: integer

minimum: 1

maximum: 60

collectionPeriodRrmUmts-Type:

description: See details in 3GPP TS 32.422 clause 5.10.21.

type: string

enum:

- 100ms

- 250ms

- 500ms

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

collectionPeriodRrmNr-Type:

description: See details in 3GPP TS 32.422 clause 5.10.30.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 60000ms

collectionPeriodM6Nr-Type:

description: See details in 3GPP TS 32.422 clause 5.10.34.

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 20480ms

- 40960ms

- 1min

- 6min

- 12min

- 30min

collectionPeriodM7Nr-Type:

description: See details in 3GPP TS 32.422 clause 5.10.35.

type: integer

minimum: 1

maximum: 60

eventListForEventTriggeredMeasurement-Type:

description: See details in 3GPP TS 32.422 clause 5.10.28.

type: string

enum:

- OUT_OF_COVERAGE

- A2_EVENT

eventThreshold-Type:

description: See details in 3GPP TS 32.422 clause 5.10.7, 5.10.7a, 5.10.13 and 5.10.14.

type: object

properties:

EventThresholdRSRP:

oneOf:

- type: integer

minimum: 0

maximum: 97

- type: integer

minimum: 0

maximum: 127

EventThresholdRSRQ:

oneOf:

- type: integer

minimum: 0

maximum: 34

- type: integer

minimum: 0

maximum: 127

EventThreshold1F:

type: object

properties:

CPICH_RSCP:

type: integer

minimum: -120

maximum: 25

CPICH_EcNo:

type: integer

minimum: -24

maximum: 0

PathLoss:

type: integer

minimum: 30

maximum: 165

EventThreshold1I:

type: integer

minimum: -120

maximum: 25

listOfMeasurements-Type:

description: See details in 3GPP TS 32.422 clause 5.10.3 for details.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M6_DL

- M6_UL

- M7_DL

- M7_UL

LTE:

type: array

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M1_EVENT_TRIGGERED

- M6

- M7

- M8

- M9

NR:

type: array

items:

type: string

enum:

- M1

- M2

- M3

- M4

- M5

- M6

- M7

- M1_EVENT_TRIGGERED

- M8

- M9

loggingDuration-Type:

description: See details in 3GPP TS 32.422 clause 5.10.9.

type: string

enum:

- 600s

- 1200s

- 2400s

- 3600s

- 5400s

- 7200s

loggingInterval-Type:

description: See details in 3GPP TS 32.422 clause 5.10.8.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

LTE:

type: array

items:

type: string

enum:

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

NR:

type: array

items:

type: string

enum:

- 0.32s

- 0.64s

- 1.28s

- 2.56s

- 5.12s

- 10.24s

- 20.48s

- 30.72s

- 40.96s

- 61.44s

- INFINITY

eventThresholdL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.X.

type: object

properties:

RSRP:

type: integer

minimum: 0

maximum: 127

RSRQ:

type: integer

minimum: 0

maximum: 127

hysteresisL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.Y.

type: integer

minimum: 0

maximum: 30

timeToTriggerL1-Type:

description: See details in 3GPP TS 32.422 clause 5.10.Z.

type: string

enum:

- 0ms

- 40ms

- 64ms

- 80ms

- 100ms

- 128ms

- 160ms

- 256ms

- 320ms

- 480ms

- 512ms

- 640ms

- 1024ms

- 1280ms

- 2560ms

- 5120ms

measurementPeriodLte-Type:

description: See details in 3GPP TS 32.422 clause 5.10.23.

type: string

enum:

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 1min

measurementPeriodUmts-Type:

description: See details in 3GPP TS 32.422 clause 5.10.22.

type: string

enum:

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

- 8000ms

- 12000ms

- 16000ms

- 20000ms

- 24000ms

- 28000ms

- 32000ms

- 64000ms

measurementQuantity-Type:

description: See details in 3GPP TS 32.422 clause 5.10.15.

type: string

enum:

- CPICH_EcNo

- CPICH_RSCP

- PathLoss

eventThresholdUphUmts-Type:

description: See details in 3GPP TS 32.422 clause 5.10.A.

type: integer

minimum: 0

maximum: 31

plmnList-Type:

description: See details in 3GPP TS 32.422 clause 5.10.24.

type: array

items:

type: object

properties:

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

mnc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mnc'

required:

- mcc

- mnc

maxItems: 16

positioningMethod-Type:

description: See details in 3GPP TS 32.422 clause 5.10.19.

type: string

enum:

- GNSS

- E-CELL_ID

reportAmount-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM1LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM4LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM5LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM6LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM7LTE-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM1NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM4NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM5NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM6NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportAmountM7NR-Type:

description: See details in 3GPP TS 32.422 clause 5.10.6.

type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- INFINITY

reportingTrigger-Type:

description: See details in 3GPP TS 32.422 clause 5.10.4.

type: array

items:

type: string

enum:

- PERIODICAL

- A2_FOR_LTE_NR

- 1F_FOR_UMTS

- 1I_FOR_UMTS_MCPS_TDD

- A2_TRIGGERED_PERIODIC_FOR_LTE_NR

- ALL_CONFIGURED_RRM_FOR_LTE_NR

- ALL_CONFIGURED_RRM_FOR_UMTS

reportInterval-Type:

description: See details in 3GPP TS 32.422 clause 5.10.5.

type: object

properties:

UMTS:

type: array

items:

type: string

enum:

- 250ms

- 500ms

- 1000ms

- 2000ms

- 3000ms

- 4000ms

- 6000ms

- 8000ms

- 12000ms

- 16000ms

- 20000ms

- 24000ms

- 28000ms

- 32000ms

- 64000ms

LTE:

type: array

items:

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 60000ms

- 360000ms

- 720000ms

- 1800000ms

- 3600000ms

NR:

type: array

items:

type: string

enum:

- 120ms

- 240ms

- 480ms

- 640ms

- 1024ms

- 2048ms

- 5120ms

- 10240ms

- 20480ms

- 40960ms

- 60000ms

- 360000ms

- 720000ms

- 1800000ms

reportType-Type:

description: Report type for logged NR MDT. See details in 3GPP TS 32.422 clause 5.10.27.

type: string

enum:

- PERIODICAL

- EVENT_TRIGGERED

sensorInformation-Type:

description: See details in 3GPP TS 32.422 clause 5.10.29.

type: array

items:

type: string

enum:

- BAROMETRIC_PRESSURE

- UE_SPEED

- UE_ORIENTATION

traceCollectionEntityId-Type:

description: See details in 3GPP TS 32.422 clause 5.10.11. Only TCE Id value may be sent over the air to the UE being configured for Logged MDT.

type: integer

excessPacketDelayThreshold-Type:

description: Excess Packet Delay Threshold for NR MDT. See details in 3GPP TS 32.422 clause 4.1.1 and 4.1.2.

type: object

properties:

fiveQIValue:

type: integer

excessPacketDelayThresholdValue:

type: string

enum:

- 0.25MS

- 0.5MS

- 1MS

- 2MS

- 4MS

- 5MS

- 10MS

- 20MS

- 30MS

- 40MS

- 50MS

- 60MS

- 70MS

- 80MS

- 90MS

- 100MS

- 150MS

- 300MS

- 500MS

excessPacketDelayThresholds-Type:

description: Array of type excessPacketDelayThreshold-Type.

type: array

items:

$ref: '#/components/schemas/excessPacketDelayThreshold-Type'

minItems: 0

maxItems: 255

traceConfig-Type:

description: Trace configuration parameters for NR. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

listOfInterfaces:

$ref: '#/components/schemas/listOfInterfaces-Type'

listOfNeTypes:

$ref: '#/components/schemas/listOfNeTypes-Type'

traceDepth:

$ref: '#/components/schemas/traceDepth-Type'

triggeringEvents:

$ref: '#/components/schemas/triggeringEvents-Type'

immediateMDTConfig-Type:

description: Immediate MDT configuration parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

listOfMeasurements:

$ref: '#/components/schemas/listOfMeasurements-Type'

reportingTrigger:

$ref: '#/components/schemas/reportingTrigger-Type'

reportAmount:

$ref: '#/components/schemas/reportAmount-Type'

reportAmountM1LTE:

$ref: '#/components/schemas/reportAmountM1LTE-Type'

reportAmountM4LTE:

$ref: '#/components/schemas/reportAmountM4LTE-Type'

reportAmountM5LTE:

$ref: '#/components/schemas/reportAmountM5LTE-Type'

reportAmountM6LTE:

$ref: '#/components/schemas/reportAmountM6LTE-Type'

reportAmountM7LTE:

$ref: '#/components/schemas/reportAmountM7LTE-Type'

reportAmountM1NR:

$ref: '#/components/schemas/reportAmountM1NR-Type'

reportAmountM4NR:

$ref: '#/components/schemas/reportAmountM4NR-Type'

reportAmountM5NR:

$ref: '#/components/schemas/reportAmountM5NR-Type'

reportAmountM6NR:

$ref: '#/components/schemas/reportAmountM6NR-Type'

reportAmountM7NR:

$ref: '#/components/schemas/reportAmountM7NR-Type'

reportInterval:

$ref: '#/components/schemas/reportInterval-Type'

eventThreshold:

$ref: '#/components/schemas/eventThreshold-Type'

collectionPeriodRrmLte:

$ref: '#/components/schemas/collectionPeriodRrmLte-Type'

collectionPeriodM6Lte:

$ref: '#/components/schemas/collectionPeriodM6Lte-Type'

collectionPeriodM7Lte:

$ref: '#/components/schemas/collectionPeriodM7Lte-Type'

collectionPeriodRrmUmts:

$ref: '#/components/schemas/collectionPeriodRrmUmts-Type'

collectionPeriodRrmNr:

$ref: '#/components/schemas/collectionPeriodRrmNr-Type'

collectionPeriodM6Nr:

$ref: '#/components/schemas/collectionPeriodM6Nr-Type'

collectionPeriodM7Nr:

$ref: '#/components/schemas/collectionPeriodM7Nr-Type'

eventThresholdUphUmts:

$ref: '#/components/schemas/eventThresholdUphUmts-Type'

measurementPeriodUmts:

$ref: '#/components/schemas/measurementPeriodUmts-Type'

measurementPeriodLte:

$ref: '#/components/schemas/measurementPeriodLte-Type'

measurementQuantity:

$ref: '#/components/schemas/measurementQuantity-Type'

beamLevelMeasurement:

$ref: '#/components/schemas/beamLevelMeasurement-Type'

positioningMethod:

$ref: '#/components/schemas/positioningMethod-Type'

excessPacketDelayThresholds:

$ref: '#/components/schemas/excessPacketDelayThresholds-Type'

loggedMDTConfig-Type:

description: Logged MDT configuration parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

traceCollectionEntityId:

$ref: '#/components/schemas/traceCollectionEntityId-Type'

loggingDuration:

$ref: '#/components/schemas/loggingDuration-Type'

loggingInterval:

$ref: '#/components/schemas/loggingInterval-Type'

eventThresholdL1:

$ref: '#/components/schemas/eventThresholdL1-Type'

hysteresisL1:

$ref: '#/components/schemas/hysteresisL1-Type'

timeToTriggerL1:

$ref: '#/components/schemas/timeToTriggerL1-Type'

mbsfnAreaList:

type: array

items:

$ref: '#/components/schemas/MbsfnArea'

reportType:

$ref: '#/components/schemas/reportType-Type'

plmnList:

$ref: '#/components/schemas/plmnList-Type'

eventListForEventTriggeredMeasurement:

$ref: '#/components/schemas/eventListForEventTriggeredMeasurement-Type'

areaConfigurationForNeighCell:

$ref: '#/components/schemas/AreaConfig'

mdtConfig-Type:

description: MDT config parameters. See details in 3GPP TS 28.622 clause 4.3.30.

type: object

properties:

anonymizationOfMdtData:

$ref: '#/components/schemas/anonymizationOfMdtData-Type'

areaScope:

type: array

items:

$ref: '#/components/schemas/AreaScope'

sensorInformation:

$ref: '#/components/schemas/sensorInformation-Type'

immediateMDTConfig:

$ref: '#/components/schemas/immediateMDTConfig-Type'

loggedMDTConfig:

$ref: '#/components/schemas/loggedMDTConfig-Type'

#-------- end of Definition of types used in Trace control NRM fragment ----------

#-------- Definition of abstract IOC Top -----------------------------------------

Top-Attr:

#  This definition will be deprecated, when all occurances of Top-Attr

#  are replaced by Top.

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

Top:

type: object

properties:

id:

type: string

nullable: true

objectClass:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

required:

- id

#-------- Definition of IOCs with new name-containments defined in other TS ------

SubNetwork-Attr:

type: object

properties:

dnPrefix:

type: string

userLabel:

type: string

userDefinedNetworkType:

type: string

setOfMcc:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

ManagedElement-Attr:

type: object

properties:

dnPrefix:

type: string

managedElementTypeList:

type: array

items:

type: string

userLabel:

type: string

locationName:

type: string

managedBy:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

userDefinedState:

type: string

swVersion:

type: string

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

SubNetwork-ncO:

type: object

properties:

ManagementNode:

$ref: '#/components/schemas/ManagementNode-Multiple'

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MeContext:

$ref: '#/components/schemas/MeContext-Multiple'

PerfMetricJob:

$ref: '#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: '#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: '#/components/schemas/TraceJob-Multiple'

ManagementDataCollection:

$ref: '#/components/schemas/ManagementDataCollection-Multiple'

NtfSubscriptionControl:

$ref: '#/components/schemas/NtfSubscriptionControl-Multiple'

AlarmList:

$ref: '#/components/schemas/AlarmList-Single'

FileDownloadJob:

$ref: '#/components/schemas/FileDownloadJob-Multiple'

Files:

$ref: '#/components/schemas/Files-Multiple'

MnsRegistry:

$ref: '#/components/schemas/MnsRegistry-Single'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

QMCJobs:

$ref: '#/components/schemas/QMCJob-Multiple'

ManagedElement-ncO:

type: object

properties:

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

PerfMetricJob:

$ref: '#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: '#/components/schemas/ThresholdMonitor-Multiple'

TraceJob:

$ref: '#/components/schemas/TraceJob-Multiple'

NtfSubscriptionControl:

$ref: '#/components/schemas/NtfSubscriptionControl-Multiple'

AlarmList:

$ref: '#/components/schemas/AlarmList-Single'

FileDownloadJob:

$ref: '#/components/schemas/FileDownloadJob-Multiple'

Files:

$ref: '#/components/schemas/Files-Multiple'

Scheduler:

$ref: '#/components/schemas/Scheduler-Multiple'

ConditionMonitor:

$ref: '#/components/schemas/ConditionMonitor-Multiple'

SupportedNotifications:

$ref: '#/components/schemas/SupportedNotifications-Single'

QMCJobs:

$ref: '#/components/schemas/QMCJob-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

ManagedFunction-Attr:

type: object

properties:

userLabel:

type: string

vnfParametersList:

type: array

items:

$ref: '#/components/schemas/VnfParameter'

peeParametersList:

type: array

items:

$ref: '#/components/schemas/PeeParameter'

priorityLabel:

type: integer

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

supportedTraceMetrics:

type: array

items:

type: string

EP_RP-Attr:

type: object

properties:

userLabel:

type: string

farEndEntity:

type: string

supportedPerfMetricGroups:

type: array

items:

$ref: '#/components/schemas/SupportedPerfMetricGroup'

TraceJob-Attr:

type: object

description: abstract class used as a container of all TraceJob attributes

properties:

jobType:

$ref: '#/components/schemas/jobType-Type'

plmnTarget:

$ref: '#/components/schemas/plmnTarget-Type'

traceReportingConsumerUri:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

traceCollectionEntityIpAddress:

$ref: '#/components/schemas/IpAddr'

traceReference:

$ref: '#/components/schemas/traceReference-Type'

jobId:

type: string

traceReportingFormat:

$ref: '#/components/schemas/traceReportingFormat-Type'

traceTarget:

$ref: '#/components/schemas/traceTarget-Type'

traceConfig:

$ref: '#/components/schemas/traceConfig-Type'

mdtConfig:

$ref: '#/components/schemas/mdtConfig-Type'

ManagedFunction-ncO:

type: object

properties:

PerfMetricJob:

$ref: '#/components/schemas/PerfMetricJob-Multiple'

ThresholdMonitor:

$ref: '#/components/schemas/ThresholdMonitor-Multiple'

ManagedNFService:

$ref: '#/components/schemas/ManagedNFService-Multiple'

TraceJob:

$ref: '#/components/schemas/TraceJob-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

VsDataContainer-Single:

type: object

properties:

id:

type: string

attributes:

type: object

properties:

vsDataType:

type: string

vsDataFormatVersion:

type: string

vsData:

nullable: true

VsDataContainer:

$ref: '#/components/schemas/VsDataContainer-Multiple'

ManagedNFService-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

nFServiceType:

$ref: '#/components/schemas/NFServiceType'

sAP:

$ref: '#/components/schemas/SAP'

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

usageState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/UsageState'

registrationState:

$ref: '#/components/schemas/RegistrationState'

ManagementNode-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

userLabel:

type: string

managedElements:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

vendorName:

type: string

userDefinedState:

type: string

locationName:

type: string

swVersion:

type: string

MnsAgent:

$ref: '#/components/schemas/MnsAgent-Multiple'

MnsAgent-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

systemDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

MeContext-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

dnPrefix:

type: string

PerfMetricJob-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

not:

required: [ conditionMonitorRef, schedulerRef ]

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

jobId:

type: string

performanceMetrics:

type: array

items:

type: string

granularityPeriod:

type: integer

minimum: 1

objectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

rootObjectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

conditionMonitorRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

schedulerRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

reportingCtrl:

$ref: '#/components/schemas/ReportingCtrl'

Files:

$ref: '#/components/schemas/Files-Multiple'

ThresholdMonitor-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

performanceMetrics:

type: array

items:

type: string

thresholdInfoList:

type: array

items:

$ref: '#/components/schemas/ThresholdInfo'

monitorGranularityPeriod:

type: integer

minimum: 1

objectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

rootObjectInstances:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

NtfSubscriptionControl-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

notificationRecipientAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

notificationTypes:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationType'

scope:

$ref: '#/components/schemas/Scope'

notificationFilter:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Filter'

HeartbeatControl:

$ref: '#/components/schemas/HeartbeatControl-Single'

HeartbeatControl-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

heartbeatNtfPeriod:

type: integer

minimum: 0

triggerHeartbeatNtf:

type: boolean

TraceJob-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: '#/components/schemas/TraceJob-Attr'

Files:

$ref: '#/components/schemas/Files-Multiple'

ManagementDataCollection-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

managementData:

$ref: '#/components/schemas/ManagementData'

targetNodeFilter:

$ref: '#/components/schemas/NodeFilter'

collectionTimeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

reportingCtrl:

$ref: '#/components/schemas/ReportingCtrl'

dataScope:

type: string

enum:

- SNSSAI

- 5QI

AlarmList-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

administrativeState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AdministrativeState'

operationalState:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/OperationalState'

numOfAlarmRecords:

type: integer

lastModification:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

alarmRecords:

description: >-

This resource represents a map of alarm records.

The alarmIds are used as keys in the map.

type: object

additionalProperties:

$ref: 'TS28532_FaultMnS.yaml#/components/schemas/AlarmRecord'

FileDownloadJob-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

fileLocation:

type: string

notificationRecipientAddress:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

cancelJob:

type: string

enum:

- TRUE

- FALSE

jobMonitor:

$ref: '#/components/schemas/FileDownloadJobProcessMonitor'

Files-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

numberOfFiles:

type: integer

jobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

jobId:

type: string

File:

$ref: '#/components/schemas/File-Multiple'

File-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

fileCompression:

type: string

fileSize:

type: integer

fileDataType:

type: string

enum:

- PERFORMANCE

- TRACE

- ANALYTICS

- PROPRIETARY

fileFormat:

type: string

fileReadyTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileExpirationTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileContent:

type: string

jobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

jobId:

type: string

MnsRegistry-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

MnsInfo:

$ref: '#/components/schemas/MnsInfo-Multiple'

MnsInfo-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

mnsLabel:

type: string

mnsType:

type: string

enum:

- ProvMnS

- FaultSupervisionMnS

- StreamingDataReportingMnS

- FileDataReportingMnS

mnsVersion:

type: string

mnsAddress:

description: Resource URI as defined in the relevant Technical Specification

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

mnsScope:

description: >-

List of the managed object instances that can be accessed using the MnS.

If a complete SubNetwork can be accessed using the MnS, this attribute may

contain the DN of the SubNetwork instead of the DNs of the individual managed

entities within the SubNetwork.

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

Scheduler-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

schedulingTimes:

type: array

items:

$ref: '#/components/schemas/SchedulingTime'

minItems: 1

schedulerStatus:

type: boolean

ConditionMonitor-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

condition:

type: string

conditionStatus:

type: boolean

SupportedNotifications-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

notificationTypes:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationType'

notificationProtocols:

type: array

items:

type: string

enum:

- HTTP

- HTTP_VES_ENCAPS

minItems: 1

QMCJob-Single:

allOf:

- $ref: '#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

serviceType:

type: string

enum:

- DASH

- MTSI

- VR

areaScope:

type: array

items:

$ref: '#/components/schemas/AreaScope'

qoECollectionEntityAddress:

$ref: '#/components/schemas/IpAddr'

pLMNTarget:

$ref: '#/components/schemas/plmnTarget-Type'

qoETarget:

type: string

qoEReference:

type: string

jobId:

type: string

sliceScope:

type: array

items:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

qMCConfigFile:

type: string

mDTAlignmentInformation:

$ref: '#/components/schemas/traceReference-Type'

availableRANqoEMetrics:

type: array

items:

type: string

enum:

- APPLAYERBUFFERLEVELLIST

- PLAYOUTDELAYFORMEDIA_STARTUP

minItems: 0

maxItems: 2

#-------- Definition of YAML arrays for name-contained IOCs ----------------------

VsDataContainer-Multiple:

type: array

items:

$ref: '#/components/schemas/VsDataContainer-Single'

ManagedNFService-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedNFService-Single'

ManagementNode-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementNode-Single'

MnsAgent-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsAgent-Single'

MeContext-Multiple:

type: array

items:

$ref: '#/components/schemas/MeContext-Single'

PerfMetricJob-Multiple:

type: array

items:

$ref: '#/components/schemas/PerfMetricJob-Single'

ThresholdMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/ThresholdMonitor-Single'

TraceJob-Multiple:

type: array

items:

$ref: '#/components/schemas/TraceJob-Single'

ManagementDataCollection-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagementDataCollection-Single'

NtfSubscriptionControl-Multiple:

type: array

items:

$ref: '#/components/schemas/NtfSubscriptionControl-Single'

FileDownloadJob-Multiple:

type: array

items:

$ref: '#/components/schemas/FileDownloadJob-Single'

Files-Multiple:

type: array

items:

$ref: '#/components/schemas/Files-Single'

File-Multiple:

type: array

items:

$ref: '#/components/schemas/File-Single'

MnsInfo-Multiple:

type: array

items:

$ref: '#/components/schemas/MnsInfo-Single'

Scheduler-Multiple:

type: array

items:

$ref: '#/components/schemas/Scheduler-Single'

ConditionMonitor-Multiple:

type: array

items:

$ref: '#/components/schemas/ConditionMonitor-Single'

QMCJob-Multiple:

type: array

items:

$ref: '#/components/schemas/QMCJob-Single'

#-------- Definitions in TS 28.623 for TS 28.532 ---------------------------------

resources-genericNrm:

oneOf:

- $ref: '#/components/schemas/VsDataContainer-Single'

- $ref: '#/components/schemas/ManagementNode-Single'

- $ref: '#/components/schemas/MnsAgent-Single'

- $ref: '#/components/schemas/MeContext-Single'

- $ref: '#/components/schemas/ManagedNFService-Single'

- $ref: '#/components/schemas/PerfMetricJob-Single'

- $ref: '#/components/schemas/ThresholdMonitor-Single'

- $ref: '#/components/schemas/TraceJob-Single'

- $ref: '#/components/schemas/ManagementDataCollection-Single'

- $ref: '#/components/schemas/NtfSubscriptionControl-Single'

- $ref: '#/components/schemas/HeartbeatControl-Single'

- $ref: '#/components/schemas/AlarmList-Single'

- $ref: '#/components/schemas/FileDownloadJob-Single'

- $ref: '#/components/schemas/Files-Single'

- $ref: '#/components/schemas/File-Single'

- $ref: '#/components/schemas/MnsRegistry-Single'

- $ref: '#/components/schemas/MnsInfo-Single'

- $ref: '#/components/schemas/Scheduler-Single'

- $ref: '#/components/schemas/ConditionMonitor-Single'

- $ref: '#/components/schemas/SupportedNotifications-Single'

- $ref: '#/components/schemas/QMCJob-Single'

<CODE ENDS>

*** END OF THIS CHANGE ***

*** START OF NEXT CHANGE ***

## D.2.10	module _3gpp-common-trace.yang

<CODE BEGINS>

module _3gpp-common-trace {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-trace";

prefix "trace3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types {prefix types3gpp; }

import _3gpp-common-yang-extensions {prefix yext3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-files { prefix files3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Trace handling

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

revision 2023-11-02 { reference CR-0294 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2023-08-10 { reference CR-0261; }

revision 2023-04-26 { reference CR-0250; }

revision 2023-02-18 { reference "CR-0234"; }

revision 2023-02-16 { reference "CR-0233"; }

revision 2022-09-30 { reference CR-0191 ; }

revision 2022-04-27 { reference "CR-0159"; }

revision 2021-10-18 { reference "CR-0139"; }

revision 2021-07-22 { reference "CR-0137"; }

revision 2021-01-25 { reference "CR-0122"; }

revision 2020-11-16 { reference "CR-0117"; }

revision 2020-08-06 { reference "CR-0102"; }

feature FilesUnderTraceJob {

description "Files shall be contained under TraceJob";

}

grouping FreqInfoGrp {

description "Represents the FreqInfo dataType.

This <<dataType>> defines the RF reference frequency and the frequency

operating bands used in a cell for a given direction (UL or DL) in FDD

or for both UL and DL directions in TDD";

leaf arfcn {

type uint32 {

range 0..3279165;

}

mandatory true;

description "RF Reference Frequency as defined in TS 38.104,

clause 5.4.2.1. The frequency provided identifies the absolute

frequency position of the reference resource block (Common RB 0)

of the carrier. Its lowest subcarrier is also known as Point A.";

}

leaf-list freqBands {

type uint32 {

range 1..1024;

}

min-elements 1;

description "List of NR frequency operating bands. Primary NR

Operating Band as defined in TS 38.104, clause 5.4.2.3.

The value 1 corresponds to n1, value 2 corresponds to NR operating

band n2, etc.";

}

}

grouping AreaConfigGrp {

description "Represents the AreaConfig dataType.

This <<dataType>> defines the area for which measurement logging should

be performed. It is described by a list of cells and a list of

frequencies.";

list freqInfo {

key arfcn;

min-elements 1;

max-elements 32;

description "It specifies the carrier frequency and bands used in

a cell.";

uses FreqInfoGrp ;

}

leaf-list pciList {

type uint32 {

range 0..1007;

}

min-elements 1;

max-elements 32;

description "List of neighbour cells subject for MDT scope.";

}

}

grouping AreaScopeGrp {

description "Represents the AreaScope dataType.

This <<dataType>> defines the area scope of MDT.

The Area Scope parameter in LTE and NR is either:

- list of Cells, identified by E-UTRAN-CGI or NG-RAN CGI.

Maximum 32 CGI can be defined.

- list of Tracking Area, identified by TAC.

Maximum of 8 TAC can be defined.

- list of Tracking Area Identity, identified by TAC with

associated plmn-Identity perTAC-List containing the

PLMN identity for each TAC. Maximum of 8 TAI can be defined.";

choice AreaScopeChoice {

leaf-list eutraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of E-UTRAN cells identified by E-UTRAN-CGI";

}

leaf-list utraCellIdList {

type string;

min-elements 1;

max-elements 32;

description "List of UTRAN cells identified by UTRAN CGI";

}

leaf-list tacList {

type types3gpp:Tac;

min-elements 1;

max-elements 8;

description "Tracking Area Code list";

}

list taiList {

description "Tracking Area Identity list";

key idx;

min-elements 1;

max-elements 8;

leaf idx { type string; }

uses types3gpp:TaiGrp;

}

}

}

grouping ExcessPacketDelayThresholdsGrp {

description "Represents the ExcessPacketDelayThresholds dataType.

This <<dataType>> defines a excess packet delay threshold information

to enable the calculation of the PDCP Excess Packet Delay in the

uplink in case of M6 uplink measurements are requested. The excess

packet delay threshold information is specified with the 5QI value

and excess packet delay threshold value.";

leaf fiveQIValue {

type uint8;

mandatory true;

description "It indicates 5QI value.";

}

leaf excessPacketDelayThresholdValue {

type decimal64 {

fraction-digits 2;

range 0.25|0.5|1|2|4|5|10|20|30|40|50|60|70|80|90|100|150|300|500 ;

}

mandatory true;

units milliseconds;

description "Value of excess packet delay threshold

for M6 UL measurement in milliseconds.";

}

}

grouping TraceReferenceGrp {

description "Represents the TraceReference dataType.

This <<dataType>> defines a globally unique identifier, which uniquely

identifies the Trace Session that is created by the TraceJob. It is

composed of the MCC, MNC (resulting in PLMN identifier) and the

trace identifier.";

uses types3gpp:PLMNId;  // mcc+mnc

leaf traceId {

type string;

mandatory true;

description "An identifier, which identifies the Trace

(together with MCC and MNC). This is a 3 byte Octet String.";

}

}

grouping MbsfnAreaGrp {

description "Represents the MbsfnArea dataType.

This <<dataType>> defines a MBSFN area. It is composed of the MBSFN Area

identifier and the carrier frequency (EARFCN).";

leaf mbsfnAreaId {

type uint32 {

range 1..max;

}

mandatory true;

description "MBSFN Area Identifier";

}

leaf earfcn{

type uint32 {

range 1..max;

}

mandatory true;

description "Carrier Frequency";

}

}

grouping TraceConfigGrp {

description "Defines the configuration parameters of TraceJob

which are specific for Trace or combined Trace and Immediate MDT.

The attribute listOfNeTypes specifies the network elements to be

traced. The optional attribute listOfInterfaces allows to specify

the individual interfaces of the network elements to be recorded.

The attribute traceDepth allows to configure the level of detail

of the information which shall be recorded.  For trace the reporting

is event based, where the triggering event is configured with

attribute triggeringEvent. For each triggering event the first and

last message (start/stop triggering event) to record  are specified.";

list listOfInterfaces {

key idx;

description "Specifies the interfaces that need to be traced in the given

ManagedEntityFunction.The attribute is applicable only for Trace. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.5 of 3GPP TS 32.422 for additional details on the

allowed values.";

leaf idx { type uint32 ; }

leaf-list MSCServerInterfaces {

type enumeration {

enum A ;

enum Iu-CS ;

enum Mc ;

enum MAP-G ;

enum MAP-B ;

enum MAP-E ;

enum MAP-F ;

enum MAP-D ;

enum MAP-C ;

enum CAP ;

}

}

leaf-list MGWInterfaces {

type enumeration {

enum Mc ;

enum Nb-UP ;

enum Iu-UP ;

}

}

leaf-list RNCInterfaces {

type enumeration {

enum Iu-CS ;

enum Iu-PS ;

enum Iur ;

enum Iub ;

enum Uu ;

}

}

leaf-list SGSNInterfaces {

type enumeration {

enum Gb ;

enum Iu-PS ;

enum Gn ;

enum MAP-Gr ;

enum MAP-Gd ;

enum MAP-Gf ;

enum Ge ;

enum Gs ;

enum S6d ;

enum S4 ;

enum S3 ;

enum S13 ;

}

}

leaf-list GGSNInterfaces {

type enumeration {

enum Gn ;

enum Gi ;

enum Gmb ;

}

}

leaf-list S-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Mg ;

enum Mr ;

enum Mi ;

}

}

leaf-list P-CSCFInterfaces {

type enumeration {

enum Gm ;

enum Mw ;

}

}

leaf-list I-CSCFInterfaces {

type enumeration {

enum Cx ;

enum Dx ;

enum Mg ;

enum Mw ;

}

}

leaf-list MRFCInterfaces {

type enumeration {

enum Mp ;

enum Mr ;

}

}

leaf-list MGCFInterfaces {

type enumeration {

enum Mg ;

enum Mj ;

enum Mn ;

}

}

leaf-list IBCFInterfaces {

type enumeration {

enum Ix ;

enum Mx ;

}

}

leaf-list E-CSCFInterfaces {

type enumeration {

enum Mw ;

enum Ml ;

enum Mm ;

enum Mi-Mg ;

}

}

leaf-list BGCFInterfaces {

type enumeration {

enum Mi ;

enum Mj ;

enum Mk ;

}

}

leaf-list ASInterfaces {

type enumeration {

enum Dh ;

enum Sh ;

enum ISC ;

enum Ut ;

}

}

leaf-list HSSInterfaces {

type enumeration {

enum MAP-C ;

enum MAP-D ;

enum Gc ;

enum Gr ;

enum Cx ;

enum S6d ;

enum S6a ;

enum Sh ;

}

}

leaf-list EIRInterfaces {

type enumeration {

enum MAP-F ;

enum S13 ;

enum MAP-Gf ;

}

}

leaf-list BM-SCInterfaces {

type enumeration {

enum Gmb ;

}

}

leaf-list MMEInterfaces {

type enumeration {

enum S1-MME ;

enum S3 ;

enum S6a ;

enum S10 ;

enum S11 ;

enum S13 ;

}

}

leaf-list SGWInterfaces {

type enumeration {

enum S4 ;

enum S5 ;

enum S8 ;

enum S11 ;

enum Gxc ;

}

}

leaf-list PDN_GWInterfaces {

type enumeration {

enum S2a ;

enum S2b ;

enum S2c ;

enum S5 ;

enum S6b ;

enum Gx ;

enum S8 ;

enum SGi ;

}

}

leaf-list eNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

}

}

leaf-list en-gNBInterfaces {

type enumeration {

enum S1-MME ;

enum X2 ;

enum Uu ;

enum F1-C ;

enum E1 ;

}

}

leaf-list AMFInterfaces {

type enumeration {

enum N1 ;

enum N2 ;

enum N8 ;

enum N11 ;

enum N12 ;

enum N14 ;

enum N15 ;

enum N20 ;

enum N22 ;

enum N26 ;

}

}

leaf-list AUSFInterfaces {

type enumeration {

enum N12 ;

enum N13 ;

}

}

leaf-list NEFInterfaces {

type enumeration {

enum N29 ;

enum N30 ;

enum N33 ;

}

}

leaf-list NRFInterfaces {

type enumeration {

enum N27 ;

}

}

leaf-list NSSFInterfaces {

type enumeration {

enum N22 ;

enum N31 ;

}

}

leaf-list PCFInterfaces {

type enumeration {

enum N5 ;

enum N7 ;

enum N15 ;

}

}

leaf-list SMFInterfaces {

type enumeration {

enum N4 ;

enum N7 ;

enum N10 ;

enum N11 ;

enum S5-C ;

enum N38 ;

}

}

leaf-list SMSFInterfaces {

type enumeration {

enum N20 ;

enum N21 ;

}

}

leaf-list UDMInterfaces {

type enumeration {

enum N8 ;

enum N10 ;

enum N13 ;

enum N21 ;

}

}

leaf-list UPFInterfaces {

type enumeration {

enum N4 ;

}

}

leaf-list ng-eNBInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

}

}

leaf-list gNB-CU-CPInterfaces {

type enumeration {

enum NG-C ;

enum Xn-C ;

enum Uu ;

enum F1-C ;

enum E1 ;

enum X2-C ;

}

}

leaf-list gNB-CU-UPInterfaces {

type enumeration {

enum E1 ;

}

}

leaf-list gNB-DUInterfaces {

type enumeration {

enum F1-C ;

}

}

}

leaf-list listOfNETypes {

type enumeration {

enum MSC_SERVER;

enum SGSN;

enum MGW;

enum GGSN;

enum RNC;

enum BM_SC;

enum MME;

enum SGW;

enum PGW;

enum ENB;

enum EN_GNB;

enum GNB_CU_CP;

enum GNB_CU_UP;

enum GNB_DU;

}

description "Specifies in which type of ManagedFunction the trace should

be activated. The attribute is applicable only for Trace with

Signalling Based Trace activation. In case this attribute is not used,

it carries a null semantic";

reference "Clause 5.4 of 3GPP TS 32.422 for additional details on the

allowed values";

}

leaf traceDepth {

when '../../jobType = "TRACE_ONLY"'

+  ' or ../../jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration  {

enum MINIMUM;

enum MEDIUM;

enum MAXIMUM;

enum VENDORMINIMUM;

enum VENDORMEDIUM;

enum VENDORMAXIMUM;

}

default MAXIMUM;

description "Specifies how detailed information should be recorded in the

Network Element. The Trace Depth is a paremeter for Trace Session level,

i.e., the Trace Depth is the same for all of the NEs to be traced in

the same Trace Session.

The attribute is applicable only for Trace, otherwise it carries a null

semantic.";

reference "Clause 5.3 of 3GPP TS 32.422";

}

}

grouping ImmediateMdtConfigGrp {

description "Represents the ImmediateMdtConfig dataType.

This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Immediate MDT or combine

Trace and Immediate MDT.

The optional attribute positioningMethod allows to specify

the positioning methods to use.

The following attributes are conditional available based on the

measurements configured in listOfMeasurements:

-reportInterval: conditional for M1 in LTE or NR and M1/M2 in UMTS,

-reportAmount: conditional for M1 in LTE or NR and M1/M2 in UMTS,

-reportingTrigger: conditional for M1 in LTE or NR and M1/M2 in UMTS,

-eventThreshold: conditional for A2 event reporting or A2 event

triggered periodic reporting,

-collectionPeriodRrmNR: conditional for M4 and M5 in NR,

-collectionPeriodM6NR: conditional for M6 in NR,

-collectionPeriodM7NR: conditional for M7 in NR,

-collectionPeriodRrmLte (conditional for M3 in LTE),

-measurementPeriodLTE (conditional for M4 and M5 in LTE),

-collectionPeriodM6Lte (conditional for M6 in LTE),

-collectionPeriodM7Lte (conditional for M7 in LTE),

-collectionPeriodRrmUmts (conditional for M4 and M5 in UMTS),

-measurementPeriodUmts (conditional for M6 and M7 in UMTS),

-measurementQuantity (conditional for 1F event reporting),

-beamLevelMeasurement (conditional for M1 in NR),

-excessPacketDelayThresholds (conditional for M6 UL measurement in NR).

For immediate MDT, the measurement reporting is dependent on the

configured measurements:

- For measurement M1 in LTE or NR, it is possible to select between

periodical, event triggered, event triggered periodic reporting or

reporting according to all configured RRM event triggers. For M1 and M2

measurement in UMTS, it is possible to select between periodical, event

triggered reporting or reporting according to all configured RRM event

triggers. Parameter reportingTrigger determines which of the reporting

methods is selected and in case of event triggered or event-triggered

periodic, which is the decisive event type. For periodical reporting,

parameters reportInterval and reportAmount determine the interval between

two successive reports and the number of reports. This means the

periodical reporting terminates after reportAmount reports have been

sent as long as reportAmount is configured with a value different from

infinity. For event-triggered periodic reporting, these two parameters

apply in addition to parameter eventThreshold which determines the

threshold of the event. In this case up to reportAmount reports are

sent with a periodicity of reportInterval after the entering condition

is fulfilled. The reporting is stopped, if the leaving condition is

fulfulled and is restarted if the configured event reoccurs. For event

based reporting, there is only one report sent after the event occurs.

The parameters to configure are reportingTrigger and eventThreshold.

In case of UMTS and 1f event reporting, additionally parameter

measurementQuantity is necessary in order to determine for which

measurement(s) the event threshold is applicable.  Parameter

beamLevelMeasurement determines whether beam level measurements shall

be included in case of NR.

- For measurement M2 in LTE or NR, reporting is according to RRM

configuration, see TS 38.321, TS 36.321 and TS 38.331, TS 36.331.

For measurement M4 in UMTS, reporting is either according to RRM

configuration, see TS 25.321 and TS 25.331 or periodic or event

triggered periodic using parameter collectionPeriodRrmUmts and

eventThresholdUphUmts.

- For measurement M3 in UMTS, the reporting is done upon

availability, see TS 37.320.

- For measurements M4, M5, M6 and M7 in NR, for measurements

M3, M4, M5, M6 and M7 in LTE and for measurements M5, M6 and M7

in UMTS periodical reporting is applied. The configurable parameter

is the interval between two measurements (collectionPeriodRrmNr,

collectionPeriodM6NR, collectionPeriodM7Nr, collectionPeriodRrmLte,

measurementPeriodLte, collectionPeriodM6Lte, collectionPeriodM7Lte,

collectionPeriodRrmUmts, measurementPeriodUmts).  If no collection

period is configured for M5 in UMTS, all available measurements are

logged according to RRM configuration.";

leaf listOfMeasurements {

when 'jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6_DL;

enum M6_UL;

enum M7_DL;

enum M7_UL;

enum M1_EVENT_TRIGGERED;

enum M6;

enum M7;

enum M8;

enum M9;

}

description "It specifies the UE measurements that shall be collected in

an Immediate MDT job. The attribute is applicable only for Immediate MDT.

In case this attribute is not used, it carries a null semantic.";

reference "3GPP TS 32.422 clause 5.10.3";

}

leaf reportingTrigger {

when 'jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum A2_FOR_LTE;

enum 1F_FOR_UMTS;

enum 1I_FOR_UMTS_MCPS_TDD;

enum A2_TRIGGERED_PERIODIC_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_LTE;

enum ALL_CONFIGURED_RRM_FOR_UMTS;

}

description "It specifies whether periodic or event based measurements

should be collected.

The attribute is applicable only for Immediate MDT and when the

listOfMeasurements is configured for M1 (for both UMTS and LTE)

or M2 (only for UMTS). In case this attribute is not used, it carries

a null semantic.";

reference "Clause 5.10.4 of 3GPP TS 32.422";

}

leaf reportInterval {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type uint32 {

range "120|240|250|480|500|640|1000|1024|2000|2048|3000|4000|"

+"5120|6000|8000|10240|12000|16000|20000|"

+"20480|24000|28000|32000|40960|60000|64000|"

+"360000|720000|1800000|3600000";

}

units milliseconds;

description "It specifies the interval between the periodical measurements

that shall be taken when the UE is in connected mode.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements. In case

this attribute is not used, it carries a null semantic.";

reference "5.10.5 of 3GPP TS 32.422";

}

leaf reportAmount {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' and ../reportingTrigger = "PERIODICAL"';

type union {

type uint32 {

range "1|4|8|16|32|64" ;

}

type enumeration {

enum INFINITY;

}

}

description "It specifies the number of measurement reports that shall be

taken for periodic reporting while the UE is in connected.

The attribute is applicable only for Immediate MDT and when

reportingTrigger is configured for periodical measurements. In

case this attribute is not used, it carries a null semantic.";

reference "Clause 5.10.6 of 3GPP TS 32.422";

}

leaf eventThreshold {

when 'jobType = "IMMEDIATE_MDT_ONLY"';

type int64;

description "Specifies the threshold which should trigger the reporting

in case A2 event reporting in LTE or 1F/1l event in UMTS. The attribute

is applicable only for Immediate MDT and when reportingTrigger is

configured for A2 event in LTE or 1F event or 1l event in UMTS. In

case this attribute is not used, it carries a null semantic.";

reference "Clauses 5.10.7 and 5.10.7a of 3GPP TS 32.422";

}

leaf collectionPeriodRRMNR {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM

configured measurement samples for M4, M5 in NR. The attribute is

applicable only for Immediate MDT. In case this attribute is not

used, it carries a null semantic.";

reference "Clause 5.10.30 of 3GPP TS 32.422";

}

leaf collectionPeriodRRMLTE {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "250|500|1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M2, M3 in LTE. The attribute is applicable only

for Immediate MDT. In case this attribute is not used, it carries a

null semantic.";

reference "Clause 5.10.20 of 3GPP TS 32.422";

}

leaf collectionPeriodM6NR {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum 120ms;

enum 240ms;

enum 480ms;

enum 640ms;

enum 1024ms;

enum 2048ms;

enum 5120ms;

enum 10240ms;

enum 20480ms;

enum 40960ms;

enum 1min;

enum 6min;

enum 12min;

enum 30min;

}

description "It specifies the collection period for the Packet Delay

measurement (M6) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "clause 5.10.34 of  TS 32.422";

}

leaf collectionPeriodM7NR {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1..60";

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for NR MDT taken by the gNB. The attribute is

applicable only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "clause 5.10.35 of  TS 32.422";

}

leaf collectionPeriodM6LTE {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|2048|5120|10240";

}

units milliseconds;

description "Specifies the collection period for the Packet Delay

measurement (M6) for MDT taken by the eNB. The attribute is applicable

only for Immediate MDT. In case this attribute is not used,

it carries a null semantic.";

reference "Clause 5.10.32 of  TS 32.422 ";

}

leaf collectionPeriodM7LTE {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 1..60 ;

}

description "It specifies the collection period for the Packet Loss Rate

measurement (M7) for LTE MDT taken by the eNB. The attribute is

applicable only for Immediate MDT. In case this attribute

is not used, it carries a null semantic.";

reference "Clause 5.10.33 of  TS 32.422 .";

}

leaf eventThresholdUphUMTS {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint16 {

range 0..31 ;

}

description "It specifies the threshold which should trigger

the reporting in case of event-triggered periodic reporting for M4

(UE power headroom measurement) in UMTS. In case this attribute is

not used, it carries a null semantic.";

reference "5.10.39 of TS 32.422";

}

leaf collectionPeriodRRMUMTS {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|1280|2048|2560|5120|"

+"10240|60000";

}

units milliseconds;

description "Specifies the collection period for collecting RRM configured

measurement samples for M3, M4, M5 in UMTS. The attribute is applicable

only for Immediate MDT. In case this attribute is not used, it carries

a null semantic";

reference "Clause 5.10.21 of 3GPP TS 32.422";

}

leaf measurementPeriodLTE {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "1024|1280|2048|2560|5120|"

+"10240|60000";

}

units milliseconds;

mandatory true;

description "It specifies the measurement period for the Data Volume and

Scheduled IP throughput measurements for MDT taken by the eNB.

The attribute is applicable only for Immediate MDT. In case this

attribute is not used, it carries a null semantic.";

reference "Clause 5.10.23 of 3GPP TS 32.422";

}

leaf measurementPeriodUMTS {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  'or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type uint32 {

range "250|500|1000|2000|3000|4000|6000|8000|12000|16000|20000|"

+"24000|28000|32000|64000";

}

units milliseconds;

mandatory true;

description "It specifies the measurement period for the Data Volume and

Throughput measurements for MDT taken by RNC.

The attribute is applicable only for Immediate MDT. In case this

attribute is not used, it carries a null semantic.";

reference "Clause 5.10.22 of 3GPP TS 32.422";

}

leaf measurementQuantity {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum CPICH_ECNO;

enum CPICH_RSCP;

enum PATHLOSS;

}

description "It specifies the measurements that are collected in an MDT

job for a UMTS MDT configured for event triggered reporting.";

reference "Clause 5.10.15 of 3GPP TS 32.422";

}

leaf beamLevelMeasurement {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type boolean;

default false;

description "Indicates whether the NR M1 beam level measurements shall

be included or not.";

reference "Clause 5.10.40 of TS 32.422";

}

leaf positioningMethod {

when 'jobType = "IMMEDIATE_MDT_ONLY"'

+  ' or jobType = "IMMEDIATE_MDT_AND_TRACE"';

type enumeration {

enum GNSS;

enum E_CELL_ID;

}

description "It specifies what positioning method should be used in the

MDT job.";

reference "Clause 5.10.19 of 3GPP TS 32.422";

}

list excessPacketDelayThresholds {

description "Excess packet delay thresholds info for M6 UL measurement.";

min-elements 1;

key idx;

leaf idx { type string; }

uses ExcessPacketDelayThresholdsGrp;

}

}

grouping LoggedMdtGrp {

description "This <<dataType>> defines the configuration parameters of

IOC TraceJob which are specific for Logged MDT or Logged MBSFN MDT.

The optional attribute plmnList allows to specify the PLMNs where

measurement collection, status indication and log reporting is allowed,

the optional attribute areaConfigurationForNeighCell allows to specify

the area for which UE is requested to perform measurements logging for

neighbour cells which have list of frequencies.  For logged MDT in UMTS

and LTE, the reporting is periodical. Parameter loggingInterval determines

the interval between the reports and parameter loggingDuration determines

how long the configuration is valid meaning after this duration has passed

no further reports are sent. In NR, the reporting can be periodical or event

based, determined by parameter reportType. For periodical reporting the

same parameters as in LTE and UMTS apply. For event based reporting,

parameter eventListForEventTriggeredMeasurement configures the event type,

namely 'out of coverage' or 'L1 event'. In case 'L1 event' is selected as

event type, the logging is performed according to parameter loggingInterval

at regular intervals only when the conditions indicated by eventThresholdL1,

hysteresisL1, timeToTriggerL1 (defining the thresholds, hysteresis and time

to trigger) are met and if UE is 'camped normally' state (TS 38.331,

TS 38.304).  In case 'out of coverage' is selected as event type, the

logging is performed according to parameter loggingInterval at regular

intervals only when the UE is in 'any cell selection' state.

Furthermore, logging is performed immediately upon transition from the

'any cell selection' state to the 'camped normally' state (TS 38.331,

TS 38.304).";

leaf traceCollectionEntityIPAddress {

when '../traceReportingFormat  = "FILE_BASED" or '

+'jobType = "LOGGED_MDT_ONLY" or jobType = "LOGGED_MBSFN_MDT"';

type union {

type inet:uri;

type inet:ip-address;

}

description "Specifies the address of the Trace Collection Entity when

the attribute traceReportingFormat is configured for the file-based

reporting. The attribute is applicable for both Trace and MDT.";

reference "Clause 5.9 of 3GPP TS 32.422";

}

leaf loggingDuration {

when 'jobType = "LOGGED_MDT_ONLY" or'

+ ' jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "600|1200|2400|3600|5400|7200";

}

units seconds;

description "Specifies how long the MDT configuration is valid at the

UE in case of Logged MDT. The attribute is applicable only for

Logged MDT and Logged MBSFN MDT. In case this attribute is not used, it

carries a null semantic.";

reference "5.10.9 of 3GPP TS 32.422";

}

leaf loggingInterval {

when 'jobType = "LOGGED_MDT_ONLY" or'

+ ' jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0|320|640|1280|2560|5120|10240|20480|"

+"30720|40960|61440";

}

units milliseconds;

description "Specifies the periodicty for Logged MDT. The attribute is

applicable only for Logged MDT and Logged MBSFN MDT. In case this

attribute is not used, it carries a null semantic.

The value 0 indicates Infinity for NR.";

reference "5.10.8 of 3GPP TS 32.422";

}

leaf reportType {

when 'jobType = "IMMEDIATE_MDT_ONLY"';

type enumeration {

enum PERIODICAL;

enum EVENT_TRIGGERED;

}

description "It specifies report type for logged NR MDT";

reference "Clause 5.10.27 of 3GPP TS 32.422";

}

leaf eventListForEventTriggeredMeasurement {

when 'jobType = "LOGGED_MDT_ONLY"';

type enumeration {

enum OUT_OF_COVERAGE ;

enum A2_EVENT ;

}

description "Specifies event types for event triggered measurement in the

case of logged NR MDT.  Each trace session may configure at most one

event. The UE shall perform logging of measurements only upon certain

condition being fulfilled:

- Out of coverage.

- A2 event.";

reference "Clause 5.10.28 of 3GPP TS 32.422";

}

leaf eventThresholdL1 {

when 'jobType = "LOGGED_MDT_ONLY" or'

+ ' jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..127";

}

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT and when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "clause 5.10.36 of TS 32.422";

}

leaf hysteresisL1 {

when 'jobType = "LOGGED_MDT_ONLY" or '

+ 'jobType = "LOGGED_MBSFN_MDT"';

type uint32 {

range "0..30";

}

description "It specifies the hysteresis used within the entry and leave

condition of the L1 event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "clause 5.10.37 of TS 32.422";

}

leaf timeToTriggerL1 {

when 'jobType = "LOGGED_MDT_ONLY" or '

+ 'jobType = "LOGGED_MBSFN_MDT"';

type int32 {

range 0|40|64|80|100|128|160|256|320|480|512|640|1024|1280|2560|5120;

}

units milliseconds;

description "It specifies the threshold which should trigger

the reporting in case of event based reporting of logged NR MDT.

The attribute is applicable only for Logged MDT, when reportType

is configured for event triggered reporting and when

eventListForEventTriggeredMeasurement is configured for L1 event.

In case this attribute is not used, it carries a null semantic.";

reference "clauses 5.10.38 of TS 32.422";

}

list pLMNList {

when 'jobType = "LOGGED_MDT_ONLY"';

key "mcc mnc";

uses types3gpp:PLMNId;

max-elements 16;

description "It indicates the PLMNs where measurement collection, status

indication and log reporting is allowed.";

reference "Clause 5.10.24 of 3GPP TS 32.422";

}

list areaConfigurationForNeighCell {

when 'jobType = "LOGGED_MDT_ONLY"';

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area for which UE is requested to perform

measurement logging for neighbour cells which have list of frequencies.

If it is not configured, the UE shall perform measurement logging for

all the neighbour cells.

Applicable only to NR Logged MDT.";

reference "3GPP TS 32.422 clause 5.10.26.";

uses AreaConfigGrp;

}

list mBSFNAreaList {

when 'jobType = "LOGGED_MBSFN_MDT"';

key "mbsfnAreaId earfcn";

max-elements 8;

description "The MBSFN Area consists of a MBSFN Area ID and Carrier

Frequency (EARFCN). The target MBSFN area List can have up to 8 entries.

This parameter is applicable only if the job type is Logged MBSFN MDT.";

reference "5.10.25 of 3GPP TS 32.422";

uses MbsfnAreaGrp;

}

}

grouping MdtConfigGrp {

description "Defines the configuration parameters of IOC

TraceJob which are specific for MDT. The attribute

anonymizationOfMdtData specifies the level of anonymization

of MDT data.  The optional attribute areaScope allows to

specify the area in terms of cells or Tracking Area/Routing

Area/Location area where the MDT data collection shall take

place. In case of RLF_REPORT_ONLY and RCEF_REPORT_ONLY the

optional attribute areaScope allows to specify the eNB or list

of eNBs or gNB or list of gNBs where the reports should be

collected.  The optional attribute sensorInformation allows to

specify  the sensor information to include.  Based on the value

configured for attribute jobType in IOC TraceJob, the attributes

immediateMdtConfig or loggedMdtConfig are available: In case of

IMMEDIATE_MDT_ONLY or IMMEDIATE_MDT_AND_TRACE the attribute

immediateMdtConfig is applicable. In case of LOGGED_MDT_ONLY or

LOGGED_MBSFN_MDT the attribute loggedMdtConfig is applicable.";

leaf anonymizationOfMDTData {

when ../areaScope ;

type enumeration {

enum NO_IDENTITY;

enum TAC_OF_IMEI;

}

default NO_IDENTITY;

description "Specifies level of MDT anonymization.";

reference "3GPP TS 32.422 clause 5.10.12.";

}

list areaScope {

key "idx";

leaf idx { type uint32 ; }

description "It specifies the area where data shall be collected.

List of eNB/list of gNB/eNB/gNB for RLF or RCEF.

List of cells/TA/LA/RA for signaling based MDT or management

based Logged MDT.

List of cells for management based Immediate MDT.

Cell, TA, LA, RA are mutually exclusive.

This attribute shall be present if MDT is supported.";

reference "Clause 5.10.2 of 3GPP TS 32.422";

uses AreaScopeGrp;

}

leaf-list sensorInformation {

type enumeration {

enum BAROMETRIC_PRESSURE;

enum UE_SPEED;

enum UE_ORIENTATION;

}

description "It specifies which sensor information shall be included in

logged NR MDT and immediate NR MDT measurement if they are available.

The following sensor measurement can be included or excluded for

the UE.";

reference "Clause 5.10.29 of 3GPP TS 32.422";

}

}

grouping TraceJobGrp {

leaf jobType {

type enumeration {

enum IMMEDIATE_MDT_ONLY;

enum LOGGED_MDT_ONLY;

enum TRACE_ONLY;

enum IMMEDIATE_MDT_AND_TRACE;

enum RLF_REPORT_ONLY;

enum RCEF_REPORT_ONLY;

enum LOGGED_MBSFN_MDT;

}

default TRACE_ONLY;

description "Specifies the MDT mode and it specifies also whether the

TraceJob represents only MDT, Logged MBSFN MDT, Trace or a combined

Trace and MDT job. The attribute is applicable for Trace, MDT, RCEF and

RLF reporting.";

reference "Clause 5.9a of 3GPP TS 32.422 for additional details on the

allowed values.";

}

list pLMNTarget {

key "mcc mnc";

description "Specifies which PLMN that the subscriber of the session to

be recorded uses as selected PLMN. PLMN Target might differ from the

PLMN specified in the Trace Reference";

reference "Clause 5.9b of 3GPP TS 32.422";

uses types3gpp:PLMNId;

}

leaf traceReportingConsumerUri {

when '../traceReportingFormat  = "STREAMING"';

type inet:uri;

description "URI of the Streaming Trace data reporting MnS consumer

(a.k.a. streaming target).

This attribute shall be present if file based trace data reporting is

supported and traceReportingFormat set to 'file based' or when

jobType is set to Logged MDT or Logged MBSFN MDT.";

reference "Clause 5.9 of 3GPP TS 32.422";

}

leaf traceCollectionEntityId  {

when '../jobType = "LOGGED_MDT_ONLY" or '

+ '../jobType = "LOGGED_MBSFN_MDT"';

type uint32;

description "It specifies the TCE Id which is sent to the UE in

Logged MDT.";

reference "Clause 5.10.11 of 3GPP TS 32.422";

}

list traceReference {

key "idx";

min-elements 1;

max-elements 1;

description "A globally unique identifier, which uniquely identifies the

Trace Session that is created by the TraceJob.

In case of shared network, it is the MCC and MNC of the Participating

Operator that request the trace session that shall be provided.

The attribute is applicable for both Trace and MDT.";

reference "Clause 5.6 of 3GPP TS 32.422";

leaf idx { type uint32 ; }

uses trace3gpp:TraceReferenceGrp ;

}

leaf jobId {

type string;

yext3gpp:inVariant;

description "Identifier of a TraceJob";

yext3gpp:inVariant;

}

leaf traceReportingFormat {

type enumeration {

enum FILE_BASED;

enum STREAMING;

}

default FILE_BASED;

description "Specifies the trace reporting format - streaming trace

reporting or file-based trace reporting";

reference "3GPP TS 32.422 clause 5.11";

}

list traceTarget {

key "targetIdType targetIdValue";

max-elements 1;

leaf targetIdType {

type enumeration {

enum IMSI;

enum IMEI;

enum IMEISV;

enum PUBLIC_ID;

enum UTRAN_CELL;

enum E_UTRAN_CELL;

enum NG_RAN_CELL;

enum ENB;

enum RNC;

enum GNB;

enum SUPI;

}

}

leaf targetIdValue {

type string;

}

description "Specifies the target object of the Trace and MDT. The

attribute is applicable for both Trace and MDT. This attribute

includes the ID type of the target as an enumeration and the ID value.

The traceTarget shall be public ID in case of a Management Based

Activation is done to an ScscfFunction. The traceTarget shall be

cell only in case of the UTRAN cell traffic trace function.

The traceTarget shall be E-UtranCell only in case of E-UTRAN cell

traffic trace function.The traceTarget shall be either IMSI or

IMEI(SV) if the Trace Session is activated to any of the following

ManagedEntity(ies):

- HssFunction

- MscServerFunction

- SgsnFunction

- GgsnFunction

- BmscFunction

- RncFunction

- MmeFunction

The traceTarget shall be IMSI if the Trace Session is activated to a

ManagedEntity playing a role of ServinGWFunction.

In case of signaling based Trace/MDT, the traceTarget attribute shall

be able to carry (IMSI or IMEI(SV)or SUPI), the mDTAreaScope attribute

shall be able to carry a list of (cell or E-UtranCell or NRCellDU or

TA/LA/RA).

In case of management based Immediate MDT, the traceTarget attribute

shall be null value, the mDTAreaScope attribute shall carry a list of

(Utrancell or E-UtranCell or NRCellDU).

In case of management based Logged MDT, the traceTarget attribute

shall carry an eBs or a RNC or gNBs. The Logged MDT should be initiated

on the specified eNB or RNC or gNB in traceTarget. The mDTAreaScope

attribute shall carry a list of (Utrancell or E-UtranCell or NRCellDU or

TA/LA/RA).

In case of RLF reporting, or RCEF reporting,  the traceTarget

attribute shall be null value, the mDTAreaScope attribute shall carry

one or list of eNBs/gNBs";

reference "3GPP TS 32.422";

}

list traceConfig {

when '../jobType = "TRACE_ONLY"'

+  ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"';

key idx;

description "Trace config";

max-elements 1;

uses TraceConfigGrp;

leaf idx { type string; }

}

list mdtConfig {

when '../jobType = "IMMEDIATE_MDT_ONLY"'

+ ' or ../jobType = "IMMEDIATE_MDT_AND_TRACE"'

+ ' or ../jobType = "RLF_REPORT_ONLY"'

+ ' or ../jobType = "RCEF_REPORT_ONLY"'

+ ' or ../jobType = "LOGGED_MBSFN_MDT"';

key idx;

description "MDT config";

max-elements 1;

uses MdtConfigGrp;

leaf idx { type string; }

}

}

grouping TraceSubtree {

description "Contains classes that manage Tracing.

Should be used in all  classes (or classes inheriting from)

- SubNnetwork

- ManagedElement

- ManagedFunction

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list TraceJob {

description "A TraceJob instance represents the Trace Control and

Configuration parameters of a particular Trace Job (see TS 32.421 and

TS 32.422 for details). It can be name-contained by SubNetwork,

ManagedElement, ManagedFunction.

To activate Trace Jobs, a MnS consumer has to create TraceJob object

instances on the MnS producer. A MnS consumer can activate a Trace Job

for another MnS consumer since it is not required the value of

traceCollectionEntityIPAddress or traceReportingConsumerUri to be

his own.

For the details of Trace Job activation see clauses 4.1.1.1.2 and

4.1.2.1.2 of TS 32.422.

When a MnS consumer wishes to deactivate a Trace Job, the MnS consumer

shall delete the corresponding TraceJob instance.

For details of management Trace Job activation/deactivation see clause

4.1.1.1.2 of TS 32.422.

The attribute traceReference specifies a globally unique ID and

identifies a Trace session. One Trace Session may be activated to

multiple Network Elements. The traceReference is populated by the

consumer that makes the request for a Trace Session.

The jobId attribute presents the job identifier of a TraceJob instance.

The jobId can be used to associate  multiple TraceJob instances.

For example, it is possible to configure the same jobId value for

multiple TraceJob instances required to produce the data (e.g. RSRP

values of M1 and RLF reports) for a specific network analysis.

The attribute traceReportingFormat defines the method for reporting

the produced measurements. The selectable options are file-based or

stream-based reporting. In case of file-based reporting the attribute

traceCollectionEntityIPAddress is used to specify the IP address to

which the trace records shall be transferred, while in case of

stream-based reporting the attribute traceReportingConsumerUri

specifies the streaming target.

The mandatory attribute traceTarget determines the target object of

the TraceJob. Dependent on the network element to which the Trace

Session is activated different types of the target object are possible.

The attribute pLMNTarget defines the PLMN for which sessions shall be

selected in the Trace Session in case of management based activation

when several PLMNs are supported in the RAN.

The attribute jobType specifies the kind of data to collect.

In case of TRACE_ONLY, the configuration parameters of attribute

traceConfig shall be applied. In case of IMMEDIATE_MDT_ONLY,

LOGGED_MDT_ONLY, RLF_REPORT_ONLY, RCEF_REPORT_ONLY and

LOGGED_MBSFN_MDT the configuration parameters of attribute

mdtConfig or a subset of these shall be applied.

In case of IMMEDIATE_MDT_AND_TRACE both attributes, traceConfig and

mdtConfig are applicable.

Creation and deletion of TraceJob instances by MnS consumers is

optional; when not supported, the TraceJob instances may be created

and deleted by the system or be pre-installed.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses TraceJobGrp ;

}

uses files3gpp:FilesSubtree {

if-feature FilesUnderTraceJob;

}

}

}

}

<CODE ENDS>

*** END OF THIS CHANGE ***