# 3GPP TSG-SA5 Meeting #152	S5-237916

# Chicago, US, 13-17, November 2023

# Source:	Nokia

# Title:	Rel-18 pCR TS 28.318 Stage 3 definition for exposing performance and prediction information for high  availability

# Document for:	Approval

Agenda Item:	6.6.4.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[A]    3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[B]    3GPP TS 28.532: “Management and orchestration; Generic management services”.

[C]    3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

# 3	Rationale

This PCR provides the Stage 3 definition for exposing performance and prediction information for high availability.

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/903 at commit 6b5d6624f1267a3a254ac2d9ce2997793009de92

# 4	Detailed proposal

It is proposed to agree to add the following text to clause 8 in TS 28.318.

First Change

## 8.1 Exposing performance and prediction information for high availability

### 8.1.1 RESTful HTTP-based solution set

-	YAML based Solution Set (Annex X).

End First Change

Begin Second Change

# X	Solution Set (SS) definitions: TS28318_NsoeuNrm.yaml

openapi: 3.0.1

info:

title: Nsoeu NRM

version: 0.2.0-alpha.1

description: >-

OAS 3.0.1 definition of the Nsoeu  NRM

Â© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.318; Network and Service Operations for Energy Utilities (NSOEU)

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.318/

paths: {}

components:

schemas:

#-------Definition of generic IOCs ----------#

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

PerfMetricsExposure:

$ref: '#/components/schemas/PerfMetricsExposure-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

PerfMetricsExposure:

$ref: '#/components/schemas/PerfMetricsExposure-Multiple'

PerfMetricsExposure-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

serviceIdentifier:

type: string

perfMetricsAreaScope:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoAreaToCellMapping'

perfMetrics:

type: array

items:

type: string

reportingCtrl:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ReportingCtrl'

reportingPeriod:

type: integer

minimum: 1

perfMetricsThresholds:

type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ThresholdInfo'

timeWindow:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

#-------Definition of generic IOCs ----------#

#-------Definition of the generic dataType ----------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

PerfMetricsExposure-Multiple:

type: array

items:

$ref: '#/components/schemas/PerfMetricsExposure-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.318 for TS 28.532 --------------------------#

resources-NsoeuNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/PerfMetricsExposure-Single'

#----- Definitions in TS 28.318 for TS 28.532 --------------------------#

End Second Change