3GPP TSG-SA5 Meeting #152 	S5-237949

Chicago, United States, 13th Nov 2023 - 17th Nov 2023

# Source:	ZTE Corporation

# Title:	DP on analysis of dependency of NFs' enhancement for measurements per UE

# Document for:		Discussion and Endorsement

Agenda Item:	6.5.4.4

# 1	Decision/action requested

The group is asked to discuss and endorse.

# 2	References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[3]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[4]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

# 3	Discussion

## 3.1	Background

3GPP SA2 asked SA5 to provide UE level measurements to support analytics conducted by NWDAF. The measurements are related to packet delay between PSA UPF and UE, and packet delay in NG-RAN, for a specific UE.

SA5 confirms that SA5 would like to provide support for the per UE measurements for the RAN part UL/DL packet delay and UL/DL packet delay between PSA_UPF and UE.

SA5 is presently discussing the options (i.e., PM or MDT) to support these per UE measurements, and would be happy to inform SA2 when these per UE measurements will be supported by the SA5 specs, targeted for the Rel-18 timeframe.

## 3.2	Observations

Thanks to offline discussion, we realized that per UE measurements here are different from traditional PMs/KPIs. In order to provide these PMs/KPIs, it requires a whole process of coordination among NFs and MnFs.

The idea and solution are quite wonderful only before no dependency of enhancement of NFs and MnFs is confirmed, or after the dependency is decoupled.

Observation #1:     If temporary ID is used to identify a UPF session or gNB session, these IDs must be supported by NWDAF, e.g. how to identify a gNB session? NWDAF has not NGAP ID, any other ID can be used?

Observation #2:    Since temporary ID is preferred, for handover/connect to idle/idle to connect/resume etc. scenarios, need to consider start/stop/change the measurement task on the target PSA/gNB.

Observation #3:    Since per UE level, for multiple PDU session, this is more complex scenario. Whether it is OK needs to be made clear.

## 3.3	Recommendations

Before SA5 provides support for the per UE measurements for the RAN part UL/DL packet delay and UL/DL packet delay between PSA_UPF and UE,  solutions need to be made clear and whether NFs and MnFs need to be enhanced. The dependency or coordination needs to be recognized.

# 4	Detailed proposal

The group is asked to endorse the recommendations in clause 3.3.