# 3GPP TSG-SA5 Meeting #152	S5-237950

# Chicago, USA,  13-17, November 2023

# Source:	Samsung

# Title:	Rel-18 pCR TS 28.318 NRM Definitions for coordinated rapid recovery

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[A]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Rationale

This pCR proposes the NRM definitions required to support the procedure defined in clause 6.1 of TS 28.318.

# 4	Detailed proposal

It is proposed to agree to the following text to clause 7 in TS 28.318.

Begin Change

## 7.Y	Model

### 7.Y.1       Class diagrams

#### 7.Y.1.1      Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document

Figure 7.Y.2.1-1: DSO Rapid Recovery NRM fragment

#### 7.Y.1.2	Inheritance

This clause depicts the inheritance relationships.

<<InformationObjectClass>>

Top

<<InformationObjectClass>>

DsoRapidRecovery

<<InformationObjectClass>>

UPSInfo

Figure 7.Y.2.2-1: DSO Rapid Recovery NRM fragment

### 7.Y.2       Class definitions

#### 7.Y.2.1	DsoRapidRecovery

##### 7.Y.2.1.1 Definition

This IOC represents the mutual information exchange between DSO and MNO to achieve Rapid Recovery of DSO’s energy service outage for all DSO sites which require service restoration.

##### 7.Y.2.1.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| dSORapidRecoveryInfo | M | T | F | F | T |

##### 7.Y.2.1.3	Attribute constraints

None.

#### 7.Y.2.1.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.2 DSORapidRecoveryInfo  <<dataType>>

##### 7.Y.2.2.1 Definition

This <<dataType>>  defines the DSO Rapid Recovery related information for a particular location/region.

##### 7.Y.2.2.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoServiceOutageInfo | M | T | T | F | T |
| dsoRapidInterventionInfo | M | T | T | F | T |
| mnoRapidInterventionInfo | M | T | T | F | T |
| dsoServiceRestoreInfo | M | T | T | F | T |

##### 7.Y.2.2.3	Attribute constraints

None.

##### 7.Y.2.2.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.3 ServiceOutageInfo  <<dataType>>

##### 7.Y.2.3.1 Definition

This <<dataType>> defines several information shared by DSO to MNO related to its energy service outage including the starting time of DSO energy service outage, outage substation locations etc.

The attribute dsoOutStartTime defines the time stamp at which the DSO energy service outage occurs. Along with outage locations and expected recovery time this could be beneficial for MNO to plan and prepare (e.g when and where to activate and de-activate) its UPS power backup at different site locations for its crucial communication services and also for assisting DSO for MRI (MNO Rapid Intervention). In case of MRI assistance MNO may reserve its UPS resources to be used when required.

The attribute dsoOutRegion defines the regions where DSO energy service outage occurs. These are usually the locations where DSO subsystems are situated. This is usually defined as latitude-longitude pair and meterId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation.

The attribute dsoRestoreTimeExpected defines the time by which DSO expects its energy distribution service to be restored through its manual dispatch and processes. This is usually a much longer time duration that may not be acceptable for some crucial public services like hospitals.

##### 7.Y.2.3.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoOutStartTime | M | T | T | F | T |
| dsoOutRegion | M | T | T | F | T |
| dsoRestoreTimeExpected | M | T | T | F | T |

##### 7.Y.2.3.3	Attribute constraints

None.

##### 7.Y.2.3.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.4 DsoInterventionInfo  <<dataType>>

##### 7.Y.2.4.1 Definition

This <<dataType>> defines several information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI).

The attribute dsoTransmissionRestoreTime defines the time at which DSO restores its Energy “transmission” service. For DSO’s distribution service to restore first its transmission line/service needs to be restored. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes but for this MNO’s communication services via MRI is required to DSO. Hence this is also the time from when the DSO expect MRI to start.The attribute dsoRequiredInterventionDuration defines time “duration” for which the DSO requires MRI in order to restore its energy distribution services rapidly. This time duration is usually defined in minutes.

The attribute dsoPriorityRecoveryLocation defines the locations of DSO substations that need recovery on priority by using smart energy services because some of the substations locations might be providing electrical energy for crucial services like hospitals, mission critical service avenues, government offices etc. This is used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MRI because the gNBs are servicing these substations. This is usually defined as latitude-longitude pair.  The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region containing substation. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation.

##### 7.Y.2.4.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoTransmissionRestoreTime | M | T | T | F | T |
| dsoRequiredInterventionDuration | M | T | T | F | T |
| dsoPriorityRecoveryLocation | M | T | T | F | T |

##### 7.Y.2.4.3	Attribute constraints

None.

##### 7.Y.2.4.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.5 MnoInterventionInfo  <<dataType>>

##### 7.Y.2.5.1 Definition

This <<dataType>> defines several information shared by MNO to DSO for feasible fulfillment of DSO’s rapid recovery requirements by MNO. It may be possible that instead of full, some part of  requested Rapid Recovery requirements that MNO is actually able to cater in terms of intervention start time, location etc.

The attribute mnoInterventionTime defines the time at which a MNO will actually be able to provide dedicated communications services to DSO (as MRI) depending on its site’s UPS backup status , engagement in other critical service avenues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services.

The attribute mnoInterventionDuration defines the time duration for which a MNO will actually be able to provide dedicated communications services to DSO (as MRI) depending on its UPS backup status, engagement in other critical service avenues etc. This time duration is usually defined in minutes.

The attribute mnoInterventionLocation defines the locations having DSO’s substations where its actually feasible for MNO to provide its dedicated communications services (as MRI) for DSO’s smart energy services usage. The criteria for location feasibility may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other locations like hospital, government offices etc. This is usually defined as latitude-longitude pair and meterId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region containing substation. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation.

##### 7.Y.2.5.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnoInterventionTime | M | T | T | F | T |
| mnoInterventionDuration | M | T | T | F | T |
| mnoInterventionLocation | M | T | T | F | T |

##### 7.Y.2.5.3	Attribute constraints

None.

##### 7.Y.2.5.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.6 ServiceRestoreInfo  <<dataType>>

##### 7.Y.2.6.1 Definition

This <<dataType>> defines information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this MNO gets to know at what time it can stop its Rapid Intervention for the DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. DSO will update this information once the energy distribution service has been restored.

The attribute dsoRestoreTimeActual defines the time at which DSO finally restores its energy distribution services rapidly  by using MRI. This time is much smaller than dsoRestoreTimeExpected.

The attribute dsoRestoreLocationActual defines the Ssbstation locations where DSO is able to finally restore its energy distribution services. These may be same or fewer than dsoPriorityRecoveryLocation or mnoInterventionLocation. This is usually defined as latitude-longitude pair and meterId . The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation.

##### 7.Y.2.6.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoRestoreTimeActual | M | T | T | F | T |
| dsoRestoreLocationActual | M | T | T | F | T |

##### 7.Y.2.6.3	Attribute constraints

None.

##### 7.Y.2.6.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.7 ServiceLocationIdentifier  <<datatype>>

##### 7.Y.2.7.1 Definition

This <<datatype>> defines information shared by DSO to MNO regarding location of DSO distribution facility in terms of latitude, longitude and meterId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is known between both the parties using out-of-band mechanism hence not subject to standardisation.

##### 7.Y.2.7.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| latitude | M | T | T | F | T |
| longitude | M | T | T | F | T |
| meterId | O | T | T | F | T |

##### 7.Y.2.7.3	Attribute constraints

None.

##### 7.Y.2.7.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 7.Y.2.8	UPSInfo

##### 7.Y.2.8.1 Definition

This IOC represents the UPS related information of MNO site or a base station. It is name-contained by DsoRapidRecovery IOC. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery

##### 7.Y.2.8.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| isUpsAvailable | M | T | T | F | T |
| totalUpsBackupDuration | M | T | T | F | T |
| remainingUpsBackupDuration | M | T | T | F | T |
| meterId | M | T | T | F | T |
| baseStationId | M | T | T | F | T |

##### 7.Y.2.8.3	Attribute constraints

None.

##### 7.Y.2.8.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

### 7.Y.3	Attribute definitions

#### 7.Y.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document in addition to those existing in clause 4.4.1 of [X].

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| dSORapidRecoveryInfo | This defines the DSO and MNO coordinated recovery related information for a particular location/region. | type: DSORapidRecoveryInfo multiplicity: 1.* isOrdered: N/A isUnique: Yes defaultValue: None isNullable: False |
| dsoServiceOutageInfo | Contains list of attributes that define several information shared by DSO to MNO related to its energy service outage like starting time of DSO energy service outage, outage locations, expected restore time of outage. | type: ServiceOutageInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRapidInterventionInfo | Contains list of attributes that define several information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI) like expected time to start MRI, expected duration of MRI, expected locations that require MRI on priority. | type: DsoInterventionInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoRapidInterventionInfo | Contains list of attributes that define several information shared by MNO to DSO for feasible fulfillment of DSO’s rapid recovery requirements by MNO. It may be possible that instead of full, some part of requested MRI requirements MNO is actually able to cater in terms of intervention start time, location, intervention duration. | type: MnoInterventionInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoServiceRestoreInfo | Contains list of attributes that define information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this information, MNO gets to know at what time it can stop its MRI services for DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. | type: ServiceRestoreInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoOutStartTime | Time stamp at which the DSO energy service outage occurs. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoOutRegion | Regions where DSO energy service outage occurs. These are the locations where DSO subsystems are situated. This is defined as latitude -longitude pair and meterId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreTimeExpected | Time by which DSO expects its energy distribution service to get restored through its manual dispatch and processes. This is usually a longer time period that may not be acceptable for some crucial public services like hospitals. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoTransmissionRestoreTime | Time at which DSO restores its Energy “transmission” service. This is the time from when the DSO expect MRI to start. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRequiredInterventionDuration | Time duration for which the DSO requires MRI in order to restore its energy distribution services rapidly. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoPriorityRecoveryLocation | Locations of DSO substations that need recovery on priority by using smart energy services. This is defined as latitude-longitude pair and meterId. This can be used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MRI. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation. | Type:ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionTime | Time at which a MNO will actually be able to provide dedicated communications services to DSO (as MRI) depending on its UPS backup status , other critical service avenues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionDuration | Time duration for which a MNO will actually be able to provide dedicated communications services to DSO (as MRI) depending on its UPS backup status , other critical service avenues etc. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mnoInterventionLocation | Locations having DSO’s substations where its actually feasible for MNO to provide its dedicated communications services (as MRI) for DSO’s smart energy services usage. The criteria for location feasibility may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other locations like hospital, government offices etc. This is defined as latitude-longitude pair and meterId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreTimeActual | Time at which DSO finally restores its energy distribution services rapidly by using MRI. This time is much smaller than dsoRestoreTimeExpected | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreLocationActual | Substation locations where DSO is able to finally restore its energy distribution services. These may be same or fewer than dsoPriorityRecoveryLocation or mnoInterventionLocation. This is defined as latitude-longitude pair and meterId. The point defined by the Lat/Log pair serves as one point of an area polygon defining the region. meterId identifies the MNO facility using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | This defines the latitude | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | This defines the longitude | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| meterId | meterId identifies the MNO facility (e.g. base station) using DSO energy services. meterId is communicated between both the parties using out-of-band mechanism hence not subject to standardisation. A meter with a unique meterId is installed by DSO in the MNO site. | type: String multiplicity: 1.* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| isUpsAvailable | It tells if the MNO site has UPS installed/available or not. There might be some sites where UPS is not installed yet or is under progress etc. Its value can be True or False. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| totalUpsBackupDuration | Total time duration for which MNO site can operate on its UPS backup i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from onset of DSO outage. It is defined in minutes. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remainingUpsBackupDuration | Remaining Time duration for which MNO site can operate on its UPS backup at a given time i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from a given time. It is defined in minutes. allowedValues: Integer with a minimum value of 1 [NOTE1] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| baseStationId | Base station identity as defined by MNO for its base stations in a given site | type: String multiplicity: 1 isOrdered: N/A isUnique:True defaultValue: None isNullable: False |
| NOTE 1: The value of remainingUpsBackupDuration can be taken from IETF RFC 1628 [11], upsEstimatedMinutesRemaining. |  |  |

### 7.Y.4	Common notifications

Refer to clause 4.5.2 of [X] for Configuration notifications.

End of Change