| 3GPP TS 32.27x V0.2.0 (2023-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; 5G Multicast-broadcast Services charging; Stage 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Architecture considerations	9

4.1	5G System architecture for Multicast and Broadcast Service	9

4.2	5G MBS Session converged charging architecture	9

5	5G MBS Session charging principles and scenarios	10

5.1	5G MBS Session charging principles	10

5.1.1	General	10

5.1.2	Requirements	10

5.1.3	Charging information	10

5.1.4	Charging Identifier	11

5.1.5	CHF selection	11

5.2	5G MBS Session converged charging scenarios	11

5.2.1	Basic principles	11

5.2.1.1	General	11

5.2.1.2	Applicable Triggers in the MB-SMF	11

5.2.1.3	MBS session charging	14

5.2.2	Message flows	14

5.2.2.1	General	14

5.2.2.2	 5G MBS charging for multicast and broadcast communication	14

5.2.2.2.1	 General	14

5.2.2.2.2	 MBS session creation	15

5.2.2.2.3	 MBS session deletion	15

5.2.2.2.4	 MBS session establishment for broadcast	16

5.2.2.2.5	 Establishment of shared delivery towards RAN node for multicast	17

5.2.2.2.6	 Establishment of individual delivery towards UPF for multicast	18

5.2.3	CDR generation	19

5.2.3.1	Introduction	19

5.2.3.2	Triggers for CHF CDR	19

5.2.3.2.1	General	19

5.2.3.2.2	Triggers for CHF CDR charging information addition	20

5.2.3.2.3	Triggers for CHF CDR partial record closure	20

5.2.3.2.4	Triggers for CHF CDR closure	20

5.2.4	Ga record transfer flows	20

5.2.5	Bmbs CDR file transfer	20

6	Definition of charging information	21

6.1	Data description for 5G MBS Session charging	21

6.1.1	Message contents	21

6.1.1.1	General	21

6.1.1.2	Charging Data Request message	22

6.1.1.3	Charging data response message	22

6.1.2	Ga message contents	23

6.1.3	CDR description on the Bmbs interface	23

6.1.3.1	General	23

6.1.3.2	MBS session charging CHF CDR data	23

6.2	5G MBS Session charging specific parameters	23

6.2.1	Definition of 5G MBS Session charging information	23

6.2.1.1	General	23

6.2.1.2	Definition of MBS session charging information	24

6.2.1.3	Definition of MBS container information	24

6.2.2	Detailed message format for converged charging	24

6.2.3	Formal 5G MBS Session charging parameter description	25

6.2.3.1	5G MBS Session CHF CDR parameters	25

6.2.3.2	5G MBS Session resources attributes	26

6.3	Bindings for 5G MBS Session charging	26

Annex <A> (informative): Change history	27

Foreword

This Technical Specification| has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is part of a series of documents that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [2], which provides an umbrella for other charging management TSs that specify:

-	the content of the CDRs per domain / subsystem / service (offline charging);

-	the content of real-time charging messages per domain / subsystem / service (online charging);

-	the functionality of online and offline charging for those domains / subsystems / services;

-	the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events).

The complete document structure for these TSs is defined in TS 32.240 [2].

The present document specifies the converged charging description for the 5G MBS Session based on the functional stage 2 description in TS 23.247 [9], TS 23.501 [10], TS 23.502 [11] and TS 23.503 [12].

This charging description includes the converged charging architecture and scenarios specific to the 5G MBS Session , with focus on the following charging scenarios:

-	MBS session charging for multicast communication.

-	MBS session charging for broadcast communication.

It further specifies the structure and content of the CDRs and the charging events for converged charging.

All references, abbreviations, definitions, descriptions, principles and requirements, used in the present document, that are common across 3GPP TSs, are defined in TR 21.905 [1]. Those that are common across charging management in 3GPP networks/domains, services or subsystems are provided in the umbrella TS 32.240 [2] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[4]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[5]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[6]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[7]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[8]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[9]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502:"Procedures for the 5G System".

[12]	3GPP TS 23.503:"Policy and Charging Control Framework for the 5G System; Stage 2".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bmbs	Reference point for the CDR file transfer from the 5G MBS Session CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N101	Reference point between MB-SMF and the CHF.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

AF	Application Function

AMF	Access and Mobility Management Function

BD	Billing Domain

CCS	Converged Charging System

CDF	Charging Data Function

CGF	Charging Gateway Function

CHF	Charging Function

CTF	Charging Trigger Function

FBC	Flow Based Charging

MBS	Multicast/Broadcast Service

MB-SMF	Multicast/Broadcast Session Management Function.

MB-UPF	Multicast/Broadcast User Plane Function

NF	Network Function

PCC	Policy and Charging Control

SMF	Session Management Function

UPF	User Plane Function

4	Architecture considerations

4.1	5G System architecture for Multicast and Broadcast Service

Figure 4.1.1 shows the non-roaming architecture for 5G Multicast and Broadcast Service as defined in TS 23.247 [9] for 5G data connectivity.

Figure 4.1.1: Non-roaming architecture for 5G Multicast and Broadcast Service in reference point representation

4.2	5G MBS Session converged charging architecture

In order to support MBS session charging, the MB-SMF embedding the CTF, generates charging events towards the CHF for converged charging.

Figure 4.2.1 depicts the 5G MBS architectural options for converged charging in service-based representation, with MB-SMF interacting with CHF.

Figure 4.2.1: 5G converged charging architecture for MBS

Figure 4.2.2 depicts the 5G converged charging architecture for MBS in reference point representation:

Figure 4.2.2: 5G converged charging architecture for MBS in reference point representation

The N101 reference point is defined for the interactions between MB-SMF and CHF in the reference point representation.

5	5G MBS Session charging principles and scenarios

5.1	5G MBS Session charging principles

5.1.1	General

The charging functions specified for the 5G MBS session charging:

-	MBS session in MB-SMF, refer to TS 23.247 [9].

-	service data flows, within MBS session, refer to TS 23.247 [9].

5.1.2	Requirements

The following are high-level charging requirements specific to 5G MBS session charging:

-	The MB-SMF shall support converged charging, if the 5G MBS charging is supported.

-	The MB-SMF shall support MBS session charging, if the 5G MBS charging is supported.

-	The MB-SMF may be capable of identifying data volumes or elapsed time for individual service data flows (flow based charging).

-	Every MBS session shall be assigned a unique identity number for billing purposes per PLMN.

5.1.3	Charging information

For the multicast communication, the MB-SMF may collect following charging information.

-	usage of the access and core network resources: the charging information shall describe the amount of multicast data transmitted;

-	usage duration: duration of MBS session is counted as the time interval from MBS session creation to MBS session deletion;

-	service area: the charging information shall describe area over which the MBS session data is distributed. MBS service area provided by AF referring to clause 6.2 of TS 23.247 [9], or a list of gNBs and UPFs that have established tunnels with MB-UPFs referring to clause 6.9 of TS 23.247 [9].

MB-SMF may report above charging information to CHF for the following cases.

-	MBS session creation and deletion;

-	Establishment and release of shared delivery towards gNBs;

-	Establishment and release of individual delivery towards UPFs.

For the broadcast communication, the MB-SMF may collect following charging information.

-	usage of the access and core network resources: the charging information shall describe the amount of broadcast data transmitted;

-	usage duration: duration of MBS session is counted as the time interval from MBS session creation to MBS session deletion;

-	service area: the charging information shall describe area over which the MBS session data is distributed. MBS service area provided by AF referring to clause 6.2 of TS 23.247 [9], or a list of gNBs that have established tunnels with MB-UPFs referring to clause 6.9 of TS 23.247 [9];

MB-SMF may report above charging information to CHF for the following cases.

-	MBS session creation, establishment and deletion.

5.1.4	Charging Identifier

Editor’s Note: The detailed definition of the charging id is FFS.

5.1.5	CHF selection

The CHF Address(es) selection by the MB-SMF can be done at the MBS session creation based on the following options:

-	NRF based discovery.

-	MB-SMF locally provisioned charging characteristics.

The priority order between these options depends on Operator's policies.

5.2	5G MBS Session converged charging scenarios

5.2.1	Basic principles

5.2.1.1	General

Converged charging may be performed by the MB-SMF interacting with CHF using Nchf specified in TS 32.290 [4] and TS 32.291 [5]. The MB-SMF shall be able to perform converged charging for each of the following:

-	Charging data related to MBS session with individual and shared traffic delivery for Multicast;

-	Charging data related to MBS session for Broadcast;

-	Charging data related to service data flows within the MBS session.

The MB-SMF shall be able to report charging events to CDF for CDR generation.

The MB-SMF shall be able to perform convergent charging by interacting with CHF, for charging data related to MBS sessions. The Charging Data Request and Charging Data Response are exchanged between the MB-SMF and the CHF, based on SCUR scenarios specified in TS 32.290 [4]. The Charging Data Request is issued by the MB-SMF towards the CHF when certain conditions (chargeable events) are met.

The MB-SMF initiates a charging session with Charging Data Request/Response [Initial], updates the charging session with Charging Data Request/Response [Update], and terminates the charging session with Charging Data Request/Response [Termination].

5.2.1.2	Applicable Triggers in the MB-SMF

When a charging event is issued towards the CHF, it includes details such as MBS charging identifier and also containers identifying the volume count, with charging condition change information.

Each trigger condition (i.e. chargeable event) defined for the 5G converged charging functionality with the associated behaviours when met is specified in this document and the basic trigger mechanism is specified in the TS 32.290[4].

Two categories of chargeable events are identified:

-	immediate report: chargeable events for which, when occurring, the current counts are closed and sent together with the charging data generated by the MB-SMF towards the CHF in a Charging Data Request. New counts are started by the MB-SMF.

-	deferred report: chargeable events for which, when occurring, the current counts are closed and stored together with the charging data generated by the MB-SMF. The stored counts will be sent to the CHF in next a Charging Data Request. New counts are started by the MB-SMF.

When more than one trigger condition to be met at same time (i.e. time stamp of triggers is the same) for the same count in the MB-SMF, the MB-SMF reports the used unit container with these triggers.

When a MBS session starts, and the converged charging is activated, the MB-SMF invokes a Charging Data Request [Initial] towards the CHF to get authorization to start based on the default triggers. The MB-SMF is optionally provided in a Charging Data Response [Initial] to override the default triggers, with a set of chargeable event triggers to be enabled, and the associated category (i.e. immediate or deferred report).

The triggers remain active until they are updated or disabled by subsequent Charging Data Response [Update] from the CHF or the MBS session is terminated.

Table 5.2.1.2-1 summarizes the set of default trigger conditions and their category which shall be supported by the MB-SMF. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Update, Termination] message sent from MB-SMF towards the CHF.

Table 5.2.1.2-1: Default Trigger conditions in MB-SMF

| Trigger Conditions | Trigger level | Converged Charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of MBS Session. | MBS session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Change of Charging conditions |  |  |  |  | Charging Data Request [Update] |
| Connection established with NG-RAN | MBS session | Deferred | Yes | Yes |  |
| Connection released with NG-RAN | MBS session | Deferred | Yes | Yes |  |
| Connection established with UPF | MBS session | Deferred | Yes | Yes |  |
| Tariff Time Change | MBS session | Deferred | Deferred | Yes |  |
| Connection released with UPF | MBS session | Deferred | Yes | Yes |  |
| Quota management |  |  |  |  |  |
| Time threshold reached | MBS session | Deferred | No | Yes |  |
| Time quota exhausted | MBS session | Deferred | No | Yes |  |
| Limit per MBS session |  |  |  |  |  |
| Expiry of data time limit per MBS session | MBS session | Immediate | No | Yes |  |
| Expiry of data volume limit per MBS session | MBS session | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | MBS session | Immediate | No | Yes |  |
| End of MBS session | MBS session | Immediate | No | No | Charging Data Request [Termination] |

For converged charging, the following details of chargeable events and corresponding actions in the MB-SMF are defined in Table 5.2.1.2-2:

Table 5.2.1.2-2: Chargeable events and their related actions in MB-SMF

| Chargeable event | Conditions | MB-SMF action |
| --- | --- | --- |
| Start of MBS session |  | Charging Data Request [Initial]. |
| Connection established with NG-RAN | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection released with NG-RAN | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection established with UPF | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection released with UPF | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Time threshold reached | If the corresponding trigger is enabled | Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time quota exhausted | If the corresponding trigger is enabled | Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of data volume limit per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Expiry of time limit per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Expiry of a limit of number of charging condition changes per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Tariff Time Change | If the corresponding trigger is enabled | Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| End of MBS session |  | Charging Data Request [Termination] Close the counts with time stamps |

5.2.1.3	MBS session charging

Converged charging allows the MB-SMF to collect charging information related to data volumes of MBS data packets received from AF or MBSTF per MBS session.

If MBS session specific converged charging is supported, this is achieved by FBC charging, see clause 5.2.1.2.

5.2.2	Message flows

5.2.2.1	General

The flows in the present document specify the interaction between the MB-SMF and the CHF for 5G MBS session converged charging functionality, in different scenarios, based on TS 23.247 [9] procedures and flows.

This interaction is based on Charging Data Request /Response specified in TS 32.290 [4], exchanged between the MB-SMF embedding the CTF and the CHF.

As a general principle, the steps in the figures for the message flows below correspond to the steps of figures in TS 23.247 [9], which is the reference. This document specifies the charging specific extension part.

5.2.2.2		5G MBS charging for multicast and broadcast communication

5.2.2.2.1		General

This clause specifies MBS session charging for multicast and broadcast communication as defined in TS 23.247 [9].

5.2.2.2.2		MBS session creation

The following figure 5.2.2.2.2-1 describes the charging procedure for MBS session creation, based on TS 23.247 [9] figure 7.1.1.2-1 and 7.1.1.3-1.

Figure 5.2.2.2.2-1: MBS session creation

Steps 1 to 11 per 3GPP TS 23.247 [9] Figure 7.1.1.2-1: MBS Session Creation without PCC,

13. If the dynamic PCC is deployed, refer to steps 21-27 in TS 23.247 [9] Figure 7.1.1.3-1: MBS Session Creation with PCC. If not, refer to step 13 in TS 23.247 [9] Figure 7.1.1.2-1: MBS Session Creation without PCC.

13ch-a.	The MB-SMF sends Charging Data Request [Initial] to the CHF for authorization for the subscriber to start the MBS session which is triggered by the start of MBS session.

13ch-b. The CHF opens the CDR for this MBS session.

13ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the MB-SMF.

Steps 14 to 22 per 3GPP TS 23.247 [9] Figure 7.1.1.2-1: MBS Session Creation without PCC.

5.2.2.2.3	 MBS session deletion

The following figure 5.2.2.2.3-1 describes charging procedures for MBS session deletion, based on TS 23.247 [9] figure 7.1.1.4-1 and 7.1.1.5-1.

Figure 5.2.2.2.3-1: MBS Session Deletion

Steps 1 to 7 per 3GPP TS 23.247 [9] Figure 7.1.1.4-1: MBS Session Deletion without PCC.

7ch-a.	The MB-SMF sends Charging Data Request [Termination] to the CHF, for terminating the charging associated with MBS session, with the trigger “End of MBS session”.

7ch-b. The CHF closes the CDR for MBS session.

7ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the MB-SMF.

Steps 8 to 14 per 3GPP TS 23.247 [9] Figure 7.1.1.4-1: MBS Session Deletion without PCC.

5.2.2.2.4		MBS session establishment for broadcast

The following figure 5.2.2.2.4-1 describes the charging procedure for MBS session establishment for broadcast, based on TS 23.247 [9] figure 7.3.1-1.

Figure 5.2.2.2.4-1: MBS session establishment for broadcast

Steps 1 to 8 per 3GPP TS 23.247 [9] Figure 7.3.1-1: MBS Session Establishment for Broadcast.

8ch-a.	The MB-SMF sends Charging Data Request [Update] to the CHF triggered by the connection established with NG-RAN.

8ch-b. The CHF updates the CDR.

8ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the MB-SMF.

5.2.2.2.5		Establishment of shared delivery towards RAN node for multicast

The following figure 5.2.2.2.5-1 describes multicast MBS session charging procedures for establishment of shared delivery towards RAN node, which occurs when the first UE is included in the context of the MBS session in the NG-RAN, or when handover to the target NG-RAN when the shared delivery tunnel has not yet been established in the target RAN node for this multicast MBS session, based on clause 7.2.1.4 of TS 23.247 [9].

Figure 5.2.2.2.5-1: Establishment of shared delivery towards RAN node

Steps 1 to 4 per 3GPP TS 23.247 [9] Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node.

4ch-a.	The MB-SMF sends Charging Data Request [Update] to the CHF when the corresponding trigger is armed.

4ch-b. The CHF updates the CDR.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the MB-SMF.

Steps 5 to 7 per 3GPP TS 23.247 [9] Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node.

5.2.2.2.6		Establishment of individual delivery towards UPF for multicast

The following figure 5.2.2.2.6-1 describes multicast MBS session charging procedures for establishment of individual delivery towards UPF, based on steps 11a-11e of TS 23.247 [9] figure 7.2.1.3-1, which occurs when the related NG-RAN does not support MBS and a tunnel between the UPF (PSA) and MB-UPF for 5GC individual MBS traffic delivery has not yet been established by the SMF for the multicast MBS session.

Figure 5.2.2.2.6-1: Establishment of individual delivery towards UPF

1. SMF decides to establish individual delivery for a multicast MBS session.

Steps 2 to 4 per 3GPP TS 23.247 [9] Figure 7.2.1.3-1: PDU Session modification for UE joining Multicast MBS session steps 11a to 11c.

4ch-a.	The MB-SMF sends Charging Data Request [Update] to the CHF when the corresponding trigger is armed.

4ch-b. The CHF updates the CDR.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the MB-SMF.

Steps 5 to 6 per 3GPP TS 23.247 [9] Figure 7.2.1.3-1: PDU Session modification for UE joining Multicast MBS session steps 11d to 11e.

5.2.3	CDR generation

5.2.3.1	Introduction

- The CHF CDRs for MBS session charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in detail the conditions for opening, adding information to, and closing the CHF CDR.

5.2.3.2	Triggers for CHF CDR

5.2.3.2.1	General

A MBS session charging CHF CDR is used to collect charging information related to the MBS session data information from a single data source (e.g. Application Service Provider) in Flow Based Charging.

A  CHF CDR shall be opened when the CHF receives Charging Data Request[Initial].

As an alternative to the default CHF behaviour, the "Individual Partial record" mechanism can be used based on Operator's policy configured in the CHF. In this case a new CDR shall be opened for each Charging Data Request[Initial, Update, Termination], charging information shall be added and the CDR shall then be closed. The Sequence Number will be incremented for each Charging Data Request[Initial, Update, Termination] received by the CHF.

5.2.3.2.2	Triggers for CHF CDR charging information addition

When the CHF receives Charging Data Request[Update], with the change conditions identified in Table 5.2.3.2.2.1 the charging information shall be added in the MBS session charging CHF CDR, and the CDR shall remain open, as the default supported mechanism.

Table 5.2.3.2.2.1: Triggers for CHF CDR charging information addition

| Trigger Conditions | Applicable for converged charging |
| --- | --- |
| Change of Charging conditions |  |
| Connection established with NG-RAN | Yes |
| Connection released with NG-RAN | Yes |
| Connection established with UPF | Yes |
| Connection released with UPF | Yes |
| Quota management triggers |  |
| Time threshold reached | Yes |
| Time quota exhausted | Yes |

In case the "Individual Partial record" mechanism is enabled, the Table 5.2.3.2.2.1 is not applicable. The charging information consists of a set of containers, which are added as "List of Multiple Unit Usage" parameter of the CHF CDR.

5.2.3.2.3	Triggers for CHF CDR partial record closure

When the CHF receives Charging Data Request [Update], with the change conditions identified in Table 5.2.3.2.3.1, the charging information shall be added in the MBS session charging CHF CDR, before the CDR is closed and a subsequent CHF CDR shall be opened with an incremented Sequence Number, as the default supported mechanism.

.

Table 5.2.3.2.3.1: Triggers for CHF CDR partial record closure

Trigger Conditions

Limit per MBS session

Expiry of data time limit per MBS session

Expiry of data volume limit per MBS session

Expiry of limit of number of charging condition changes

In case the "Individual Partial record" mechanism is enabled, the Table 5.2.3.2.3.1 is not applicable.

5.2.3.2.4	Triggers for CHF CDR closure

For MBS session charging, when the CHF receives Charging Data Request[Termination], the charging information shall be added in the MBS session charging CHF CDR and the CDR shall be closed.

5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [6].

5.2.5	Bmbs CDR file transfer

Details of the Bmbs protocol application are specified in TS 32.297 [7].

6	Definition of charging information

6.1	Data description for 5G MBS Session charging

6.1.1	Message contents

6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [4] and include charging information. The Charging Data Request can be of type [Initial, Update, Termination].

Table 6.1.1.1 describes the use of these messages for converged charging.

Table 6.1.11.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | MB-SMF | CHF |
| Charging Data Response | CHF | MB-SMF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2].

6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the MB-SMF as used for 5G converged charging for MBS session.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | - | Described in TS 32.290 [4] |
| NF Consumer Identification | M | - | Described in TS 32.290 [4] |
| NF Functionality | M | - | Described in TS 32.290 [4] |
| NF Name | OC | - | Described in TS 32.290 [4] |
| NF Address | OC | - | Described in TS 32.290 [4] |
| NF PLMN ID | OC | - | Described in TS 32.290 [4] |
| Invocation Timestamp | M | - | Described in TS 32.290 [4] |
| Invocation Sequence Number | M | - | Described in TS 32.290 [4] |
| Multiple Unit Usage | OC | - | Described in TS 32.290 [4] This field is not applicable to QBC. |
| Rating Group | M | - | Described in TS 32.290 [4] |
| Used Unit Container | OC | - | Described in TS 32.290 [4] |
| Triggers | OC | - | This field is described in TS 32.290 [4] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| MBS Container Information | OC | - | This field holds the 5G MBS session container specific information described in clause 6.2.1.3. |
| MBS Session Charging Information | OM | - | This field holds the MBS session specific information described in clause 6.2.1.2. This field is applicable to FBC. |

6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF to MB-SMF as used for 5G converged charging for MBS session.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Category for offline only charging | Description |
| --- | --- | --- | --- |
| Session Identifier | OC | - | Described in TS 32.290 [4] |
| Invocation Timestamp | M | - | Described in TS 32.290 [4] |
| Invocation Result | OC | - | Described in TS 32.290 [4] |
| Invocation Result Code | OC | - | Described in TS 32.290 [4] |
| Failed Parameter | OC | - | Described in TS 32.290 [4] |
| Failure Handling | OC | - | Described in TS 32.290 [4] |
| Invocation Sequence Number | M | - | Described in TS 32.290 [4] |
| Multiple Unit Information | OC | - | Described in TS 32.290 [4] This field is not applicable to QBC. |
| Result Code | OC | - | Described in TS 32.290 [4] |
| Rating Group | M | - | Described in TS 32.290 [4] |
| Time Quota Threshold | OC | - | Described in TS 32.290 [4] |
| Triggers | OC | - | This field is described in TS 32.290 [4] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| Triggers | OC | - | This field is described in TS 32.290 [4] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |
| MBS Session Charging Information | OM | - | This field holds the MBS session specific information described in clause 6.2.1.2. |

6.1.2	Ga message contents

See clause 5.2.4.

6.1.3	CDR description on the Bmbs interface

6.1.3.1	General

This clause describes the CDR content and format generated for 5G MBS session charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [8].

6.1.3.2	MBS session charging CHF CDR data

If enabled, CHF CDRs for MBS session charging shall be produced for each MBS session.

The fields of MBS session charging CHF CDR are specified in table 6.1.3.2.1.

Table 6.1.3.2.1: MBS session charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| NF Consumer Information | M | This field holds the information of the MB-SMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. MB-SMF) |
| NF Name | OC | This field holds the name of the MB-SMF used. |
| NF Address | OC | This fields holds the IP Address of the MB-SMF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the MB-SMF. |
| List of Multiple Unit Usage | OM | This field holds a list of changes in charging conditions for all service data flows within this MBS session. |
| Rating Group | OM | This field holds the rating group. |
| Used Unit Container | OC | This field holds the used units and information connected to the reported units. |
| Triggers | OC | This field holds the reason for closing the used unit container. |
| Trigger Timestamp | OC | This field holds the timestamp of the trigger. |
| Time | OC | This field holds the amount of used time. |
| Downlink Volume | OC | This field holds the amount of used volume in downlink direction. |
| Rating Indicator | OC | This field indicates if the units have been rated or not. |
| Local Sequence Number | M | This field holds the container sequence number. |
| MBS Container Information | OC | This field holds the 5G MBS session specific information defined in clause 6.2.1.3. |
| Record Opening Time | M | Described in TS 32.298 [8] |
| Duration | M | Described in TS 32.298 [8] |
| Record Sequence Number | C | Described in TS 32.298 [8] |
| Cause for Record Closing | M | Described in TS 32.298 [8] |
| Diagnostics | OM | Described in TS 32.298 [8] |
| Local Record Sequence Number | OM | Described in TS 32.298 [8] |
| Record Extensions | OC | Described in TS 32.298 [8] |
| MBS Session Charging Information | OM | This field holds the 5G MBS session specific information defined in clause 6.2.1.2. |

6.2	5G MBS Session charging specific parameters

6.2.1	Definition of 5G MBS Session charging information

6.2.1.1	General

The Charging Information parameter used for 5G MBS session charging is provided in the following clauses.

6.2.1.2	Definition of MBS session charging information

MBS specific charging information used for 5G data connectivity charging is provided within the MBS Session Charging Information.

The detailed structure of the MBS Session Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of MBS Session Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| MBS Session ID | M | This field holds identifier of MBS session. |
| MBS Service Type | M | This field holds the type of the MBS session. |
| MBS Service Area | OC | This field holds MBS Service Area served by the MBS session. |
| QoS Information | OC | This field holds the QoS applied to MBS session. |
| MBS Session Start Time | OC | This field holds the timestamp when MBS session starts. |
| MBS Session Stop Time | OC | This field holds the timestamp when MBS session terminates. |
| Serving Network Function ID | OC | This field holds the identity of the serving network function. It may have multiple occurrences. |
| Serving Network Function Information | M | This field holds the Information of the serving network function: - AMF for the MBS sessions being served by MB-SMF in non-roaming |
| AMF Identifier | OC | This field holds the AMF identifier. |

6.2.1.3	Definition of MBS container information

Used Unit Container, described in table 6.2.1.3.1, specific charging information used for 5G data connectivity charging is provided within the MBS Container Information described in table 6.2.1.3.1.

Table X.X.X.Y.1: Structure of MBS Container Information

| Information Element | Category | Description |
| --- | --- | --- |
| Time of First Usage | OC | This field holds the Timestamp when the first transmitted IP packet of the service data flow matching the current used unit container. |
| Time of Last Usage | OC | This field holds the Timestamp when the last transmitted IP packet of the service data flow matching the current used unit container. |
| QoS Information | OC | This field holds the QoS applied during the service data container interval. |
| Established Connection Information | OC | This field holds a list of NG-RAN nodes establishing connection, or a list of UPFs establishing connection with MB-UPF. |

6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by MB-SMF for 5G data connectivity converged charging or offline only charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2.1 defines the basic structure of the supported fields in the Charging Data Request message for 5G data connectivity converged charging for MB-SMF to support 5G MBS charging.

Table 6.2.2.1: Supported fields in Charging Data Request message

| Information Element | Functionality of MB-SMF | FBC |
| --- | --- | --- |
|  | Charging Service | Converged Charging |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | -UT- |
| NF Consumer Identification |  | IUT- |
| Invocation Timestamp |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Triggers |  | -UT- |
| Multiple Unit Usage |  | IUT- |
| Rating Group |  | IUT- |
| Used Unit Container |  | -UT- |
| Triggers |  | -UT- |
| MBS Container Information |  | -UT- |
| MBS Session Charging Information |  | IUT- |
| MBS Service Type |  | IUT- |
| MBS Service Area |  | IUT- |
| QoS Information |  | IUT- |
| MBS Session Start Time |  | I--- |
| MBS Session Stop Time |  | --T- |
| Serving Network Function ID |  | IUT- |

Table 6.2.2.2 defines the basic structure of the supported fields in the Charging Data Response message for 5G data connectivity converged charging for MB-SMF to support 5G MBS charging.

Table 6.2.2.2: Supported fields in Charging Data Response message

| Information Element | Functionality of MB-SMF | FBC |
| --- | --- | --- |
|  | Charging Service | Converged Charging |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | I--- |
| Invocation Timestamp |  | IUT- |
| Invocation Result |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Triggers |  | IU-- |
| Multiple Unit Information |  | IU-- |
| Result Code |  | IU-- |
| Rating Group |  | IU-- |
| Time Quota Threshold |  | IU-- |
| Triggers |  | IU-- |
| MBS Session Charging Information |  | IU-- |
| MBS Service Type |  | - |
| MBS Service Area |  | - |
| QoS Information |  | - |
| MBS Session Start Time |  | - |
| MBS Session Stop Time |  | - |
| Serving Network Function ID |  | - |

6.2.3	Formal 5G MBS Session charging parameter description

6.2.3.1	5G MBS Session CHF CDR parameters

The detailed definitions, abstract syntax and encoding of the MBS session charging CHF CDRs parameters are specified in TS 32.298 [8].

6.2.3.2	5G MBS Session resources attributes

The detailed definitions of resources attributes used for 5G MBS session charging are specified in TS 32.291 [5].

6.3	Bindings for 5G MBS Session charging

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | SA5#151 | S5-237033 |  |  |  | Initial skeleton | 0.0.0 |
| 2023-10 | SA5#151 | S5-236913 |  |  |  | Add CHF selection mechanism for MB-SMF | 0.1.0 |
|  |  | S5-236915 |  |  |  | Add 5MBS CDR genartion requirements |  |
|  |  | S5-236916 |  |  |  | Add 5G MBS charging information definition |  |
|  |  | S5-236917 |  |  |  | MBS Tariff change trigger and charging event |  |
|  |  | S5-236918 |  |  |  | Add reference point between MB-SMF and CHF and MBS charging ID |  |
|  |  | S5-236920 |  |  |  | Add 5G MBS charging procedure for multicast and broadcast communication creation and deletion |  |
|  |  | S5-236921 |  |  |  | Add 5G MBS charging procedure of broadcast session establishment |  |
|  |  | S5-236922 |  |  |  | Add 5G MBS charging procedure of shared and individual delivery establishment for multicast |  |
| 2023-11 | SA5#152 | S5-238019 |  |  |  | Rel-18 pCR 32.27x Add scope | 0.2.0 |
|  |  | S5-238020 |  |  |  | Rel-18 pCR 32.27x Add references |  |
|  |  | S5-238021 |  |  |  | Rel-18 pCR 32.27x Add definitions of terms, symbols and abbreviations |  |
|  |  | S5-238022 |  |  |  | Rel-18 pCR TR 32.27x MBS Charging Principles |  |
|  |  | S5-238023 |  |  |  | Rel-18 pCR 32.27x Add general information for 5MBS charging |  |
|  |  | S5-238024 |  |  |  | Rel-18 pCR 32.27x Add MBS session charging information |  |
|  |  | S5-238025 |  |  |  | Rel-18 pCR 32.27x Add Bmbs interface information |  |
|  |  | S5-238026 |  |  |  | Rel-18 pCR 32.27x Add Ga interface information |  |