# 3GPP TSG SA WG5 Meeting #152                 	           S5-238006

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Add message flows for time synchronization service charging

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the message flows in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Describe the TSC and TS services enabled by 5G System features, including

-	time synchronization services, for subscribe/unsubscribe/activate/modify/deactivate the time synchronization service with PTP or ASTI based time distribution.

-	QoS and TSC assistance services, for setup/modify the TSN QoS requirement or time sensitive communication traffic characteristics.

-	TSC and TS service notification, for notifying AF with the time synchronization status or change of QoS and TSC assistance information.

Describe the message flows for time synchronization service charging from TSCTSF, using IEC, PEC or ECUR scenario.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

5.2.2.3	TSC and TS service enabler charging from TSCTSF

5.2.2.3.1	General

This clause below describes the message flow for TSC and TS service enabler charging from TSCTSF, when trusted AF interact with TSCTSF directly.

5.2.2.3.2	Time synchronization service charging – IEC

The following figure 5.2.2.3.2-1 describes time synchronization service charging from TSCTSF in the IEC scenario.

Figure 5.2.2.3.2-1: Time synchronization service charging from TSCTSF --- IEC

1.	AF Request/Subscribe to TSCTSF service: the TSCTSF receives a service request/subscribe message from an AF with the parameters as required by the NF service, e.g. time synchronization service (un)subscription, (de)activation, modification, defined in TS 23.502 [4] clause 4.15.9.

1ch-a.	Charging Data Request [Event]: The TSCTSF sends Charging Data Request [Event] to CHF, triggered by “AF request/subscribe received”.

1ch-b.	Create CDR: The CHF creates a CDR for the request/subscribe of NF services provided by TSCTSF.

1ch-c.	Charging Data Response [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in TS 23.502 [12].

3.	Response to AF: TSCTSF sends the service response message to inform AF.

5.2.2.3.3	Time synchronization service charging – PEC

The following figure 5.2.2.3.2-1 describes time synchronization service charging from TSCTSF in the PEC scenario.

Figure 5.2.2.3.3-1: Time synchronization service charging from TSCTSF --- PEC

1.	AF Request/Subscribe to TSCTSF service: the TSCTSF receives a service request/subscribe message from an AF with the parameters as required by the NF service, e.g. time synchronization service (un)subscription, (de)activation, modification, defined in TS 23.502 [4] clause 4.15.9.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in TS 23.502 [12].

3.	Response to AF: TSCTSF sends the service response message to inform AF.

3ch-a.	Charging Data Request [Event]: The TSCTSF sends Charging Data Request [Event] to CHF, triggered by “Response to AF”.

3ch-b.	Create CDR: The CHF creates a CDR for the request/subscribe of NF services provided by TSCTSF.

3ch-c.	Charging Data Response [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

5.2.2.3.4	Time synchronization service charging – ECUR

The following figure 5.2.2.3.2-1 describes time synchronization service charging from TSCTSF in the ECUR scenario.

Figure 5.2.2.3.4-1: Time synchronization service charging from TSCTSF --- ECUR

1.	AF Request/Subscribe to TSCTSF service: the TSCTSF receives a service request/subscribe message from an AF with the parameters as required by the NF service, e.g. time synchronization service (un)subscription, (de)activation, modification, defined in TS 23.502 [4] clause 4.15.9.

1ch-a.	Charging Data Request [Initial]: The TSCTSF sends Charging Data Request [Initial] to CHF with requested unit, triggered by “AF request/subscribe received”.

1ch-b.	Open CDR: The CHF opens a CDR for the request/subscribe of NF services provided by TSCTSF.

1ch-c.	Charging Data Response [Initial]: The CHF acknowledges by sending Charging Data Response [Initial] to the TSCTSF.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in TS 23.502 [12].

3.	Response to AF: TSCTSF sends the service response message to inform AF.

4ch-a.	Charging Data Request [Termination]: The TSCTSF sends Charging Data Request [Termination] to CHF, triggered by “Response to AF”.

4ch-b.	Close CDR: The CHF close the CDR.

4ch-c.	Charging Data Response [Termination]: The CHF acknowledges by sending Charging Data Response [Termination] to the TSCTSF.

End of change