# 3GPP TSG SA WG5 Meeting #152                 	          S5-238007

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Add message flows for QoS and TSC assistance service charging

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the message flows in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Describe the message flows for QoS and TSC assistance service charging from TSCTSF, using IEC, PEC or ECUR scenario.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

5.2.2.3.5	QoS and TSC assistance service charging – IEC

The following figure 5.2.2.3.5-1 describes QoS and TSC assistance service charging from TSCTSF in the IEC scenario.

Figure 5.2.2.3.5-1: QoS and TSC assistance service charging from TSCTSF --- IEC

1.	Ntsctsf_QoSandTSCAssistance request: the TSCTSF receives a Ntsctsf_QoSandTSCAssistance service request/subscribe message from an AF with the parameters as required by the NF service, e.g. AF session with required QoS, defined in TS 23.502 [4] clause 4.15.6.6.

1ch-a.	Charging Data Request [Event]: The TSCTSF sends Charging Data Request [Event] to CHF, triggered by “AF request/subscribe received”.

1ch-b.	Create CDR: The CHF creates a CDR for the request/subscribe of NF services provided by TSCTSF.

1ch-c.	Charging Data Response [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in clause 4.15.6.6/4.15.6.6a TS 23.502 [12].

3.	Ntsctsf_QoSandTSCAssistance response: TSCTSF sends the Ntsctsf_QoSandTSCAssistance service response message to inform AF.

5.2.2.3.6	QoS and TSC assistance service charging – PEC

The following figure 5.2.2.3.6-1 describes QoS and TSC assistance service charging from TSCTSF in the PEC scenario.

Figure 5.2.2.3.6-1: QoS and TSC assistance service charging from TSCTSF --- PEC

1.	Ntsctsf_QoSandTSCAssistance request: the TSCTSF receives a Ntsctsf_QoSandTSCAssistance service request/subscribe message from an AF with the parameters as required by the NF service, e.g. AF session with required QoS, defined in TS 23.502 [4] clause 4.15.6.6.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in clause 4.15.6.6/4.15.6.6a TS 23.502 [12].

3.	Ntsctsf_QoSandTSCAssistance response: TSCTSF sends the Ntsctsf_QoSandTSCAssistance service response message to inform AF.

3ch-a.	Charging Data Request [Event]: The TSCTSF sends Charging Data Request [Event] to CHF, triggered by “Response to AF”.

3ch-b.	Create CDR: The CHF creates a CDR for the request/subscribe of NF services provided by TSCTSF.

3ch-c.	Charging Data Response [Event]: The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

5.2.2.3.7	QoS and TSC assistance service charging – ECUR

The following figure 5.2.2.3.7-1 describes QoS and TSC assistance service charging from TSCTSF in the ECUR scenario.

Figure 5.2.2.3.7-1: QoS and TSC assistance service charging from TSCTSF --- ECUR

1.	Ntsctsf_QoSandTSCAssistance request: the TSCTSF receives a Ntsctsf_QoSandTSCAssistance service request/subscribe message from an AF with the parameters as required by the NF service, e.g. AF session with required QoS, defined in TS 23.502 [4] clause 4.15.6.6.

1ch-a.	Charging Data Request [Initial]: The TSCTSF sends Charging Data Request [Initial] to CHF with requested unit, triggered by “AF request/subscribe received”.

1ch-b.	Open CDR: The CHF opens a CDR for the request/subscribe of NF services provided by TSCTSF.

1ch-c.	Charging Data Response [Initial]: The CHF acknowledges by sending Charging Data Response [Initial] to the TSCTSF.

2.	TSCTSF service fulfilment procedure: TSCTSF performs the actions needed to fulfil the service request/subscribe, following the message flows for each scenario as specified in clause 4.15.6.6/4.15.6.6a TS 23.502 [12].

3.	Ntsctsf_QoSandTSCAssistance response: TSCTSF sends the Ntsctsf_QoSandTSCAssistance service response message to inform AF.

3ch-a.	Charging Data Request [Termination]: The TSCTSF sends Charging Data Request [Termination] to CHF, triggered by “Response to AF”.

3ch-b.	Close CDR: The CHF close the CDR.

3ch-c.	Charging Data Response [Termination]: The CHF acknowledges by sending Charging Data Response [Termination] to the TSCTSF.

End of change