# 3GPP TSG SA WG5 Meeting #152                 	          S5-238012

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Define the information for time synchronization service

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the information for time synchronization service in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

The structure for Time Synchronization Information and Time Synchronization Status Information are specified, applicable for time synchronization service charging by TSCTSF.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.2.1.6	Definition of Time Synchronization Information

The detailed structure of the Time Synchronization Information can be found in table 6.2.1.6-1.

Table 6.2.1.6-1: Structure of Time Synchronization Information

| Information Element | Category | Description |
| --- | --- | --- |
| Distribution method of timing information | OM | This parameter indicates the distribution method of timing information, defined in clause 5.27.1.9 in TS 23.501 [11]. |
| TSN time domain number | OC | This parameter indicates the gPTP domain (identified by a domain number) that is assumed by the CNC as the reference clock for time information in the scheduled traffic (gate control) information, PSFP information and bridge delay related information, defined in Annex K.1 in TS 23.501 [11]. |
| Temporal validity information | OC | This parameter indicates the duration when the time synchronization service is active for the targeted AF sessions, defined in clause 5.27.1.11 in TS 23.501 [11]. |
| Spatial validity information | OC | This parameter indicates the geographical area (e.g. a civic address or shapes) or a TA list in which time synchronization service is enabled for the targeted AF sessions, defined in clause 5.27.1.10 in TS 23.501 [11]. |
| Time synchronization error budget | OC | This parameter indicates the time synchronization budget for the time synchronization service, defined in clause 5.27.1.9 in TS 23.501 [11]. |
| Synchronization state | OC | This parameter indicates the state of the node synchronization, defined in clause 5.27.1.12 TS 23.501 [11]. |
| Clock quality | OC | This parameter holds the quality information of clock, including traceability to GNSS, traceability to UTC, frequency stability, clock accuracy, defined in clause 5.27.1.12 TS 23.501 [11]. |
| Parent time source | OC | This parameter indicates the primary source the node is currently using, defined in clause 5.27.1.12 TS 23.501 [11]. |

End of change