3GPP TSG-SA5 Meeting #152 	S5-238033

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0024 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.256 Addition of CHF to CHF flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of flows for the CHF-to-CHF connection in the LBO roaming scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of flows for CHF-to-CHF connection and renaming of the SMF to two CHFs scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CHF-to-CHF connection will be undefined for the LBO roaming scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5.1, 5.2.2.5.2, 5.2.2.5.3, 5.2.2.5.4, 5.2.2.5.5, 5.2.2.5.x1 (new), 5.2.2.5.x2 (new), 5.2.2.5.x3 (new), and 5.2.2.5.x4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS 32.256 CR 0022 |  |  |
| Other comments: |  | The Figure 4.2.2.x refers to the new architecture introduced in CR 0022 (SA5#151). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-237782 |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.5.1	General

In roaming, for registration management charging, the AMF in VPLMN:

-	shall support PEC scenario with V-CHF.

-	may support IEC or ECUR scenario with V-CHF.

-	may support PEC, IEC or ECUR scenario with H-CHF.

In roaming, for registration management charging, the V-CHF:

-	may support PEC, IEC or ECUR scenario with H-CHF.

The architecture in figure 4.2.2.2 is applicable for flows in figures 5.2.2.5.2.1, 5.2.2.5.3.1, and 5.2.2.5.4.1, and architecture in figure 4.2.2.x is applicable for flows in figures 5.2.2.5.x1.1, 5.2.2.5.x2.1, 5.2.2.5.x3.1, and 5.2.2.5.x4.1

Second change

##### 5.2.2.5.2	Registration - PEC to HPLMN and AMF to H-CHF and V-CHF

The following figure 5.2.2.5.2.1 describes a Registration charging message flow in PEC to HPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [201], where the AMF interacts with H-CHF and V-CHF.

Figure 5.2.2.5.2.1: Roaming Registration – PEC to HPLMN and AMF to H-CHF and V-CHF

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502 [201], with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. Upon Registration completed trigger, the AMF in VPLMN may send Charging Data Request [Event] to H-CHF for the UE successful registration, indicating "roamer out".

21ch-b. The H-CHF creates the CDR for this registration.

21ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21ch-d. The AMF in VPLMN sends Charging Data Request [Event] to the V-CHF for the UE successful registration, indicating "roamer in".

21ch-e. The V-CHF creates the CDR for this registration.

21ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21b-24.	Registration procedure continues as per steps 21b to 24 of figure 4.2.2.2.2-1 of TS 23.502 [201].

Third change

##### 5.2.2.5.3	Registration – IEC to HPLMN and AMF to H-CHF and V-CHF

The following figure 5.2.2.5.3.1 describes a Registration charging message flows in IEC to HPLMN scenario for roaming, based on the clause 5.2.2.2.2, where the AMF interacts with H-CHF and V-CHF.

Figure 5.2.2.5.3.1: Roaming Registration – IEC to HPLMN and AMF to H-CHF and V-CHF

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502 [201], with PCF, AUSF and UDM in HPLMN.

16ch-a. Upon Registration checked trigger, the AMF in VPLMN sends Charging Data Request [Event] to H-CHF for the UE registration to be granted authorization, rated and accounted, indicating "roamer out".

16ch-b. Accounting and Rating control by the H-CHF for the registration and CDR creation.

16ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN, granting authorization to the registration.

21ch-a. Upon Registration completed trigger, the AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful registration, indicating "roamer in".

21ch-b. The V-CHF creates the CDR for this registration.

21ch-c. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

17 to 24. Same steps as in Figure 5.2.2.2.2.1.

Fourth change

##### 5.2.2.5.4	Registration – ECUR to HPLMN and AMF to H-CHF and V-CHF

The following figure 5.2.2.5.4.1 describes a Registration charging message flows in ECUR to HPLMN scenario for roaming, based on the clause 5.2.2.2.2, where the AMF interacts with H-CHF and V-CHF.

Figure 5.2.2.5.4.1: Roaming Registration – ECUR to HPLMN and AMF to H-CHF and V-CHF

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502 [201] , with PCF, AUSF and UDM in HPLMN.

16ch-a. Upon Registration checked trigger, the AMF sends Charging Data Request [Initial] to H-CHF for the UE registration to be granted authorization, indicating "roamer out".

16ch-b. Account and Rating control by the H-CHF for the registration and open a CDR.

16ch-c. The H-CHF acknowledges by sending Charging Data Response [Initial] to the AMF in VPLMN, granting authorization to the registration.

17 to 21. Same steps as in Figure 5.2.2.2.2.1.

21ch-a. The AMF in VPLMN sends Charging Data Request [Termination] to H-CHF for the UE successful registration.

21ch-b. The H-CHF closes the CDR for this registration.

21ch-c. The H-CHF acknowledges by sending Charging Data Response [Termination] to the AMF in VPLMN.

21ch-d. The AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful registration, indicating "roamer in".

21ch-e. The V-CHF creates the CDR for this registration.

21ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21b to 24. Same steps as in Figure 5.2.2.2.2.1.

Fifth change

##### 5.2.2.5.5	Deregistration – PEC to HPLMN and AMF to H-CHF and V-CHF

The following figure 5.2.2.5.5.1 describes a deregistration UE-initiated or Network-initiated in PEC to HPLMN for roaming, based on figure 4.2.2.3.2-1 and figure 4.2.2.3.3-1 of TS 23.502 [201], where the AMF interacts with H-CHF and V-CHF.

Figure 5.2.2.5.5.1: Roaming Deregistration – PEC to HPLMN and AMF to H-CHF and V-CHF

1.	Deregistration Request from the UE or towards the UE.

1ch-a. Upon Deregistration trigger, the AMF in VPLMN sends Charging Data Request [Event] to H-CHF for the UE successful deregistration, indicating "roamer out".

1ch-b. The H-CHF creates the CDR for this deregistration.

1ch-c. The H-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

1ch-d. The AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful deregistration, indicating "roamer in".

1ch-e. The V-CHF creates the CDR for this deregistration.

1ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN

2-8	Deregistration procedure steps.

Sixth change

##### 5.2.2.5.x1	Registration - PEC to HPLMN and AMF to V-CHF

The following figure 5.2.2.5.x1.1 describes a Registration charging message flow in PEC to HPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [201], where the AMF interacts with V-CHF and V-CHF interacts with H-CHF.

Figure 5.2.2.5.x1.1: Roaming Registration – PEC to HPLMN and AMF to H-CHF

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502 [201], with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a.	The AMF in VPLMN sends Charging Data Request [Event] to the V-CHF for the UE successful registration, indicating "roamer in".

21ch-b.	The V-CHF creates the CDR for this registration.

21ch-c.	Upon reception of Charging Data Request [Event] for a Registration completed trigger, indicating "roamer in", the V-CHF may send Charging Data Request [Event] to H-CHF for the UE successful registration, indicating "roamer out".

21ch-d.	The H-CHF creates the CDR for this registration.

21ch-e.	The H-CHF acknowledges by sending Charging Data Response [Event] to the V-CHF.

21ch-f.	The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN. Depending on operator polices this step may be performed before 21ch-c.

21b-24.	Registration procedure continues as per steps 21b to 24 of figure 4.2.2.2.2-1 of TS 23.502 [201].

Seventh change

##### 5.2.2.5.x2	Registration – IEC to HPLMN and AMF to V-CHF

The following figure 5.2.2.5.x2.1 describes a Registration charging message flows in IEC to HPLMN scenario for roaming, based on the clause 5.2.2.2.2, where the AMF interacts with V-CHF and V-CHF interacts with H-CHF.

Figure 5.2.2.5.x2.1: Roaming Registration – IEC to HPLMN and AMF to V-CHF

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502 [201], with PCF, AUSF and UDM in HPLMN.

16ch-a.	Upon Registration checked trigger, the AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE registration to be granted authorization, rated and accounted, indicating "roamer in".

16ch-b.	The V-CHF creates the CDR for this registration.

16ch-c.	Upon reception of Charging Data Request [Event] for a Registration completed trigger, indicating "roamer in", the V-CHF may send Charging Data Request [Event] to H-CHF for the UE successful registration, indicating "roamer out".

16ch-d.	Accounting and Rating control by the H-CHF for the registration and CDR creation.

16ch-e.	The H-CHF acknowledges by sending Charging Data Response [Event] to the V-CHF, granting authorization to the registration.

16ch-f.	The V-CHF forwards the acknowledges from the H-CHF by sending Charging Data Response [Event] to the AMF in VPLMN, granting authorization to the registration.

17 to 21.	Same steps as in figure 5.2.2.2.2.1.

21ch.	The AMF and V-CHF may perform a PEC following clause 5.2.2.5.2 steps 21ch-d to 21-ch-f.

21b to 24.	Same steps as in figure 5.2.2.2.2.1.

Eighth change

##### 5.2.2.5.x3	Registration – ECUR to HPLMN and AMF to V-CHF

The following figure 5.2.2.5.x3.1 describes a Registration charging message flows in ECUR to HPLMN scenario for roaming, based on the clause 5.2.2.2.2, where the AMF interacts with V-CHF and V-CHF interacts with H-CHF.

Figure 5.2.2.5.x3.1: Roaming Registration – ECUR to HPLMN and AMF to V-CHF

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502 [201], with PCF, AUSF and UDM in HPLMN.

16ch-a.	Upon Registration checked trigger, the AMF in VPLMN sends Charging Data Request [Initial] to V-CHF for the UE registration to be granted authorization, rated, and accounted, indicating "roamer in".

16ch-b.	The V-CHF opens the CDR for this registration.

16ch-c.	Upon reception of Charging Data Request [Initial] for a Registration completed trigger, indicating "roamer in", the V-CHF may send Charging Data Request [Initial] to H-CHF for the UE successful registration, indicating "roamer out".

16ch-d.	Accounting and Rating control by the H-CHF for the registration and opens the CDR.

16ch-e.	The H-CHF acknowledges by sending Charging Data Response [Initial] to the V-CHF, granting authorization to the registration.

16ch-f.	The V-CHF forwards the acknowledges from the H-CHF by sending Charging Data Response [Initial] to the AMF in VPLMN, granting authorization to the registration.

17 to 21.	Same steps as in figure 5.2.2.2.2.1.

21ch-a.	The AMF in VPLMN sends Charging Data Request [Termination] to H-CHF for the UE successful registration.

21ch-b.	The V-CHF closes the CDR for this registration.

21ch-c.	Upon reception of Charging Data Request [Termination] for the UE successful registration, the V-CHF may send Charging Data Request [Termination] to H-CHF for the UE successful registration.

21ch-d.	The H-CHF closes the CDR for this registration.

21ch-e.	The H-CHF acknowledges by sending Charging Data Response [Termination] to the V-CHF.

21ch-f.	The V-CHF forwards the acknowledgement from the H-CHF by sending Charging Data Response [Termination] to the AMF in VPLMN.

21b to 24.	Same steps as in figure 5.2.2.2.2.1.

Ninth change

##### 5.2.2.5.x4	Deregistration – PEC to HPLMN and AMF to V-CHF

The following figure 5.2.2.5.x4.1 describes a deregistration UE-initiated or Network-initiated in PEC to HPLMN for roaming, based on figure 4.2.2.3.2-1 and figure 4.2.2.3.3-1 of TS 23.502 [201], where the AMF interacts with V-CHF and V-CHF interacts with H-CHF.]

Figure 5.2.2.5.x4.1: Roaming Deregistration – PEC to HPLMN and AMF to V-CHF

1.	Deregistration Request from the UE or towards the UE.

1ch-a.	Upon Deregistration trigger, the AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful deregistration, indicating "roamer out".

1ch-b.	The V-CHF creates the CDR for this deregistration.

1ch-c.	Upon reception of Charging Data Request [Event] for the UE successful deregistration trigger, indicating "roamer in", the V-CHF may send Charging Data Request [Event] to H-CHF for the UE successful registration, indicating "roamer out".

1ch-d.	The H-CHF creates the CDR for this deregistration.

1ch-e.	The H-CHF acknowledges by sending Charging Data Response [Event] to the V-CHF.

1ch-f.	The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN. Depending on operator polices this step may be performed before 1ch-c.

2-8.	Deregistration procedure steps, following figure 4.2.2.3.2-1 and figure 4.2.2.3.3-1 of TS 23.502 [201]

End of changes