3GPP TSG-SA5 Meeting #152 	S5-238054

# Chicago, USA, 13 Nov 2023 – 17 Nov 2023	Revision of S5-237476

# Source:	Amdocs, Nokia, Nokia Shanghai Bell

# Title:	Charging Domain in Nchf Converged Charging SBI

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# Rationale

This pCR is to propose a solution for the study topic for CHF selection by Charging Domain.

# 4	Detailed proposal

First change

### 5.4.4	Possible Solutions

#### 5.4.4.x	Solution #4.x:  Specify Charging Domain in Nchf Converged Charging SBI

A possible solution for key issue #4c is to enhance the Nchf_ConvergedCharging SBI so that a consumer NF indicates the intended domain(s) or subsystem(s) in a charging request to the CHF producer.

A list of ‘requested’ charging domain(s) and subsystem(s) could be added to Nchf_ConvergedCharging SBI, analogous to NodeFunctionality (i.e. as an enumerated list). This is necessary because the charging container (e.g., PDUSessionInformation) in a service request could be applicable to two or more charging domains or subsystems supported by a single CHF.

Table 5.4.4.x-1: New Attribute in Charging Data Request

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestedChargingDomain | ChargingDomain | OC | 0..1 | A charging domain or a subsystem for which charging is requested by NF consumer |  |

The data type ‘ChargingDomain’ could be defined as an enumerated list. A sample enumeration is shown below.

Table 5.4.4.x-2: Sample Enumeration Values for Requested Charging Domain

| Enumeration value | Description |
| --- | --- |
| NEF_NB_API | Network Exposure Function Northbound API Domain Charging |
| Data_Connectivity | Data Connectivity Domain Charging |
| 5G_Connection_And_Mobility | 5G Connection and Mobility Domain Charging |
| SMS | SMS Domain Charging |
| IMS | IP Multimedia System Domain Charging |
| MMS | Multimedia Messaging Service Domain Charging |
| MultiMedia_Telephony | Multimedia Telephony Domain Charging |
| Proximity_Based_Services | Proximity Based Services Domain Charging |
| MBMS | Multimedia Broadcast and Multicast Service Domain Charging |
| NSPA | Network Slice Performance and Analytics Domain Charging |
| NSM | Network Slice Management Domain Charging |
| NSAC | Network Slice Admission Control Domain Charging |
| NSAA | Network Slice Authorization and Authentication Domain Charging |
| Edge_Computing | Edge Computing Domain Charging |

The use of ‘requested charging domain’ could be negotiated between consumer and producer via SupportedFeatures. A new ‘SupportedFeature’ can be added, as defined below

Table 5.4.4.x-3: Supported Feature for Charging By Domain

| Feature number | Feature Name | Description |
| --- | --- | --- |
| X | ChargingDomain | This feature indicates support for charging for one or more charging domains or subsystems |

End of changes