3GPP TSG-SA5 Meeting #152 	S5-238073

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0037 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 32.254 Correcting identifiers as a list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 32.291 the external individual identifier can be a list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the external identifier as a list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies between specifications may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.4.1, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291 CR 0517 TS 32.298 CR 0964 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-237759 |  |  |  |  |  |  |  |  |

First change

##### 6.3.1.4.1	Definition of the NEF API Charging Information

Network Exposure Function API specific charging information is provided within the NEF API Charging Information. The detailed structure of the NEF API Charging Information can be found in table 6.3.1.4.1.

Table 6.3.1.4.1: Structure of the NEF API Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| External Individual Identifier | OC | This parameter holds the external identifier or the MSISDN associated to the GPSI of the individual UE. |
| External Individual Id List | OC | This parameter holds a list of external identifiers, used instead of External Individual Identifier if there is more than one. |
| External Group Identifier | OC | This parameter holds the external identifier for a group of individual UE(s). |
| Internal Group Identifier | OC | The internal Identifier identifying a group of individual UE(s). |
| API Direction | M | This field holds the direction to indicate if it is an API invocation from an AF or notification to an AF. |
| API Target Network Function | OC | This field holds the identifier of the network function that either is the destination of the API invocation or triggers the notification: e.g. PCF, UDM, AMF, SMF, UPF. |
| API Result Code | OC | This parameter holds the result of API Invocation. |
| API Name | M | This field holds the name of the API invoked. |
| API Reference | OC | This field holds the reference to the definition of the format of the API invocation, this can be a URI or refence to the standard where it's specified |
| API Content | OC | This field holds the actual content of the API invocation, in the format described by the API Reference |

Second change

### 6.3.4	Detailed message format for converged charging

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.4.1 illustrates the basic structure of the supported fields in the Charging Data Request for exposure function API online charging.

Table 6.3.4.1: Supported fields in Charging Data Request message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/T/U/E |
| Session Identifier |  | ITE |
| Subscriber Identifier |  | ITE |
| NF Consumer Identification |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Sequence Number |  | ITE |
| Retransmission Indicator |  | --- |
| One-time Event |  | --E |
| One-time Event Type |  | --E |
| Notify URI |  | I-- |
| Triggers |  | ITE |
| Multiple Unit Usage |  | ITE |
| NEF API Charging Information |  |  |
| External Individual Identifier |  | ITE |
| External Individual Id List |  | ITE |
| External Group Identifier |  | ITE |
| Internal Group Identifier |  | ITE |
| API Direction |  | ITE |
| API Target Network Function |  | ITE |
| API Result Code |  | ITE |
| API Name |  | ITE |
| API Reference |  | ITE |
| API Content |  | ITE |

Table 6.3.4.2 illustrates the basic structure of the supported fields in the Charging Data Response for exposure function API converged charging.

Table 6.3.4.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Result |  | ITE |
| Invocation Sequence Number |  | ITE |
| Session Failover |  | ITE |
| Triggers |  | --- |
| Multiple Unit Information |  | I-E |

End of changes