3GPP TSG-SA5 Meeting #152	S5-238110

# Chicago, US, 13 - 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | draftCR | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.104 Predictions of maintenance management related PMs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no enabling data and outputs for PM predictions for maintanance management related use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added enabling data (PMs) for maintanance management use case. - The enabling data for other use cases have been expanded to list the PMs. - All the use cases have been covered, hence the note that was added below the requirements have been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No enabling data and output available for PM predictions for maintanance management use cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x.3, 8.4.x.1.2, 8.4.x.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-238110 is a revision of S5-237628 |  |  |  |  |  |  |  |  |

Start of First change

### 7.2.x	Prediction and statistics of Management data

##### 7.2.x.1	Description

This use case is for the analytics (predictions and statistics) of the management data including 5G PMs defined in TS 28.552 [4] and 5G KPIs defined in TS 28.554 [5].

##### 7.2.x.2	Use case

Certain scenarios might need a prediction or statistics on existing management data, e.g. prediction or statistics of single or multiple PMs and KPIs. The MnS consumer of this use case (who may also be a MnS producer) might be interested in the analytics (statistics or prediction) on existing management data (PMs and KPIs) which can be then consumed to produce services on other use cases such as mobility optimization or load balancing etc. The MnS consumer gets a specific type of prediction or statistics (e.g., mean, standard deviation, correlation, etc.)  of specific management data (PMs/KPIs) on network objects.

Statistics of the management data include the statistical calculation on the data themselves over a period in the past and/or statistics of management data following a sliding window procedure to produce statistics. Predictions of management data may include predictions of the data themselves over a future period, and/or predictions of the data following a sliding window procedure.

The main objective of this use case is to provide analytics (statistics and/or predictions) on the existing management data.

##### 7.2.x.3	Requirements

Table 7.2.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SPM_MDA-01 | MDA capability for management data analytics shall be able to produce predictions for the requested management data. | Prediction and statistics of Management data |
| REQ-SPM_MDA-02 | MDA capability for management data analytics shall be able to produce statistics for the requested management data. | Prediction and statistics of Management data |

Start of next change

### 8.4.x	Predictions of Management data

#### 8.4.x.1	MDA assisted PM predictions

##### 8.4.x.1.1	MDA type

The MDA type for predictions of management data is: Predictions.PMData.

###### 8.4.x.1.2	Enabling data

###### 8.4.x.1.2.1	Mobility management performance related predictions

The enabling data for mobility management related performance measurements are provided in table 8.4.x.1.2.1-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.1-1: Enabling data for mobility management related PMs

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | Handover related performance measurements | Inter-gNB handovers (clause 5.1.1.6.1 of TS 28.552 [4]). Inter-gNB handovers (clause 5.1.1.6.4 of TS 28.552 [4]). Inter-gNB handovers (clause 5.1.1.6.2 of TS 28.552 [4]). Inter-gNB handovers (clause 5.1.1.6.3 of TS 28.552 [4]). Inter-gNB handovers (clause 5.1.1.6.6 of TS 28.552 [4]). Inter-gNB handovers (clause 5.1.1.6.7 of TS 28.552 [4]). Virtualised resource usage measurement (clause 6.2 of TS 28.552 [4]) Physical radio resource utilization of the target gNB, see clause 5.1.1.2 of TS 28.552 [4]; Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]. |
| MDT reports | UE measurements related to RSRP, RSRQ, SINR (serving cell and neighbour cells) and UE location information | RSRPs, RSRQs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |

###### 8.4.x.1.2.2	Coverage related predictions

The enabling data for coverage related performance measurements are provided in the table 8.4.x.1.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.2-1: Enabling data for coverage analytics related PMs

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | Coverage related performance measurements | SS-RSRP distribution per SSB (clause 5.1.1.22.1 of TS 28.552 [4]). SS-RSRP distribution per SSB of neighbor NR cell (clause 5.1.1.22.2 of TS 28.552 [4]) RSRP distribution per neighbor E UTRAN cell (clause 5.1.1.22.3 of TS 28.552 [4]) Type 1 power headroom distribution (clause 5.1.1.26.1 of TS 28.552 [4]). Wideband CQI distribution (clause 5.1.1.11.1 of TS 28.552 [4]). Timing Advance distribution for NR Cell (clause 5.1.1.33.1 of TS 28.552 [4]) Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). RSRP related measurements (clause 6.1 of TS 32.425 [12]). UE power headroom related measurements (clause 6.3 of TS 32.425 [12]). Wideband CQI distribution (clause 4.10.1.1 of TS 32.425 [12]). Average sub-band CQI (clause 4.10.1.2 of TS 32.425 [12]). UE Rx - Tx time difference related measurements (clause 6.4 of TS 32.425 [12]). AOA related measurements (clause 6.5 of TS 32.425 [12]). Timing Advance Distribution (clause 4.10.2 of TS 32.425 [12]). Number of UE CONTEXT Release Request initiated by eNodeB/RN (clause 4.1.5.1 of TS 32.425 [12]). |
| MDT reports | UE measurements related to RSRP, RSRQ, SINR (serving cell and neighbour cells) and UE location information | RSRPs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |

###### 8.4.x.1.2.3	SLS related predictions

The enabling data for SLS related performance measurements are provided in the table 8.4.x.1.2.3-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.3-1: Enabling data for SLS related PMs

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | SLS related performance measurements | RAN UE Throughput (clause 6.3.6 in TS 28.554 [5]). Mean number of PDU sessions of network and network Slice Instance (clause 6.4.1 in TS 28.554 [5]). Mean registered subscribers of network and network slice through AMF (see clause 6.2.1 in TS 28.554 [5]). Maximum packet size for a network slice subnet (see clause 6.3.11 of TS 28.541 [5]). |
| QoE data | The QoE data of the different services | QoE data (TS 26.247 [22] and TS 26.114 [23] can be acquired through the procedures defined in TS 28.405 [8]). |

###### 8.4.x.1.2.4	Energy Saving related predictions

The enabling data for energy saving related performance measurements are provided in the table 8.4.x.1.2.4-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.4-1: Enabling data for Energy Saving related PMs

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | Energy saving related performance measurements | PNF Power Consumption: (Clause 5.1.1.19.2 of TS 28.552 [4].) PNF Energy consumption (Clause 5.1.1.19.3 of TS 28.552 [4].) SS-RSRP distribution per SSB (beam) of serving NR cell (Clause 5.1.1.22.1 of TS 28.552 [4].) SS-RSRP distribution per SSB (beam) of neighbor NR cell (Clause 5.1.1.22.1 of TS 28.552 [4].) PDCP Data Volume of NR cells (Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]) Traffic load variation (Clause 5.1.1.2 and 5.1.1.4 of TS 28.552 [4].) UE throughput (Clause 5.1.1.3 of TS 28.552 [4].) Delay related measurements of UPF (Clause 5.4 of TS 28.552 [4].) Data volume of UPF (Clause 5.4 of TS 28.552 [4].) Virtual resource usage of NF(Clause 5.7.1 of TS 28.552 [4].) |
| QoE data | The QoE data of the different services | The measurements that are collected are DASH and MTSI measurements (TS 28.406 [9]). |

###### 8.4.x.1.2.5	Critical Maintenance management related predictions

The enabling data for critical maintenance management related performance measurements are provided in the table 8.4.x.1.2.5-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.5-1: Enabling data for Critical Maintenance management related PMs

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | Energy saving related performance measurements | Mean number of DRBs being allocated (clause 5.1.1.10.9 of TS 28.552 [4]). Number of requested preparations for handovers from 5GS to EPS (clause 5.1.1.6.3.1 of TS 28.552 [4]). Number of requested resource allocations for handovers from EPS to 5GS (clause 5.1.1.6.3.4 of TS 28.552 [4]) Number of requested preparations for EPS fallback handovers (clause 5.1.1.6.3.10 of TS 28.552 [4]) Number of successful executions for EPS fallback handovers (clause 5.1.1.6.3.13 of TS 28.552 [4]) Number of QoS flows attempted to modify (clause 5.1.1.13.4.1 of TS 28.552 [4]) |

End of Changes

##### 8.4.x.1.3	Analytics output

The specific information elements of the analytics output for predictions, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.3-1.

Table 8.4.x.1.3-1: Analytics output for predictions

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| pmPredictions | This information element defines the predicted values for the below performance measurements. - Mobility related performance measurements listed in table 8.4.5.1.2-1 and 8.4.5.2.2-1 in the current document. - Coverage related performance measurements listed in table 8.4.1.1.2-1 and 8.4.1.2.2-1 in the current document. - SLS related performance measurements listed in table 8.4.2.1.2-1, 8.4.2.2.2-1, 8.4.2.3.2-1, 8.4.2.4.2-1 and 8.4.2.5.2-1 in the current document. - Energy saving related performance measurements listed in table 8.4.4.1.2-1 in the current document. - Maintenance management related performance measurements listed in table 8.4.6.1.2-1 in the current document. | M | type: PmPrediction multiplicity: 1..* isOrdered: False isUnique: True defaultValue: none isNullable: False |

End of Changes