3GPP TSG-SA5 Meeting #152	S5-238111

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS 28.105 Use case and requirements for ML Inference activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NTT DOCOMO, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML inference activation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML inference activation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-237499 |  |  |  |  |  |  |  |  |

Start of First change

### 6.5.4	AI/ML inference function configuration management

#### 6.5.4.1	Description

The AI/ML inference function and the associated ML entity may need to be managed and configured to conduct inference in the 5G system to align with the consumer´s expectation, e.g., to enable the AI/ML inference function to perform inference.

The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML inference function.

#### 6.5.4.2	Use cases

##### 6.5.4.2.1	Managing AI/ML-based distributed Network Energy Saving

An AI/ML-based Distributed Network Energy Saving function may use one or more ML entities to derive energy saving recommendations.

This function needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Network Energy Saving function. The activation and deactivation actions for AI/ML-based Distributed Network Energy Saving conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

##### 6.5.4.2.2	Managing AI/ML-based distributed Mobility Optimization

An AI/ML-based Distributed Mobility Optimization function may use one or more ML entities to derive handover recommendations.

This function needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Mobility Optimization function. The activation and deactivation actions for AI/ML-based Distributed Mobility Optimization conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

##### 6.5.4.2.3	Managing AI/ML-based distributed Load balancing

An AI/ML-based Distributed Load balancing function may use one or more ML entities to derive load balancing recommendations.

This function needs to be managed. The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML-based Distributed Load balancing function. The activation and deactivation actions for AI/ML-based Distributed Load balancing conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF-01 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to request to manage the Energy saving inference function | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- AI/ML_INF-02 | The MnS producer of AI/ML-based Distributed Mobility Optimization should enable an authorized MnS consumer to request to manage the Mobility Optimization inference function | Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) |
| REQ- AI/ML_INF-03 | The MnS producer of AI/ML-based Distributed Load balancing should enable an authorized MnS consumer to request to manage the Load balancing inference function | Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ-AIML_ INF_ACT-1 | The MnS producer for AI/ML inference management should have a capability allowing an authorized MnS consumer to activate and deactivate an ML inference function. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ-AIML_ INF_ACT-2 | The MnS producer for AI/ML inference management should have a capability to allow an authorized MnS consumer to provide the policy for activating and deactivating inference function. Note: The policies instructing the ML MnS producer on how or/and when to activate which ML capabilities. | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |

End of Changes