3GPP TSG-SA5 Meeting #152	S5-238158

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0499 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add new measurement on Average DL GTP packet delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | P.I. WORKS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 28.552 section 5.4.7.1 already defines QoS Monitoring based GTP measurements related with UL packet delay between NG-RAN and PSA UPF. There is no measurement defined for average DL packet delay between NG-RAN and PSA UPF at PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new measurement for evaluating average DL packet delay between NG-RAN and PSA UPF by PSA UPF using QoS Monitoring based GTP measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator cannot monitor average DL packet delay between NG-RAN and PSA UPF by PSA UPF using QoS Monitoring based GTP measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.7.x (new), A.61 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on CR0500. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modified section

#### 5.4.7.x	DL packet delay between NG-RAN and PSA UPF

##### 5.4.7.x.y	Average DL GTP packet delay between PSA UPF and NG-RAN

a)	This measurement provides the average DL GTP packet delay between PSA UPF and NG-RAN. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF samples the GTP packets for QoS monitoring based on the policy provided by OAM or SMF.

NOTE:  The sampling rate may vary for different S-NSSAI and different 5QIs, and the specific sampling rate is up to implementation unless given by the QoS monitoring policy.

For each GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information (see 23.501 [4] and 38.415 [31]):

-	T1 received in the GTP-U header of the monitoring response packet in the “DL Sending Time Stamp Repeated” part of UL PDU Session Information (PDU Type 1) Format as defined in 38.415 [31] indicating the local time that the DL GTP PDU was sent by the PSA UPF;

-	T2 received in the GTP-U header of the monitoring response packet in the “DL Received Time Stamp” part of UL PDU Session Information (PDU Type 1) Format as defined in 38.415 [31] indicating the local time that the DL GTP PDU was received by the NG-RAN;

-	The 5QI and S-NSSAI associated to the GTP PDU.

The PSA UPF counts the number (N) of GTP PDU monitoring response packets for each 5QI and each S-NSSAI respectively, and takes the following calculation for each 5QI and each S-NSSAI:

i=1N(T2i-T1i)N

d)	Each measurement is a real representing the average delay in microseconds.

e)	GTP.DelayDlPsaUpfNgranMean.5QI, where 5QI identifies the 5QI; 
GTP.DelayDlPsaUpfNgranMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

Second modified section

# A.61	Monitoring of one way delay between PSA UPF and NG-RAN

The DL and UL one way delay has direct impact to users' experience for some types of services (e.g., URLLC). The one way delay between PSA UPF and NG-RAN is part of the end to end one-way delay and is not expected very long comparing to the delay in between NG-RAN and UE.

In case the PSA UPF and NG-RAN are time synchronised, the UL or DL one way delay can be measured by PSA UPF and the DL one way delay can be measured by NG-RAN.

The measurements on the one way DL and UL delay between PSA UPF and NG-RAN can be used to evaluate and optimize the DL and UL user plane delay performance between 5GC and NG-RAN.

End of modifications