3GPP TSG-SA5 Meeting #152	S5-238281

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	pCR 28.865 Add conclusion of issue#2 service assurance for video monitoring

# Document for:	Approval

Agenda Item:	6.9.2.4

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	S5-234729: "draft TR 28.865 Study on deterministic communication service assurance"; v0.5.0

3	Rationale

This tdoc addresses the conclusion of issue#2 service assurance for video monitoring and provide proposals for the normative work.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

5.2	Issue #2: Service assurance for video monitoring

5.2.1	Description

In TS 22.104, some UCs are provided, e.g. periodic communication for video-operated remote control, Real-time streaming data transmission (video data) from a mobile robot to the guidance control system. For video operated remote control services, the video backhaul and remote control are performed together.

The use case of video monitoring is a large uplink URLLC service. This type of service features large uplink data packets, high bandwidth, stability of bandwidth and latency. Typical scenarios include remote control of gantry cranes or bridge cranes in smart ports and remote surgery in smart medical care. The service requirements modeling is studied from the frame-level.

The three-layer model of service experience, service quality and network performance for video monitoring is depicted in figure 3.

Figure 3: Service requirement modelling of video monitoring service

Service experience layer: establishes service experience related indicators for video monitoring based on subjective and objective evaluation. Analyzes frame-level features and provides experience-layer KQIs.

Service quality layer: studies the relation between KQIs and QoS indicators. The impacts on the guaranteed bandwidth, frame skipping rate and the assumed packet loss rate on the equivalent frame impairment rate are uniformly distributed. It describes the impacts of indicators such as delay jitter, rate distribution, and burst packet loss ratio on the frame arrival time distribution in different scenarios.

Network capability layer: studies the relation between QoS and air interface bandwidth/delay, radio network performance measurements, and core network bandwidth/delay requirements, core network performance measurements and capacity etc.

The above information is used as the assurance objective of the video backhaul service. The deterministic communication service assurance should support network preparation based on the assurance objective. The DCSA MnS producer collects related data, analyzes service experience and network performance, optimizes and verifies deterministic communication service experience, and achieving the SLA assurance objective through the above steps. The DCSA MnS producer may need to collaborate with other related management services, such as performance management, fault management, MDA, closed loop control, and provisioning.

DCSA-REQ-VideoMon 1: 3GPP management system should provide service assurance capabilities for video monitoring services.

DCSA-REQ-VideoMon 2: 3GPP management system should provide capabilities to enable consumers to monitor the service requirements fulfilment status and the corresponding analysis information for video monitoring service

5.2.2	Potential solutions

5.2.2.1	Potential solution #1: Video monitoring service

5.2.2.1.1	Introduction

In the deterministic communication service requirement analysis phase, network requirements (data rate, delay, and reliability) are analyzed. Network deployment requirements of different areas need to be considered for network preparation. Deterministic communication services have high SLS requirements. For example, the planning of video monitoring services focus on uplink coverage, uplink capacity, uplink data rate, and E2E latency etc. It demands on high delay stability of video frame transmission. The network requirements need to be derived based on the application characteristics of specific services. For example, network preparation information, such as network capacity, coverage, reliability, and data rate estimation, needs to be output based on the three-layer service requirement model of the video monitoring service.

5.2.2.1.2	Description

1. Data collection

Collect service experience data and network performance data of video monitoring services based on the three-layer service requirement modeling. For example, the following data may be collected: latency and throughput related performance data defined in TS 28.552 [7] and TS 28.554 [8]. Abnormal information such as large round-trip delay, large jitter, low data rate distribution, high packet loss rate, and service quality deterioration etc.

Taking the video monitoring services in a large port as an example, it requires high uplink bandwidth, low latency, high availability, mobility, and high reliability. Network preparation is performed to meet these requirements, e.g., deployment and provisioning of the necessary network functions.

2. Analytics and demarcation

Service and network analysis of deterministic communication services includes monitoring, poor-QoE identification, demarcation, and root cause analysis. Identify abnormal issues, determine the impact scope, and restore services.

Monitoring and the poor-QoE identification requires two steps:

- Builds the three-layer service requirement model based on service characteristics, including poor experience information.

- Collects experience, service quality, and network-related indicators and abnormal incidents to identify poor-QoE users and services.

Fault demarcation is used to generate service interruption/ poor experience incident when a service anomaly occurs. The E2E network KPI, alarm information, terminal exceptions are correlated and analyzed. Demarcation results for video monitoring services are provided accordingly.

For deterministic communication services, the stability and deadline of latency are defined at data packet level from end to end perspective according to TS 22.104 [3]. The service characteristics and requirements regarding service experience, service quality and network performance quality for video monitoring service are described in clause 5.2.1. The monitoring of the service requirements fulfilment status and the corresponding MDA analysis information for video monitoring service should be provided from management perspective. The abnormal issues should be identified and means should be taken to solve the problems. To support the assurance of video monitoring services, some abnormal issues and some new MDA analysis outputs are as in the following table:

Table D: MDA analytics output of abnormal issues which may violate the service requirements

| Attribute Name | Description |
| --- | --- |
| Communication service interruption | The reception failure of an uplink data packet upon the deadline of the survival time. It is used to identify latency issues that may exceed survival time. |
| Excessive RTT | The times that the RTT is greater than a threshold exceed a predefined value within a detection period. It is used to identify end to end latency issues. |
| High packet loss rate | The uplink packet loss rate is greater than a threshold within a detection period. It is used to identify potential packet level reliability issues. |
| Abnormal data rate | The uplink traffic is greater than a traffic threshold and the number of consecutive occurrence periods reaches the threshold within a detection period. It is used to identify issues of guaranteed data rate that do not meet the requirements. |

Some performance measurements are currently available in TS 28.552 [7], e.g. packet drop rate, average packet delay, IP latency measurements, UE throughput etc, which may be used as the enabling data for the above analysis.

3. Optimization and verification

Optimization:

For video monitoring services, delay-related optimization and uplink data rate optimization are considered. To optimize delay-related problems, it is needed to analyze the average delay, delay jitter, and delay reliability. It can analyzed the delay-related factors of the air interface, such as the uplink and downlink slot assignment proportion, air interface resource capacity, and scheduling policy.

Uplink data rate optimization: The coverage, interference, resource allocation policy, and scheduling policy are optimized to make the uplink rate better match the SLS requirement.

Verification:

According to the service assurance objective of deterministic communication, that is, the quality of experience indicator of each service meets the deterministic requirement after optimization, it is further verified that the network capability meets the deterministic requirement. Select indicators that reflect video monitoring service experience, such as stalling and interruption, or indicators that reflect network performance, such as packet loss rate, rate, and delay.

5.2.3	Conclusion - Impact on normative work

In TS 23.501 (System architecture for the 5G System), the QoS model of Mission Critical Video and "Live" Uplink Streaming were defined. The video monitoring service is a kind of uplink URLLC service which is uplink bandwidth consuming and latency sensitive when used for video assisted mission critical operations.

- The management of general aspects for the uplink focused URLLC service requirements should be considered for the normative work: new requirements for the deterministic aspects for the uplink focused URLLC service: latency of operation response time, uplink guaranteed data rate, distribution of data packet arrival time (or transfer interval) etc, e.g. the impacts on serviceProfile, sliceProfile, and the corresponding performance measurements reporting.

- To support this uplink focused URLLC service, impacts on MDA capability and the analytics output in table D are suggested to be further defined.

End of change