3GPP TSG-SA5 Meeting #152 	S5-238299

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0193 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify verbs in attribute definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the attribute table the attribute expectationVerb is described and examples are provided. The actual verbs are given as examples are not normatively defined and should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the allowed values for expectation verbs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-238299 is revision of S5-237727 |  |  |  |  |  |  |  |  |

Start of 1st Change

#### 6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). The intentExpectations are arranged in an ordered list with order such that the most important intentExpectations are on the top of the list. allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific noted notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the performance targets values allowedValues: DELIVER, ENSURE vendor extensions are allowed | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that are required to be applied on. It can be class name of the managed object. allowedValues: see scenario specific IntentExpectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget. The expectionTargets are arranged in an ordered list with order such that the most important expectionTargets are on the top of the list. | type: ExpectationTarget multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", " IS_EQUAL_TO_OR_LESS_THAN”, "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF", "IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single value when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of values when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: ValueRangeType multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS_EQUAL_TO_OR LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", "IS_NOT_ONE_OF","IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextCondition The value will be a single real number when the contextCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN". The value will be a pair of real numbers when the contextCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the contextCondition is "IS_ONE_OF", "IS_NOT_ONE_OF","IS_ALL_OF". See NOTE 1. | type: Real multiplicity: 1..* isOrdered: False isUnique: True defaultValue: Null isNullable: True |
| intentPriority | It expresses the priority of the stated intent. AllowedValue: values in the range [1-100] where 1 indicates the highest priority and 100 indicates the lowest priority. Note: The handing of the priorities across consumer is left to implementation | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 1 isNullable: False |
| geoArea | It describes a geographical area defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pLMNId | It describes the information of a PLMN identification defined in 3GPP 28.658[10] AllowedValue: As defined by the data type | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dateTime | It describes the information of a date time defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeWindow | It describes the information of a time window (including startTime, endTime) defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| geoCoordinate | It describes the information of a geoCoordinate defined in 3GPP TS 28.622[6]. AllowedValue: As defined by the data type | type: GeoCoordinate multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intentAdminState | It describes the intent administrative state, which enables the MnS consumer to suspend an intent or cancel the suspension for a suspended intent. A suspended intent means this intent is not considered for fulfilment allowedValues: "ACTIVATED", "DEACTIVATED" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "ACTIVATED" isNullable: False |
| intentReference | It indicates the associated intent instance allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentReportReference | It indicates the associated intent report instance(s) allowedValues: Not Applicable | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| observationPeriod | It represents the observation period of the fulfilmentInfo for corresponding ExpectationTargets, IntentExpectations and Intent. The observation period can be assigned by MnS consumer through requesting the MnS producer to set attribute "observationPeriod". MnS producer also can assign the observation period if MnS consumer didn’t assign it. The observation time is expressed in seconds. allowedValues: Not Applicable | type: Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentConflictReports | It describes the conflict information which is reported for associated intent instance if needed. allowedValues: Not Applicable | type: IntentConflictReport multiplicity: * isOrdered: False isUnique: TRUE defaultValue: None isNullable: False |
| conflictType | It describes the type of intent conflict. allowedValues: INTENT_CONFLICT, EXPECTATION_CONFLICT, TARGET_CONFLICT | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingIntent | It describes the DN of the conflicting intent allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingExpectation | It describes the expectationId of the conflicting IntentExpectation with an Intent allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| conflictingTarget | It describes the targetName of the conflicting ExpectationTarget with an IntentExpectation allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedSolutions | It describes the recommendedSolutions to resolve intent conflict. allowedValues: the concrete enum value for recommendedSolution is FFS. | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationFulfilmentResults | It includes the expectationFulfilmentInfo and targetFulfilmetResults for each IntentExpectation. The expectationFulfimentInfo describes status of fulfilment of an intentExpectation and the related reasons for infeasible status. allowedValues: Not Applicable | type: ExpectationFulfilmentResult multiplicity:1.. * isOrdered: False isUnique: TRUE defaultValue: None isNullable: False |
| targetFulfilmentResults | It includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget. The targetFulfilmentInfo describes status of fulfilment of an expectationTarget and the related reasons for infeasible status. The targetAchieveValue describes current performance value for the ExpectationTarget. allowedValues: Not Applicable | type: TargetFulfilmentResult multiplicity: 1..* isOrdered: False isUnique: TRUE defaultValue: None isNullable: False |
| targetAchievedValue | It describes the value that has been achieved for the expectation target at the time at which the report is generated. allowedValues: Not Applicable | type: Number multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFeasibilityCheckReport | It describes the intent feasibility check information which is reported if needed. allowedValues: Not Applicable | type: IntentFeasibilityCheckReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| feasibilityCheckResult | It describes the result of intent fulfilment feasibility check allowedValues: FEASIBLE, INFEASIBLE | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| infeasibilityReasons | It describes the reason (e.g. invalid intent expression, the intent conflict) of the result of intent fulfilment feasibility check is INFEASIBLE Editor’s Note: the ENUM value for infeasibilityReason is FFS. | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentHandlingCapabilityList | It describes the list of expectation object information and expectation target information which can be supported by intent handling function. allowedValues: Not Applicable | type: IntentHandlingCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentHandlingCapabilityId | A unique identifier of property of intent handling capability should be supported by the intent handling function of MnS producer. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationObjectType | It describes the expectation object type which can be supported by a specific intent handling function of MnS producer. allowedValues: objectType defined in clause 6.2.1.3.2. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedExpectationTargetNames | It describes the supported expectation targets for the supported expectation object type. allowedValues: targetName defined in clause 6.2.1.3.3 | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| lastUpdatedTime | It describes the time for the latest update of the IntentReport Instance | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Note 1: For "IS_ALL_OF", the value shall be a match of the entire list. |  |  |

End of Changes