3GPP TSG-SA5 Meeting #154	S5-241816

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0563 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Measurements family name update as per clause 3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Aligning measurement names as per family name in clause 3.3 for NSOEU |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement family name is updated in three measurements of NSOEU |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect measurements names exist in specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.1.4	Performance measurements for NSOEU

#### 5.1.4.1	Measurements related to cell service status

##### 5.1.4.1.1	Total cell In-Service duration

a)	This measurement provides the total time duration for which an active cell in a gNB-DU remains In-Service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “In-Service” it means the active cell is operational, and it is able to serve UEs. The active cells are considered as In-Service until the gNB-DU reports the Service Status as “Out-Of-Service” to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE messages.

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tin and Tout. Where Tin is the time stamp when the Service Status is reported as “In-Service” for a cell by gNB-DU to gNB-CU and Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for the same cell in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total in-service duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod as defined in TS 28.622 [13] clause 4.3.16.

d)	Each measurement is an integer value representing the number of minutes for which the cell was in-Service.

e)	OEU.CellInServiceTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.4.1.2	Total cell outage duration

a)	This measurement provides the total time duration for which a cell in a gNB remains out of service. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “Out-Of-Service” it means the active cell is not operational, and it is not able to serve UEs. The active cells are considered as Out-of-Service until the gNB-DU reports the Service Status as In-Service to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE message

b)	CC.

c)	This measurement is obtained in two steps. First step involves determining the time interval between the time stamps Tout and Tin. Where Tout is the time stamp when Service Status is reported as “Out-Of-Service” by gNB-DU to gNB-CU for a cell and Tin is the time stamp when the Service Status is reported as “In-Service” for the same cell by gNB-DU to gNB-CU in the GNB DU CONFIGURATION UPDATE messages. Second step involves summing up all these time intervals of a cell which fall within the desired observation time frame Tobv. This gives total outage duration of a cell within the observation time frame Tobv. Where, Tobv is monitorGranularityPeriod as defined in TS 28.622 [13] clause 4.3.16.

d)	Each measurement is an integer value. It represents the number of seconds for which the cell was Out-Of-Service

e)	OEU.CellOOSTimeTotal.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.4.1.3	Average cell outage duration

a)	This measurement provides the average time duration of an outage i.e. average time for which a cell in a gNB-DU remains out of service per outage. The gNB-DU reports the Service Status of the Active cells to the gNB-CU. If the Service Status value is “Out-Of-Service” it means the active cell is not operational, and it is not able to serve UEs. The active cells are considered as Out-of-Service until the gNB-DU reports the Service Status as In-Service to the gNB-CU. The gNB-DU reports the Service Status to gNB-CU using the GNB DU CONFIGURATION UPDATE message.

b)	CC.

c)	This measurement is obtained by dividing the measurement OEU.CellOOSTimeTotal.NCGI (clause 5.1.4.1.2) with Noutage where Noutage is the number of occurrences of outage for that cell within the desired observation time frame Tobv where, Tobv is monitorGranularityPeriod as defined in clause 4.3.16 of TS 28.622 [13].This gives an average duration per outage of a cell.

d)	Each measurement is an integer value representing the average number of seconds for which the cell was Out-Of-Service per outage occurrence.

e)	OEU.CellOOSTimeAverage.NCGI, where NCGI is NR Cell Global Identifier as defined in TS 28.541 [16]. The number of measurements is equal to the number of NCGIs (cells).

f)	GNBCUCPFunction.

g)	Valid for packet switching.

h)	5GS.

End of Change