3GPP TSG-SA5 Meeting #159	S5-25xxxx

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:                         Rakuten Mobile

# Title:	pCR TR 28.869 Annex example for CRD based deployment

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.869 v1.1.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add an example to show CRD-based deployment of 3GPP NF

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.869 [1].

1st Change

Annex <X>: Support for LCM of NF deployments using Kubernetes Custom Resource Definitions

Figure X-1 depicts an example scenario for the interaction between the 3GPP management system and the orchestration and management entity for the LCM of a NF deployment using native Kubernetes® Custom Resource Definition.

An authorized NF LCM client requests the deployment of a 3GPP NF by creating the Kubernetes® object based on the CRD definition. A Kubernetes NF operator is a software entity residing in a K8S cluster which monitors the CRD database in the control plane for changes. It understands the requirements of the CRD and converts them into corresponding native K8S objects.

Figure X-1: Example scenario for interaction with orchestration and management entity for 3GPP NF creation procedure

The NF LCM client invokes interactions with a Kubernetes control plane by using a Custom Resource Definition.  When a K8S CRD object is created in the corresponding control plane, notification updates are sent to the operator. Based on the CRD payload, the operator creates the relevant objects to realize the 3GPP NF in the Kubernetes worker nodes. Following the completion of the required interactions with the Kubernetes cluster, the NF operator updates the status of the CRD to reflect the operational state of the 3GPP NF. Upon receiving any further CRD updates, the cycle repeats resulting in the modification of the 3GPP NF workloads. To terminate the NF, the CRD is deleted by the LCM client.

End of Changes