3GPP TSG-SA5 Meeting #159 	S5-250323

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR Add use cases and requirements on LCM for Reinforcement Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions based on the state of, actions taken in, and rewards from the RL environment. RL can be used in 5G for various AI/ML supported functions, e.g., Energy Saving, Mobility Load Balancing and Mobility Robustness Optimization. This contribution is to add the use cases and requirements for managing the RL in 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added use cases and requirements on management of Reinforcement Learning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2b.2.x (new), 6.2b.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modified section

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: a manageable representation of an ML model algorithm.

NOTE 1:	an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

NOTE 2:	ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

NOTE 3:	ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

ML model initial training: a process of training an initial version of an ML model.

ML model re-training: a process of training a previously trained version of an ML model and generate a new version.

NOTE 4:	a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

ML model joint training: a process of training a group of ML models.

ML training function: a logical function with ML model training capabilities.

ML model testing: a process of evaluating the performance of an ML model using testing data different from data used for model training and validation.

ML model joint testing: a process of evaluating the performance of a group of ML models using testing data different from data used for model training and validation.

ML testing function: a logical function with ML model testing capabilities.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

ML model deployment: a process of making a trained ML model available for use in the target environment.

ML model loading: a process of making a trained ML model available to an inference function.

AI/ML activation: a process of enabling the inference capability of an AI/ML inference function.

AI/ML deactivation: a process of disabling the inference capability of an AI/ML inference function.

Reinforcement Learning: a machine learning approach in which an RL agent learns to make or refine decisions through trial and error by interacting with an RL environment, observing its states, taking actions, and receiving rewards.

Next modified section

#### 6.2b.2.x	Management of Reinforcement Learning

Reinforcement Learning (RL) is a type of machine learning in which an agent learns to make decisions by interacting with an environment. The agent observes the current state of the environment, takes actions, and receives rewards based on those actions. Through a trial-and-error approach, the agent aims to develop a policy that maximises its cumulative reward over time.

RL can be used in 5G system for various AI/ML supported functions, e.g., Energy Saving (ES), Mobility Load Balancing (MLB) and Mobility Robustness Optimization (MRO).

An RL agent functionally consists of two main parts: a training component and an inference component.

The training component learns to generate or update the RL model from the outcomes of its actions in an RL environment (e.g., a live 5G subnetwork or a simulation network). Training can occur in one of two ways:

-	Online RL: in Online RL, the RL model is trained and applied in real time through direct interaction between the RL agent components and the RL environment. In this approach, the RL agent’s training component continuously learns by receiving rewards, and observing state transitions resulting from its actions, while the RL agent’s inference component uses the trained model in real time to determine actions in the RL environment.

Figure 6.2b.2.x-1: Online RL for 5G system

-	Offline RL: Instead of direct interaction, the training process relies on a pre-collected dataset from a data collection entity. This dataset consists of state transition, action, and reward information collected over a period of time from the RL environment, allowing the model to learn without real-time interaction with the environment.

Figure 6.2b.2.x-2: Offline RL for 5G system

Once the RL model is trained and deployed (if training and inference occur on separate entities), the inference component adopts the RL model and makes decisions based on state transitions in the RL environment.

To enable, facilitate, and manage RL for 5GS, the 3GPP management system needs to support the following RL components in performing their respective functions:

-	RL agent training component: collects the data for training, train the RL model and report the training result. The data for training includes state transitions, rewards from the RL environment for online RL, or the pre-processed data set for a period of time from the data collection entity for offline RL, as well as the actions taken by the RL agent inference component. The trained RL model would be used to determine actions to be taken for observed state conditions. Additionally, the RL agent training component needs to indicate the RL environment(s) (e.g., a live subnetwork or a simulation subnetwork) for which an RL model has been trained.

-	RL agent inference component: deploys and activates the trained RL model (RL policy), receives state transitions from the RL environment, and determine actions based on the RL model. The RL agent inference component needs to be configured with the information about the RL environment(s) where a trained RL model can be used.

-	RL environment: executes the actions determined by the RL agent inference component and reports rewards to the RL agent training component for online RL or to the data collection entity for offline RL.

Next modified section

### 6.2b.3	Requirements for ML model training

Table 6.2b.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to request ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-02 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-03 | The ML training MnS producer shall have a capability allowing the authorized ML training MnS consumer to specify the AI/ML inference name of the ML model to be trained. | ML model training requested by consumer (clause 6.2b.2.1) |
| REQ- ML_TRAIN-FUN-04 | The ML training MnS producer shall have a capability to provide the training result to the ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-05 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML model. (See Note) | ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-06 | The ML training MnS producer shall have a capability to provide the version number of the ML model when it is generated by ML model re-training to the authorized ML training MnS consumer. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2) |
| REQ- ML_TRAIN-FUN-07 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML model training. | ML model training requested by consumer (clause 6.2b.2.1), ML model training initiated by producer (clause 6.2b.2.2), ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-08 | The ML training MnS producer should have a capability to provide the grouping of ML models to an authorized ML training MnS consumer to enable coordinated inference. | ML model joint training (clause 6.2b.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The ML training MnS producer should have a capability to allow an authorized ML training MnS consumer to request joint training of a group of ML models. | ML model joint training (clause 6.2b.2.6) |
| REQ- ML_TRAIN-FUN-10 | The ML training MnS producer should have a capability to jointly train a group of ML models and provide the training results to an authorized consumer. | ML model joint training (clause 6.2b.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to discover the properties of available ML models including the contexts under which each of the models were trained. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to select an ML model to be used for inference. | ML models and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to request for information and be informed about the available alternative ML models of differing complexity and performance. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_SELECT-04 | The 3GPP management system shall have a capability to provide a selected ML model to the authorized ML training MnS consumer. | ML model and ML model selection (clause 6.2b.2.3) |
| REQ-ML_TRAIN- MGT-01 | The ML training MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML model training, e.g. to modify the request or to delete the request. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model Training Processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-02 | The ML training MnS producer shall have a capability allowing an authorized ML training MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training. | ML model training requested by consumer (clause 6.2b.2.1), Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer (e.g. the function/model different from the function that generated a request for ML model training) to request for a report on the outcomes of a specific training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized ML training MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the ML training function to report to any authorized ML training MnS consumer about specific ML model training process and/or report about the outcomes of any such ML model training process. | Managing ML model training processes (clause 6.2b.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an ML training function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall have a capability to enable an authorized consumer to provide to the ML Training MnS producer, a training data quality score, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. | Handling errors in data and ML decisions (clause 6.2b.2.5) |
| REQ-ML_VLD-01 | The ML training MnS producer should have a capability to validate the ML models during the ML model training process and report the performance of the ML models on both the training data and validation data to the authorized consumer. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-ML_VLD-02 | The ML training MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML model training and validation process. | ML model validation performance reporting (clause 6.2b.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2b.2.8) |
| REQ-ML_TRAIN_PM-1 | The ML Training MnS producer should have a capability to allow an authorized consumer to get the capabilities about what kind of ML models the ML training function is able to train. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-2 | The ML Training MnS producer should have a capability to allow an authorized consumer to query what performance indicators are supported by the ML model training for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-3 | The ML Training MnS producer should have a capability to allow an authorized consumer to select the performance indicators from those supported by the ML training function for reporting the training performance for each ML model. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-ML_TRAIN_PM-4 | The ML Training MnS producer should have a capability to allow an authorized consumer to provide the performance requirements for the ML model training using the selected the performance indicators from those supported by the ML training function. | Performance indicator selection for ML model training (clause 6.2b.2.9.2) |
| REQ-RL_TRAIN_01 | The ML Training MnS producer for RL agent training component shall have a capability to report RL types (i.e., online RL, offline RL) to an authorized consumer. | Management of Reinforcement Learning (6.2b.2.x) |
| REQ-RL_TRAIN_02 | The ML Training MnS producer for RL agent training component shall have a capability to report the result of the trained RL model, which may contain the updated actions with the state conditions and/or newly explored actions for some state conditions. | Management of Reinforcement Learning (6.2b.2.x) |
| REQ-RL_TRAIN_03 | The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to get the information about the RL environment for which an RL model has been trained. | Management of Reinforcement Learning (6.2b.2.x) |
| REQ-RL_TRAIN_04 | The ML Training MnS producer for RL agent training component shall have a capability to allow an authorized consumer to provide information about the RL environment for which an RL model is to be trained. | Management of Reinforcement Learning (6.2b.2.x) |
| REQ-RL_TRAIN_05 | The ML Training MnS producer for RL agent training component shall have a capability to report the statistics of the rewards for the actions taken by an RL model. | Management of Reinforcement Learning (6.2b.2.x) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML model supports) of ML model. |  |  |

Next modified section

##### 6.5.1.2.1	AI/ML inference performance evaluation

During AI/ML inference, the AI/ML inference function (including e.g., MDAF, NWDAF or RAN functions) uses one or more ML models for inference to generate the AI/ML inference output. The performance of a running ML model may degrade over time due to changes in network state, which will affect the related network performance and service. Thus, it is necessary to evaluate performance of the ML model during the AI/ML inference process. If the inference output is executed, the network performance related to each AI/ML inference function also needs to be evaluated.

The AI/ML inference consumer (e.g., a Network or Management function) may take some actions according to the AI/ML inference output provided by the AI/ML inference function. If the actions are taken accordingly, the network performance is expected to be optimized. Each AI/ML inference function has its specific focus and will impact the network performance from different perspectives.

The AI/ML inference consumer may choose to not take any actions for various reasons, e.g., lacking confidence in the inference output, avoiding potential conflict with other actions or when no actions are needed or recommended at all according to the inference output.

For supporting the evaluation of the performance of the AI/ML inference function and ML model, the MnS consumer of ML inference performance management needs to be able to obtain the inference output generated by each AI/ML inference function, and the corresponding actions taken by each AI/ML supported function according to the inference output. The MnS consumer can then evaluate the performance based on the inference output, the actions taken, and related network measurements, and report the evaluation result to the MnS consumer. In case of RL, the RL agent inference component (see clause 6.2b.2.x), which is part of the AI/ML supported function, takes actions based on the decisions (inference output) produced according to the RL model for the observed state transitions.

Depending on the performance evaluation results, some actions (e.g., deactivate the running model, start re-training, replace the running ML model with a new one, etc) can be taken to avoid generating the inaccurate inference output.

To monitor the performance during AI/ML inference, the MnS producer responsible for AI/ML inference performance management can perform evaluation periodically. The performance evaluation period may be determined based on the network change speed. Besides, a consumer (e.g., an operator) may wish to control and manage the performance evaluation capability. For example, the operator may configure the performance evaluation period of a specified ML model.

End of modified sections