3GPP TSG-SA5 Meeting #159	S5-250340

Sophia-Antipolis, France, 16 - 21 February 2025

# Source:	China Telecom

# Title:	Rel-19 DP on Charging Aspects of 5GC LoCation Services

# Document for:	Discussion

Agenda Item:	7.2

1	Decision/action requested

The group is asked to discuss and endorse the proposal.

2	References

[1]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[3]	3GPP WID S5-241827: “Rel-19 New WID on Charging Aspects of Ranging and Sidelink Positioning”.

[4]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

3	Rationale

3.1 Background

The service description for 5GC Location Services (LCS) is documented by SA1 in TS 22.071 [1] with the following use cases in various applications:

1. Public Safety Services

5GC LCS plays a crucial role in enhancing emergency response systems. For instance, accurate real-time location information of a UE can assist emergency services in rapidly locating and responding to incidents such as accidents or natural disasters.

2. Location-Sensitive Charging

Certain applications, such as roaming or region-specific services, could implement location-sensitive charging. For example, users could be charged differently depending on their location—such as urban versus rural settings—to reflect service availability and infrastructure costs.

3. Tracking Services

5GC LCS supports real-time tracking of people and assets, improving fleet management, patient monitoring, and IoT device tracking. This helps organizations enhance operational efficiency and ensure safety in sectors like logistics and healthcare.

4. Traffic Monitoring

5GC LCS enables traffic monitoring services by aggregating anonymized location data from users. This data can assist to identify congestion and optimize traffic flow.

5. Enhanced Call Routing

Location-based routing ensures that calls are directed to the most appropriate resources based on a user’s geographical location. For instance, emergency calls can be routed to the nearest response center, reducing latency and improving outcomes.

6. Entertainment and Community Services

Location-based gaming, augmented reality (AR), and social applications rely on accurate and timely location data. For example, AR games require frequent updates of user location.

The Architecture of 5GC LCS and various Location Services Procedures are defined by SA2 in 3GPP TS 23.273 [2] with the architecture illustrated in Figure 1.

Figure 1: Non-roaming reference architecture for Location Services in reference point representation [2]

3.2 Considerations for Charging Aspects of 5GC LCS

Related work in SA5 has been conducted on the normative work of 5GS charging for Ranging and Sidelink Positioning [3], which shares similarities with 5GC LCS. However, Ranging and Sidelink Positioning represents only one scenario within 5GC LCS. It is necessary to specify charging solutions specifically for other 5GS LCS scenarios and the converged charging architecture whether can be reusable for 5GC LCS needs further investigation.

Figure 2: Ranging and Sidelink Positioning Converged charging architecture [4]

4	Detailed proposal

The group is asked to endorse the proposal in the rationale.