3GPP TSG-SA5 Meeting #159	S5-250345

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:                         China Mobile, NTT DOCOMO

# Title:	pCR Add Potential detailed solution for 3GPP management architecture evolution

# Document for:	Approval

Agenda Item:	6.19.6

1	Decision/action requested

For approval

2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

3	Rationale

The contribution proposes to add a potential detailed solution for 3GPP management architecture evolution.

4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

5.2	Use of industry solutions for management of cloud-native network functions

Editor's Note: This clause describes the use cases, issues, requirements, and solutions related to WT-2.

5.2.1	Use case #1: 3GPP management architecture evolution to support LCM of NF Deployment instance

5.2.1.1	Description

3GPP specifications specify the interaction with ETSI NFV MANO for the LCM of the virtualized part of NF (VNF). The requirements, use cases and procedures for the management of NF containing the virtualized part are specified in TS 28.531[7] for 5G network. The interfaces for the interaction with ETSI NFV MANO are provided via Os-Ma-nfvo and the Ve-Vnfm-em reference points specified in ETSI GS NFV-IFA013 [8] and ETSI GS NFV-IFA008 [9].

There are newly developed and evolved industry solutions for management and orchestration of cloud native applications that leverage industry standards, e.g. Kubernetes based solution. Such industry solutions including ETSI NFV MANO and non ETSI NFV MANO can be used to address challenges related to NF Deployment instances e.g. for hybrid cloud deployments that deploy cloud native applications with hyperscale cloud providers. This use case considers the scenarios where the 3GPP management architecture is flexible to support use of ETSI NFV MANO and Non ETSI NFV MANO for the LCM of NF Deployment instance.

5.2.1.2	Potential requirements

REQ _LCM-1 The deployment management reference point should support the capability allowing the 3GPP management system to interact with an orchestration and management entity to perform the LCM of NF Deployment instance.

5.2.1.3	Potential solutions

5.2.1.3.1	High level architecture evolution

Introduce a generic orchestration and management entity that interacts with 3GPP management system for LCM of NF Deployment instance via the new deployment management reference point.

The figure 5.2.1.3-1 below illustrates the interaction between 3GPP management system and orchestration and management entity in simplified view:

Figure 5.2.1.3-1: High level architecture evolution solution for lifecycle management of NF Deployment instance

The deployment management reference point includes the interfaces that 3GPP supports today with ETSI NFV MANO but not limited to it, e.g. Kubernetes based API.

This clause provides a high-level solution for 3GPP management architecture evolution. In order to facilitate the analysis of the impact of potential architecture evolution on the 3GPP management system, detailed potential solutions are presented in the following clauses.

5.2.1.3.X	Orchestration and management based on ETSI NFV-MANO

In this solution, the orchestration and management considers the latest version of NFV-MANO architecture specified in ETSI GS NFV 006[18] and the deployment management reference point includes the exchange of information over the interfaces that 3GPP supports with ETSI NFV-MANO (ETSI GS NFV-IFA 013 [8] and ETSI GS NFV-IFA 008 [9]).

ETSI NFV has developed since Release 4 a set of specifications that enable support for containerized deployments in bare metal and VMs using the NFV-MANO framework. Details are provided in Annex G. In Annex H details are provided regarding the operations performed in NFV-MANO upon an instantiation request for the case of NF Deployments in containers.

As ETSI NFV MANO architectural framework is evolving this Deployment management reference point is expected to be updated accordingly to accommodate latest NFV-MANO developments.

Figure 5.2.1.3-X: Potential architecture evolution based on ETSI NFV-MANO

To support this solution, TS28.531 and TS28.533 already references to Release 4 NFV-MANO specification work with support for containers, while additional investigations are needed, for example, updating the ManagedFunction IOC in TS 28.622 through enriching the vnfParametersList with additional attributes necessary for the case of NF Deployments.

Possible extensions:

- Adding additional parameters to identify components of the NF Deployment, such as VNFC, to understand the association with specific NF Deployment micro-services.

- Adding additional parameter to understand the instantiation state of the NF Deployment.

- Adding additional parameters to understand which specific deployable modules have been used for the instantiation (similarly to flavour).

- Updating the related 3GPP management system specifications  to consider ETSI NFV Release 4 or 5 specification work (e.g., TS 28.526, TS 28.527) to support containerized deployments using NFV-MANO.

2nd Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification".

[3]	ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV);Architectural Framework; Report on VNF generic OAM functions".

[4]	3GPP TR 28.834: "Technical Specification Group Services and System Aspects; Study on management of cloud-native Virtualized Network Functions (VNF)".

[5]	SP-230764 New WID on Management of cloud-native Virtualized Network Functions.

[6]	3GPP TS 28.526: "Technical Specification Group Services and System Aspects; Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[7]	3GPP TS 28.531: "Technical Specification Group Services and System Aspects; Management and orchestration; Provisioning".

[8]	ETSI GS NFV-IFA 013 (V5.2.1) (2024-12): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[9]	ETSI GS NFV-IFA 008 (V5.2.1) (2024-12): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

…

End of Changes