3GPP TSG-SA5 Meeting #159	S5-250428

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0073 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.313 Use case and requirements for renewable energy based LBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solve WT4 in objectives of WID Energy efficiency and energy saving aspects of 5G networks and services (SP-24140). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case and requirements for renewable energy based LBO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss the renewable energy based LBO feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.1.5, 6.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services"

[4]	3GPP TS 38.321 "NR; Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 28.552 "Management and orchestration; 5G performance measurements".

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements".

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2".

[8]	Void

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[10]	Void

[11]	3GPP TS 28.531 "Management and orchestration; Provisioning".

[12]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[13]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[14] 	Void

[15]	3GPP TS 32.508: "Telecommunication management; Procedure flows for multi-vendor plug-and-play eNodeB connection to the network".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	Void

[18]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[19]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[20]	3GPP TS 32.422: "Subscriber and equipment trace; Trace control and configuration management".

[21]	3GPP TS 28.315: "Management and orchestration; Plug and Connect; Procedure flows".

[22]	3GPP TS 28.314: " Management and orchestration; Plug and Connect; Concepts and requirements”.

[23]	3GPP TS 28.317 " Management and orchestration; Self-configuration of Radio Access Network Entities (RAN NEs)".

[x]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

Next Change

6.1.1.5	LBO (Load Balancing Optimisation)

REQ-DLBO-FUN-1 Provisioning MnS for D-LBO function should have a capability allowing an authorized consumer to set or update the ranges of HO and/or reselection parameters, and control parameters for LBO function.

REQ-DLBO-FUN-2 Performance assurance MnS for D-LBO function should have a capability allowing the authorized consumer to collect the LBO related performance measurements that are used to evaluate the LBO performance.

REQ-DLBO-FUN-3 Provisioning MnS for D-LBO function should have a capability to notify the authorized consumer about the LBO actions being performed.

REQ-DLBO-FUN-x Provisioning MnS for D-LBO function should have a capability allowing an authorized consumer to give indication to LBO function to consider renewable energy information for load transferring.

REQ-DLBO-FUN-y Performance assurance MnS for D-LBO function should have a capability allowing the authorized consumer to collect the renewable energy related performance measurements that are used to evaluate the LBO performance.

Next Change

6.4.1.5	LBO (Load Balancing Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically distribute user traffic among neighboring cells to ensure the radio resources are efficiently used, while providing quality end-user experience and performance. |  |
| Actors and Roles | D-SON management function to support LBO function. |  |
| Telecom resources | - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - D-LBO is in operation. |  |
| Begins when | The D-SON management function decides to enable D-LBO function. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to set the handover and/or reselection parameters ranges (see clause 15.5.1.4 in TS 38.300 [7]), and to enable the D-LBO function. |  |
| Step 2 (M) | The D-LBO function perform load balancing as describe in clause 15.5 in TS 38.300 [7] and may notify D-LBO management function when the LBO action has been performed. |  |
| Step 3 (M) | The D-SON management function collects LBO related measurements. In case the gNB is powered by renewable energy, the D-SON management function may collect renewable energy related metrics, such as carbon emission KPIs defined in TS 28.554 [x]. |  |
| Step 4 (M) | The D-SON management function analyses the measurements to evaluate the LBO performance and may request the producer of provisioning MnS to update the ranges for HO and/or reselection parameters and/or indication of considering renewable energy related metrics when performing LBO. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The LBO performance has been optimized. |  |
| Traceability | REQ-DLBO-FUN-1, REQ-DLBO-FUN-2, REQ-DLBO-FUN-3, REQ-DLBO-FUN-x, REQ-DLBO-FUN-y |  |

End of Changes