3GPP TSG-SA5 Meeting #159	S5-250429

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0074 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.313 Use case and requirements for renewable energy based LBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solve WT4 in objectives of WID Energy efficiency and energy saving aspects of 5G networks and services (SP-24140). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add solution and procudure for renewable energy based LBO in TS 28.313. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss the renewable energy based LBO feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.5; 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.1.5	LBO (Load Balancing Optimisation)

7.1.5.1	MnS component type A

Table 7.1.5.1-1: D-LBO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 5 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - establishStreamingConnection operation - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

7.1.5.2	MnS Component Type B definition

7.1.5.2.1	Control information

The parameter is used to control the LBO function.

Table 7.1.5.2.1-1: D-LBO control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| D-LBO function control | This attribute allows the operator to enable/disable the LBO functionality. See attribute dlboControl in TS 28.541 [13]. | Boolean On, off |
| D-LBO function considering renewable energy control | This attribute allows the operator to enable/disable the consideration of renewable energy metrics for LBO functionality. See attribute dlboRenewableControl in TS 28.541 [13]. | Boolean On, off |

7.1.5.2.2	Parameters to be updated

Table 7.1.5.2.2-1: Ranges of HO and cell selection parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger Low | This parameter defines the maximum allowed lower deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerLow in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Maximum deviation of Handover Trigger High | This parameter defines the maximum allowed upper deviation of the Handover Trigger, from the default point of operation (see clause 15.5.1.4 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). See attribute maximumDeviationHoTriggerHigh in TS 28.541 [13]. | [-20 .. 20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.1.4 in TS 38.300 [7]). See attribute minimumTimeBetweenHoTriggerChange in TS 28.541 [13]. | [0 .. 604800] in unit Seconds |

Table 7.1.5.2.2-2: D-LBO parameter of neighbour cells

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Load Balancing Allowed | This indicates if mobility load balancing is allowed or prohibited from source cell to target cell. See attribute isMLBAllowed in TS 28.541 [13]. | Boolean TRUE, FALSE |

7.1.5.3	MnS Component Type C definition

7.1.5.3.1	Performance measurements

Performance measurements related LBO are captured in Table 7.1.5.3.1-1:

Table 7.1.5.3.1-1. D-LBO related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| DL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the downlink (see clause 5.1.1.2.1 in TS 28.552 [5]). |  |
| UL Total PRB Usage | This measurement provides the total usage (in percentage) of physical resource blocks (PRBs) on the uplink (see clause 5.1.1.2.2 in TS 28.552 [5]). |  |
| Distribution of DL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the downlink (see clause 5.1.1.2.3 in TS 28.552 [5]). |  |
| Distribution of UL Total PRB Usage | This distribution measurement is to monitor when a cell may experience overload situation in the uplink (see clause 5.1.1.2.4 in TS 28.552 [5]). |  |
| DL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in downlink for data traffic (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| UL PRB used for data traffic | This measurement provides the number of physical resource blocks (PRBs) in average used in uplink for data traffic (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Mean number of RRC Connections | This measurement provides the mean number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Max number of RRC Connections | This measurement provides the maximum number of users in RRC connected mode during the granularity period (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |
| Mean number of stored inactive RRC Connections | This measurement provides the mean number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Max number of stored inactive RRC Connections | This measurement provides the maximum number of users in RRC inactive mode during each granularity period (see clause 5.1.1.4.3 in TS 28.552 [5]). |  |
| Carbon emission | This measurement provides carbon emission of gNB (see clause xxx in TS 28.554). |  |

Next Change

8.2.4	LBO (Load Balancing Optimisation)

Figure 8.2.4-1 depicts a procedure that describes how D-SON management function can manage the LBO function. It is assumed that the D-SON management function has consumed the performance assurance MnS to create PM jobs to collect handover related measurements and optionally renewable energy related measurements.

Figure 8.2.4-1: D-LBO procedure

1. The D-SON management function consumes the management service for NF provisioning with modifyMOIAttributes operation (see clause 5.1.3 in TS 28.532 [3]) to configure the ranges of HO and/or reselection parameters and LBO parameter of neighbour cells for the LBO function.

1.a The MnS of provisioning sets the ranges for MRO function (NOTE).

2. The D-SON management function consumes the NF provisioning MnS with modifyMOIAttributes operation to enable the LBO function for a given NR cell if it is not enabled.

2.a The provisioning MnS enables the LBO function (NOTE).

3. The LBO function collects real-time load information to determine and perform actions to balance the traffic loads among NR cells and the LBO function may indicates its recommendation of LBO parameters of neighbour cells to D-SON management function based on its real-time load information.

4. D-SON management function collects LBO related performance measurements, which may include the value of handover related metrics and renewable energy related metrics. Renewable energy related metrics represent the usage of renewable energy of gNBs.

5. The D-SON management function analyses the measurements to evaluate the LBO performance.

6. The D-SON management function consume the MnS of provisioning with modifyMOIAttributes operation to update the ranges of handover parameters if the LBO failed to meet expection. Additionally, the D-SON management function may send an indication to consider renewable energy related metrics when performing load transferring between different gNBs.

6.a. The MnS of provisioning updates the ranges of HO and/or reselection parameters (NOTE).

NOTE: The interface between provisioning MnS and D-LBO function is not subject to standardization.

End of Changes