# 3GPP TSG-SA5 Meeting #159	S5-250548

# 17 - 21 February 2025, Sophia Antipolis, France

# Source:	Ericsson Hungary

# Title:		Discussion paper: Modify YANG mapping of Subnetwork

# Document for:	   Endorsement

# Agenda Item:	6.4.27

# 1	Decision/action requested.

The group is asked to endorse the proposal.

# 2	References

[1] 	3GPP TS 32.160 Management service template

[2] 	3GPP TS 28.622 Integration Reference Point (IRP); Information Service (IS)

[3]	RFC 8528 YANG Schema Mount

# 3	Rationale

Observation 1: Recursive containment of Subnetwork instances leads to strange addressing

32.160 clause 6.2.7 defines a pattern for recursive containment e.g. when a Subnetwork1 MOI contains another Subnetwork2 MOI. This pattern does not result in YANG containment (a list entry representing Subnetwork1 containing a list entry representing Subnetwork2), instead it results in a list where Subnetwork1 and Subnetwork2 are sibling entries of the same list schema node. The basic reason for this pattern is that YANG is not an inherently object-oriented modeling language.

The result of this pattern is that a MOI with a DN

Subnetwork=Subnetwork1, Subnetwork=Subnetwork2, Subnetwork=Subnetwork3, ManagedElement=ME1, PerfmetricJob=Job1

Will have a “YANG address”

/Subnetwork[id=“Subnetwork3”]/ManagedElement[id=”ME1”]/PerfmetricJob[id=”Job1”]             (omitting prefixes here for brevity)

The YANG address does not include any Subnetworks containing other Subnetworks. A YANG address containing all containing Subnetworks would look like:

/Subnetwork[id=“Subnetwork1”] /Subnetwork[id= “Subnetwork2”] /Subnetwork[id= “Subnetwork3”] /ManagedElement[id=”ME1”]/PerfmetricJob[id=”Job1”]

(A YANG address is used in read and write Netconf operations and notifyChangeMOIs notifications format of the address is different in each case, but the content is the same. )

Different parts being present in the DN and the YANG address creates problems and confusion.

Observation 2: The current method of recursive containment adds restrictions on the id of a subnetwork

NRM Stage 2 to allows Subnetwork1 with id=”a” to contain another Subnetwork2 also with id=”a”. Subnetwork1 and Subnetwork2 will have different DNs so they can be handled properly:

DN of Subnetwork1: Subnetwork=a

DN of Subnetwork2: Subnetwork=a, Subnetwork=a

The current YANG mapping does not allow this, as both Subnetworks are represented as entries of the same list, so the values of “id” (which is the YANG key) must be different.

## 3.1	Use Yang schema mount for Subnetwork in Subnetwork containment

Mount any Subnetwork that is name-contained in another Subnetwork under the mountpoint children-of-SubNetwork of the containing Subnetwork.

Remove “leaf-list parents” and “leaf-list containedChildren” as they are not needed any more.

yangmnt:mount-point children-of-SubNetwork { description "Mountpoint for ManagedElement"; reference "RFC8528 YANG Schema Mount"; } }

Today the mountpoint children-of-SubNetwork can only include a ManagedElement. It should also contain contained (child) Subnetworks.

yangmnt:mount-point children-of-SubNetwork { description "Mountpoint for ManagedElements or contained Subnetworks"; reference "RFC8528 YANG Schema Mount"; } }

The mount information in the MS Word text in 28.623 shall be updated. When implementing the NRM vendors shall implement YANG schema mount to the required level. See RFC 8528[3] section 3.4.

## 3.2	Define multiple level the of subtree list – not recommended

list SubNetwork { key id; uses top3gpp:Top_Grp; container attributes { uses SubNetworkGrp; } } list SubNetwork { key id; uses top3gpp:Top_Grp; container attributes { uses SubNetworkGrp; } list Subnetwork { … list Subnetwork {…} } } }

The solution seems to work if we define a reasonable depth of embedded Subnetwork lists and allow vendors to add more. However, there are problems with the solution and thus it is not recommended:

- The number of containment levels need to be pre-defined in design time.

- Augment statements must include an absolute-schema-nodeid which would need to include the  exact containment steps including all Subnetworks. As the number of Subnetwork containment levels is unknown, it would not be possible to use “augment” statements at all.

- Any statements  (leafrefs, instance-identifiers, must or when statements) that use absolute references will not work. Although these can be avoided, it will be a risk if they are introduced at any time.

# 4	Detailed proposal

The group is requested to endorse the below recommendations.

Recommendation 1: Modify the Subnetwork-in-subnetwork name containment to use YANG schema mount as defined in clause 3.1